3GPP TSG-WG SA2 Meeting #161	S2-2402063

Athens, Greece, 26 February – 1 March, 2024	(revision of S2-2400804)

Source:	Huawei, HiSilicon

Title:	New Use Case for WT#2: VFL between NWDAFs

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: Propose a use case for using VFL among NWDAFs for improving the accuracy and effectiveness of analytics when NWDAFs are not allowed to share their ML Models.

1. Introduction

In an attempt to expand the possible collaborations for training processes in NWDAF architecture, the TR 23.700-84 defined the KI#2 to investigate the scenarios that can benefit from joint collaborative training based on Vertical Federated Learning principles as well as the associated mechanisms to support such processes.

2. Discussion

This contribution identifies the scenario where analytics aggregation with NWDAFs from different vendors without interoperability cannot use current HFL for the generation of the aggregated analytics output based on the collaborative training of ML models, therefore, aligned ML models. In such scenario the ML models of the multiple vendor NWDAFs cannot be shared, preventing the HFL mechanisms to be used.

Therefore, VFL among NWDAFs is proposed.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-84.

# * * * * First change * * * *

5.1.y	Use Case #y: Vertical Federated Learning among NWDAFs

This use case is related to the KI#2 on studying the support for Vertical Federated Learning with focus on the identification of the scenario where VFL is relevant for the joint ML model training among NWDAFs.

when multiple NWDAFs instances are deployed in a mobile network, it is possible that one single NWDAF instance cannot generate the required analytics output e.g. for different area of interest or PLMNs. In details:

On roaming scenarios, e.g., when VPLMN aggregates its own generated analytics (i.e, H-analytics output) with the requested analytics from V-RE-NWDAF (V-analytics output) as specified in TS 23.288 Clause 6.1.5.3. Or, the NWDAF from different vendors cannot share the model for the same samples (e.g. AoIs) during analytic aggregation.

When analytics aggregation is used within the same mobile network, the aggregation mechanism will be applied to any analytics that can be generated by NWDAF with target of reporting set to “any UE” (therefore, the subscription necessarily defines the AoI parameter). Examples of such analytics ID are:

-	Slice load analytics (Clause 6.3);

-	Service experience (Clause 6.4): for slice, for application, for an Edge Application over a UP path, for an Edge Application over a UP path

-	User data congestion (Clause 6.8)

-	QoS Sustainability (Clause 6.9)

-	Dispersion analytics (Clause 6.10)

-	DN Performance Analytics (Clause 6.14)

The alternative for supporting proper analytics as mentioned above, when NWDAFs cannot share their ML models, is to use VFL as the collaborative joint ML technique.

NOTE :  The VFL between roaming scenario will be studied in the context of this use case based on the architecture defined in TS 23.288 clause 6.1.5.2 and 6.1.5.3, if it does not require different mechanism as non roaming.

# * * * * End of changes * * * *