3GPP TSG-SA2 Meeting #107	S2-2402087

26 Feb-01 Mar 2024, Athens, Greece	(revision of xx-yyxxxx)

# Source:	China Telecom, SK Telecom

# Title:	New WID: TEI19_NetFailTol; Enhancing Network Failure Tolerance

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancing Network Failure Tolerance

Acronym: TEI19_NetFailTol

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MINT_Ph2 | SA1 | 970041 | Minimization of Service Interruption During Core Network Failure Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 920062 | Stage 2 of MINT | R17 MINT enables a UE to perform Disaster Roaming to a 5G PLMN in the case of RAN failure. |

3	Justification

This WID proposes two solutions to address two different/independent network failure scenarios.

Scenario 1: Enabling UEs to perform Disaster Roaming when UDM/UDR/AUSF is functional and the rest of core network fails.

To enhance readability, the PLMN in a Disaster Condition and the PLMN offering Disaster Roaming service are referred to as PLMND and PLMNA, respectively.

R17 MINT enables a UE to obtain service from PLMNA when a Disaster Condition applies to the UE's registered PLMN (PLMND). However, Disaster Condition only applies to NG-RAN nodes. Based on SA1 requirements introduced in R19 MINT_Ph2, the application of MINT function should include core network failure. But it should exclude any core network failure that renders PLMND unable to authenticate its subscribers. Therefore, UDM/UDR/AUSF in PLMND remains functional.

In the case of the failure of NG-RAN nodes, Disaster Roaming with local breakout and with home routing are both supported. In the case of core network failure, it is likely that the SMF in PLMND fails and home routing cannot be supported. If the AMF in PLMNA determines that the Disaster Condition applies to core network failure, the AMF controls that PDU Sessions requested by the Disaster Inbound Roaming UEs are only established in LBO, by performing a DNN replacement based on local configuration. Whether IMS voice can be supported with local breakout is based on operator agreement between PLMND and PLMNA. How the AMF determines that the Disaster Condition applies to core network failure is via OAM, and is out of scope of 3GPP.

Scenario 2: Enabling non-roaming UEs to obtain service from the registered PLMN when the UDM/UDR in the registered PLMN fails. The rest of the core network is functional.

In the case of UDM/UDR failure, Nudm_UECM_Registration and Nudm_SDM_Get services become not available. Non-roaming UEs previously authenticated can still complete mobility/periodic registration procedures if the AMF has registered to UDM and retrieved subscription data before the failure occurs and the AMF does not change. However, when the non-roaming UEs previously authenticated perform EPS to 5GS handover, or inter NG-RAN node N2 based handover, or idle mode mobility with an AMF change, the AMF changes and the new AMF shall retrieve subscription data from UDM to complete the registration procedures.

It is proposed that, when Nudm_UECM_Registration and Nudm_SDM_Get services are not available, the AMF may use local configuration to complete the registration procedures triggered by handover or idle mode mobility with an AMF change. The AMF may determine the local configuration based on the S-NSSAI(s) received from the UE and the UE’s SUPI to consider UE’s characteristics.

It is proposed that, when the AMF determines that Nudm_UECM_Registration and Nudm_SDM_Get services become available, the AMF may perform step 13 and step 14 in the Registration procedure of clause 4.2.2.2.2 or the AMF may deregister UEs (in a controlled manner without causing signaling overload) so that UEs perform registration again.

The SMF shall retrieve Session Management Subscription data from UDM when the SMF does not have it for requested SUPI, DNN and S-NSSAI (e.g. when non-roaming UEs previously authenticated have not established any PDU sessions since registration, or they request a new PDU Session, or they perform EPS to 5GS handover without N26 interface.)

It is proposed that, when Nudm_UECM_Registration and Nudm_SDM_Get services are not available, the SMF may use local configuration to complete the PDU Session Establishment procedures. The SMF may determine the local configuration based on the SUPI, DNN and S-NSSAI received from the AMF to consider UE’s characteristics.

It is proposed that, when the SMF determines that Nudm_UECM_Registration and Nudm_SDM_Get services become available, the SMF may retrieve the Session Management Subscription data and register with the UDM.

4	Objective

This new WID aims to enable the following to enhance network failure tolerance. It contains two Work Tasks: WT1 is an enhancement of R17 MINT based on SA1 requirements, and WT2 is not related to R17 MINT.

WT1:

Applying Disaster Roaming to core network failure with functional UDM/UDR/AUSF.

The AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure and establishes PDU Session in local breakout by performing a DNN replacement based on local configuration.

NOTE 1:	In WT1, UDM/UDR/AUSF of the PLMN in Disaster Condition needs to be functional to support Disaster Roaming.

NOTE 2:	How the AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure is via OAM, and is out of scope of 3GPP.

WT2:

When Nudm_UECM_Registration and Nudm_SDM_Get services are not available, the AMF may use local configuration to complete the registration procedures triggered by EPS to 5GS handover, or inter NG-RAN node N2 based handover, or idle mode mobility with an AMF change for the non-roaming UEs previously authenticated.

When Nudm_UECM_Registration and Nudm_SDM_Get services are not available, the SMF may use local configuration to complete the PDU Session Establishment procedures for the non-roaming UEs previously authenticated.

When Nudm_UECM_Registration and Nudm_SDM_Get services become available, the AMF and SMF may perform registration with the UDM and retrieve subscription data for the non-roaming UEs previously authenticated.

NOTE 3:	In WT2, UEs do not perform roaming and continue to receive service from the registered PLMN with UDM/UDR failure.

The expected time for completing this work is 1 TUs. 0.5 time slots in one SA2 meeting for WT1 and 0.5 time slots in next SA2 meeting for WT2.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | - Allow the support of MINT under core network failure. - Allow the AMF in the PLMN providing Disaster Roaming service to determine that the Disaster Condition applies to core network failure and establish PDU Session in local breakout. | SA#106 (Dec 2024) |  |
| TS 23.502 | - Allow the AMF to complete the registration procedures triggered by EPS to 5GS handover, or inter NG-RAN node N2 based handover, or idle mode mobility with an AMF change for the non-roaming UEs previously authenticated, when Nudm_UECM_Registration and Nudm_SDM_Get services are not available - Allow the SMF to complete the PDU Session Establishment procedures for the non-roaming UEs previously authenticated, when Nudm_UECM_Registration and Nudm_SDM_Get services are not available - Allow the AMF and SMF to perform registration with the UDM and retrieve subscription data when Nudm_UECM_Registration and Nudm_SDM_Get services become available | SA#106 (Dec 2024) |  |

6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

ZTE

CATT

Inspur

vivo

Huawei

Rakuten Mobile