3GPP TSG-SA2 Meeting #107	S2-XXXX

26 Feb-01 Mar 2024, Athens, Greece

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draftCR | Rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applying Disaster Roaming to core network failure (WT1 of TEI19_NetFailTol) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetFailTol |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on SA1 requirements introduced in R19 MINT_Ph2, the application of MINT function should include core network failure. But it should exclude any core network failure that renders the network subject to a disaster unable to authenticate its subscribers. In the case of core network failure, Disaster Roaming with home routing cannot be supported. The AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure and establishes PDU Session in local breakout only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applying Disaster Roaming to core network failure with functional UDM/UDR/AUSF. The AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure and establishes PDU Session in local breakout only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE experiences a long period of service interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.1, 5.40.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.1	General

Subject to operator policy and national/regional regulations, 5GS provides Disaster Roaming service (e.g. voice call and data service) for the UEs from PLMN(s) with Disaster Condition. The UE shall attempt Disaster Roaming only if:

-	there is no available PLMN which is allowable (see TS 23.122 [17]);

-	the UE is not in RM-REGISTERED and CM-CONNECTED state over non-3GPP access connected to 5GCN;

-	the UE cannot get service over non-3GPP access through ePDG;

-	the UE supports Disaster Roaming service;

-	the UE has been configured by the HPLMN with an indication of whether Disaster roaming is enabled in the UE set to "disaster roaming is enabled in the UE" as specified in clause 5.40.2; and

-	a PLMN without Disaster Condition is able to accept Disaster Inbound Roamers from the PLMN with Disaster Condition.

In this Release of the specification, the Disaster Condition is not limited to the failure of NG-RAN nodes, as long as the PLMN with Disaster Condition is able to authenticate its subscribers. In the case of NG-RAN failure, Disaster Roaming with local breakout and with home-routed are supported. In the case of core network failure, Disaster Roaming is only supported with local breakout.

* * * * Start of 2nd Change * * * *

5.40.4	Registration for Disaster Roaming service

For a UE to receive Disaster Roaming service from a PLMN providing Disaster Roaming service, the UE sends a NAS Registration Request message with Registration Type value "Disaster Roaming Initial Registration" or "Disaster Roaming Mobility Registration Update":

-	When the AMF in the PLMN providing Disaster Roaming service receives a NAS Registration Request with Registration Type set to "Disaster Roaming Initial Registration" or "Disaster Roaming Mobility Registration Update";

-	the AMF controls if the UE is allowed to access Disaster Roaming service in the area with Disaster Condition as specified in clause 4.2.2.2.2 of TS 23.502 [3];

-	the AMF may provide the Disaster Roaming service indication to AUSF and UDM as specified in clause 4.2.2.2.2 of TS 23.502 [3] and TS 33.501 [29]. The AMF may provide the Disaster Roaming service indication to SMF as specified in clause 4.3.2 of TS 23.502 [3].

NOTE 1:	The AUSF and the UDM are configured with Disaster Condition via OAM based on operator policy and the request by the government agencies. Based on this local configuration and/or the Disaster Roaming service indication, the AUSF can execute authentication of the UE, and the UDM can provides the subscription data for a Disaster Roaming service to the AMF and/or the SMF.

In the case of core network failure, the AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure and establishes PDU Session in local breakout by performing a DNN replacement based on local configuration.

NOTE 2:	How the AMF in the PLMN providing Disaster Roaming service determines that the Disaster Condition applies to core network failure is via OAM, and is out of scope of 3GPP.

NOTE 3:	Whether IMS voice can be supported with local breakout is based on operator agreement between the PLMN providing Disaster Roaming service and the PLMN with Disaster Condition.

To support the Disaster Roaming service, the PLMN providing Disaster Roaming service is configured to support communication with the network entities in the HPLMN of the UE, i.e. configurations related to roaming interfaces for communication between serving PLMN and HPLMN shall be deployed in the affected entities. This communication between the PLMNs need only be enabled during the Disaster Condition.

The Disaster Roaming service is limited to the impacted geographic area with Disaster Condition. The NG-RAN nodes and AMF in the PLMN providing Disaster Roaming service are configured with the area information, i.e. a list of TAIs which can be formulated by the PLMN providing the Disaster Roaming service based on the geographic area with Disaster Condition in the other PLMN(s).

The AMF in the PLMN providing Disaster Roaming service provides the mobility restriction list to the NG-RAN as specified in clause 5.3.4.1.1 considering the area with Disaster Condition, and also indicating that EPC is not an allowed core network.

NOTE 4:	From the perspective of emergency services, a UE is following procedures as described in clause 4.24 of TS 24.501 [47] when registered for Disaster Roaming service.

* * * * End of Change * * * *