SA WG2 Meeting S2#161	S2-2402339

February 26 – March 01, 2024, Athens, Greece	revision of S2-240xxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2: Update of solution #7 on IMS capability exposure via DC3/DC4

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an update of solution #7.

# Introduction

The paper addresses several open editor notes in solution #7.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

## 6.7	Solution #7: Solution for IMS capability exposure via DC3/DC4

### 6.7.1	Description

The architecture to enable IMS capability exposure in the context of IMS data channel (DC) sessions is based on Figure AC.2.1-1 in clause AC.2.1 of TS 23.228 [5] and is depicted in Figure 6.7.1-1.

Figure 6.7.1-1: IMS capability exposure architecture

This solution defines the exposure of IMS capabilities related to DC session via DC3/DC4.

#### 6.7.1.1	Events to be exposed

With this solution, there are two types of events, which can be exposed via DC3/DC4 interfaces (see Figure 6.7.1-1) from DCSF to DCAS via Ndcsf services.

The first type of events are call related events:

-	establishing, updating, answering, terminating of IMS calls.

NOTE: The exact list of call related events will be defined by stage 3.

The second type of events are DC related events, for example:

-	establishing, updating, terminating of bootstrap data channels,

-	establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels, and

-	downloading of an application via an existing bootstrap data channel.

#### 6.7.1.2	Information to invoke the exposure service

DCAS provides following information to invoke a service on DC3/DC4:

-	Information used for DC establishment, DC update, and DC termination:

-	DC application binding information,

-	User ID which includes Calling ID and/or Called ID,

-	User ID which the DC application apply to,

-	Session ID which the DC application apply to, and

-	DC identifier to determine a DC that is updated or terminated.

-	Information used for DC event subscription and notification:

-	type of event related to specific User ID(s), i.e., application DC establishment/updating/termination, downloading of a specific DC application.

-	Information used for IMS event exposure:

-	type of IMS event related to specific User ID(s), i.e., establishing, updating, answering, terminating of IMS calls.

#### 6.7.1.3	Information to be exposed

One Ndcsf service can provide session control capabilities (e.g. Ndcsf_dc_session_control). This service allows DCAS to create DC session and take control on DC session via DCSF.

DCSF can return to DCAS the following information on DC3/DC4:

-	response code (success or failure);

-	other information, e.g. session identifier (session ID), IP address of the UE, location, MDC2 identifier, user identifier (user ID), PVNI, DC identifier, Call ID which can be used to determine a DC uniquely;

-	notifications to the reported events (DC establishment/termination, downloaded DC application, IMS event exposure).

### 6.7.2	Procedures

#### 6.7.2.1	Event Subscription and Notification

The event subscription and event notification can be realized in two different ways (Approach 1 and 2 below).

Figure 6.7.2.1-1 depicts the call flow of approach 1 where both call and DC related events are subscribed and exposed via the DCSF provided services. A call related event subscription request is then forwarded from DCSF to IMS AS.

Figure 6.7.2.1-1: Approach 1 – Call and DC related event subscription and exposure via DCSF

IMS AS registration: The IMS AS register its address and potentially other information (e.g., event exposure capabilities) in the HSS per user during IMS registration. Another option would be that IMS AS registers its profile in the NRF per group of users it is potentially serving. For the second option, IMS AS selection by the S-CSCF by evaluating initial filter criteria must be aligned with the IMS AS profile entry in the NRF.

DCSF registration: DCSF registers its NF Profile and capability information in the NRF. As part of the capability information, the DCSF can e.g., provide user ID(s), user ID ranges and/or single events or type of events and exposure services it is capable to handle.

In Approach 1, the NEF or DCAS discover first a DCSF to subscribe for call and DC related events, the DCSF forwards the subscribe request to the IMS AS for call related events (see Figure 6.7.2.1-1, step 1).

If the subscription from the DC Application Server is via DC3, DC Application Server sends a request to a NEF for an event with the event information included in an event subscription request. As described in clause 6.7.1.1, there are two types of capabilities, which can be subscribed and exposed. Specifically, for call related events, the subscription request is forwarded to IMS AS via DC1 from DCSF. DC Application Server can provide the information listed in clause 6.7.1.2 to invoke the exposure service.

The NEF discovers a DCSF from the NRF. The request includes the event information received from the DCAS. The NRF responds to the NEF with one or several DCSF(s), which are able to expose the subscribed event based on the capability information and the received event information. NEF selects a DCSF which is identified by the DCSF identity, which is part of the registered NF Profile (e.g., IP address or FQDN). The NEF then sends a subscribe request to the selected DCSF.

NOTE: If the subscription from the DCAS via DC4, the DCAS discovers a DCSF directly via NRF and subscribes for events at the DCSF.

DC related events can be handled by the DCSF itself, thus NEF request for subscribing to such events terminates at the DCSF. DCSF can then notify the NEF or DC AS about the event (see Figure 6.7.2.1-1, step 3).

For call related events, the DCSF fetches the IMS AS address from HSS based on user identity or optionally, IMS AS can be discovered via NRF (e.g., based on user ID ranges). DCSF forwards the event subscription request to the IMS AS via DC1 and receives event notification also via DC1 from IMS AS. DCSF forwards then the event notification to the NEF or DCAS (see Figure 6.7.2.1-1, step 3).

DCSFresponds to DC Application Server via DC3/DC4 whether the event subscription is successful or not.

DC Application Server respond(s) to DCSF (and IMS AS) via DC3/DC4 (and DC1).

Figure 6.7.2.1-2 depicts the call flow of Approach 2 where call related events are subscribed and exposed via IMS AS provided services and DC related events are subscribed and exposed via DCSF provided services. Based on the event type (e.g. call related event or DC related event), NEF or DCAS decide to subscribe at IMS AS or DCSF. Once NEF or DCAS have decided to subscribe at IMS AS or DCSF, they must discover the IMS AS or DCSF by means described above and send an event subscription request to IMS AS or DCSF (see Figure 6.7.2.1-2, steps 1 and 1a). If the event occurs, IMS AS or DCSF send a notification to NEF or DCAS (see Figure 6.7.2.1-2, steps 3 and 3a).

Figure 6.7.2.1-2: Approach 2 - Call and DC related event subscription and exposure via IMS AS and DCSF

#### 6.7.2.2	Person-to-Person (P2P) Application Data Channel Setup

Based on Figure AC.7.2.1-1 in clause AC.7.2.1 of TS 23.228 [5], Figure 6.7.2.2-1 depicts a call flow for establishing an Application Data Channel triggered by IMS capability exposure in a person to person use case.

Figure 6.7.2.2-1: Person-to-Person (P2P) application data channel set up triggered by IMS capability exposure signalling procedure

The steps in the call flow are as follows:

1.	Event subscribed and exposed following the procedure described in clause 6.7.2.1. The event can be, e.g. an application has been downloaded to UE#1.

2.	DC Application Server requests IMS AS to establish a data channel with session ID, user IDs, binding information, through (NEF and) DCSF.

3.	IMS AS determines whether the media change request event should be notified to a DCSF and selects a DCSF.

4.	IMS AS notifies the DCSF of the media change request event.

5.	The DCSF determines the DC control policy.

6.	The DCSF responds to the notification received in step 4.

7.	IM AS starts IMS DC negotiation with UE#1 and UE#2.

8.	IMS IM AS sends a SIP reINVITE request to I/S-CSCF.

9.	I/S-CSCF sends the SIP reINVITE request to Terminating Network/UE#2.

10.	Terminating Network/UE#2 responds a SIP 200 OK with audio/video SDP offer to I/S-CSCF.

11.	I/S-CSCF sends the SIP 200 OK with audio/video SDP offer to IMS AS.

12.	IMS AS sends a SIP reINVITE request with audio/video, application DC SDP offer to UE#1, through I/S-CSCF and P-CSCF.

13.	UE#1 responds a SIP 200 OK with audio/video, application DC SDP offer to IMS AS, through P-CSCF and I/S-CSCF.

14.5IMS AS notifies DCSF of the successful media change, with session ID and a list of media information.

15.	DCSF responds IMS AS.

16.	IMS AS sends a SIP ACK with audio/video SDP answer to I/S-CSCF.

17.	I/S-CSCF sends the SIP ACK with audio/video SDP answer to Terminating Network/UE#2.

18.	IMS AS returns a message to DC Application Server, through DCSF (and NEF), to notify if the application DC establishment is successful or failed. If the establishment is failed, step 24 shall be skipped.

19.	The application data channel between the UE#1 and the UE#2 is established. In this example, it is not anchored in MF/MRF.

#### 6.7.2.3	Person-to-Application (P2A) Application Data Channel Setup

Based on Figure AC.7.2.2-1 in clause AC.7.2.2 of TS 23.228 [5], Figure 6.7.2.3-1 depicts a call flow for establishing an Application Data Channel in a person to application use case. Note that there are two options to reserve the MDC2 media resources. The first option is that the media resources are reserved by DC Application Server before the application DC establishment request. The second option is that the media resource reservation is requested by IMS AS and resources are reserved by MF/MRF after the application DC establishment request.

Figure 6.7.2.3-1: Person-to-Application (P2A) application data channel set up signalling procedure

The steps in the call flow are as follows:

1.	Event subscribed and exposed following the procedure described in clause 6.7.2.1. The event can be, e.g. an application has been downloaded to UE#1.

2.	Optionally, DC Application Server can reserve the media resource, before it starts the request of data channel establishment.

3.	If the media resource is not reserved in step 2, the DC Application Server requests IMS AS to establish an application data channel with session ID, user IDs, and binding information, through (NEF and) DCSF. If the media resource is reserved in step 2, together with the request, the reserved media resource shaw also be sent, so that DCSF is informed about the reserved media resource. The SDP offer to UE#1 is also provided.

4.	IMS AS determines whether the media change request event should be notified to a DCSF and selects a DCSF.

5.	IMS AS notifies the DCSF of the media change request event.

6.	The DCSF determines the DC control policy.

7.	The DCSF responds to the notification received in step 5.

8.	If the media resource is not reserved in step 2, IMS AS requests MF/MRF to reserve media resource for the originating side and the MDC2, for DC Application Server use. If the media resource is reserved in step 2, the media resource reservation for the MDC2 in this step shaw be skipped.

9.	After the media resource is reserved, IMS AS responds DCSF with the information of the reserved media resource.

10.	If the media resource is not reserved in step 2, in this step, DCSF sends the MDC2 SDP offer received from MF/MRF to the DC Application Server via DC3/DC4. If the media resource is reserved in step 2, this step shaw be skipped.

11.	If the media resource is not reserved in step 2, in this step, DC Application Server returns an MDC2 SDP answer and is prepared for UE#1 traffic through MDC2. If the media resource is reserved in step 2, this step shaw be skipped.

12.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

13.	IM AS updates the MF/MRF resource.

14.	IM AS responds DCSF with the updated information of the reserved media resource.

15.	DCSF responds IMS AS.

16.	IMS AS starts IMS DC negotiation with UE#1 and UE#2.

17.	IMS AS sends a SIP reINVITE request to I/S-CSCF for media re-negotiation between UE#1 and UE#2.

18.	I/S-CSCF sends the SIP reINVITE request to Terminating Network/UE#2.

19.	Terminating Network/UE#2 responds a SIP 200 OK with audio/video SDP offer to I/S-CSCF.

20.	I/S-CSCF sends the SIP 200 OK with audio/video SDP offer to IMS AS.

21.	IMS AS sends a SIP reINVITE request with audio/video, application DC SDP offer to UE#1, through I/S-CSCF and P-CSCF.

22.	UE#1 responds a SIP 200 OK with audio/video, application DC SDP answer to IMS AS, through P-CSCF and I/S-CSCF.

23.	IMS AS notifies DCSF of the successful media change, with session ID and a list of media information.

24.	DCSF responds IMS AS.

25.	IMS AS sends a SIP ACK with audio/video SDP answer to I/S-CSCF.

26.	I/S-CSCF sends the SIP ACK with audio/video SDP answer to Terminating Network/UE#2.

27.	IMS AS returns a message to DC Application Server, through DCSF (and NEF), to notify if the application DC establishment is successful or failed. If the establishment is failed, step 24 shall be skipped.

28.	The application data channel between the UE#1 and DC Application Server is established via MF/MRF. MF/MRF forwards data channel traffic between the UE#1 and DC Application Server based on MDC2 media point information received in step 8.

#### 6.7.2.4	Enhancements to IMS AS services

Editor's note:	Exact enhancement of the services is FFS.

#### 6.7.2.5	Enhancements to HSS services

Editor's note:	Exact enhancement of the services is FFS.

### 6.7.3	Service definition

#### 6.7.3.1	DCSF services

One Ndcsf service can provide event capability exposure (e.g., Ndcsf_EventExposure).

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_EventExposure | Subscribe | Subscribe/Notify | NEF, DCAS |
|  | Unsubscribe |  | NEF, DCAS |
|  | Notify |  | NEF, DCAS |

#### 6.7.3.2	Ndcsf_EventExposure service

##### 6.7.3.2.1	General

This service enables the consumer NF to subscribe and get notified of events.

The following events can be subscribed by a NF consumer:

-	IMS session control events of a specific IMS subscriber

-	IMS media control events of a specific IMS subscriber

-	IMS data channel related events

The following service operations are defined for the Ndcsf_EventExposure service:

-	Ndcsf_EventExposure_Subscribe

-	Ndcsf_EventExposure_Unsubscribe

-	Ndcsf_EventExposure_Notify

##### 6.7.3.2.2	Ndcsf_EventExposure_Subscribe operation

Service operation name: Ndcsf_EventExposure_Subscribe

Description: The consumer NF subscribes to receive an event, or if the event is already defined in DCSF, then the subscription is updated.

Inputs, Required: NF ID, Target of Event Reporting (either UE ID(s), or UE IPv4 address(es), or UE IPv6 prefix(es), or Internal/External Group Identifier, or indication that any UE is targeted), (set of) Event ID(s), Notification Target Address (+ Notification Correlation ID) and Event Reporting Information.

Inputs, Optional: Event Filter(s) associated with each Event ID, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the existing subscription), Subscription expiry time.

NOTE:	In the case of untrusted DCAS, NEF ID is used as NF ID.

Outputs, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on local policy).

Outputs, Optional: None.

##### 6.7.3.2.3	Ndcsf_EventExposure_Unsubscribe service operation

Service operation name: Ndcsf_EventExposure_Unsubscribe

Description: The consumer NF unsubscribes for a specific event.

Inputs, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

##### 6.7.3.2.4	Ndcsf_EventExposure_Notify service operation

Service operation name: Ndcsf_EventExposure_Notify

Description: The DCSF provides the previously subscribed event information to the consumer NF which has subscribed to that event before.

Inputs, Required: Notification Correlation Information, Session ID, Event ID, time stamp.

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, DC stream ID, Session case (it indicates if this is an originating or terminating IMS session), Media info list (Media ID, Media type (DC, Audio, or Video), DC port, DC mapping and configuration information: binding information, MDC1/MDC2 media endpoint address (IP address/port number, etc.)), Event specific parameter list.

Output, Required: None.

Output, Optional: None.

Editor's Note:	The complete Event specific parameter list is FFS.

### 6.7.4	Impacts on existing nodes and functionality

Impacted nodes and functionality are as follows:

-	DC Signalling Function (DCSF): The DCSF interacts with the DCAS for exposes DC resource and session control related capabilities to IMS capability exposure via DC3/DC4. The DCSF sends call related event subscription to IMS AS via DC1. The DCSF receives call related event exposure from IMS AS via DC1.

-	Network Exposure Function (NEF): It shall securely expose IMS capability and events provided by DCSF to DCAS exposure and exposes new services to external AF.

-	DC Application Server (DCAS): An application server that interacts with the DCSF (through NEF) for IMS capability exposure using DC3/DC4 interfaces. DCAS acts as AF.

-	IMS Application Server (IMS AS): The IMS AS interacts with the DCSF via DC1 for event subscription and exposure.

# * * * * End of changes* * * *