SA WG2 Meeting #161	S2-2402379

26 February - 1 March, 2024, Athens, Greece

Source:	China Mobile

Title:	KI #3, Update sol: Update to solution #9

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract: This paper proposes to update solution #9 for KI#3 of FS_NG_RTC_Ph2.

1. Discussion

This contribution proposes to update the solution #9 to KI#3.

3. Proposal

It is proposed to capture the following changes into TR 23.700-77.

# * * * * First change * * * *

6.9	Solution #9: Data Channel Interworking mechanism between DCMTSI UE and MTSI UE

6.9.1	Description

This solution addresses Key Issue#3 "Data channel interworking with MTSI UE" to support data channel interworking mechanism between DCMTSI UE and MTSI UE, which covers:

-	how to enhance IMS architecture and procedures to provide some form of IMS data channel applications service experience by data channel interworking with MTSI UE supporting only legacy media types where it is appropriate.

This solution proposes a method to achieve the one-sided interactive communication between a DCMTSI client in terminal and an MTSI client in terminal.

6.9.2	Procedures

6.9.2.1	Interworking between a DCMTSI client and an MTSI client

6.9.2.1.1	General

The procedures described in this clause is applied to some data channel use cases in which it's a audio session and only unidirectional interaction is needed between both UEs. Real-time Screen Sharing as specified in TR 22.873 [16] is used as an example to describe the procedure. For example, in Real-time Sharing Screen use case, only UE A supports IMS data channel and it establishes application data channels with the MF in its local network and transfers its mobile screen data, including RTP stream and RTCP stream, over the application data channels. The MF decapsulates RTP data and RTCP data from the data channels, encapsulates and sends these data through video media streams to UE B. Figure 6.9.2.1-1 depicts the workflow below.

Figure 6.9.2.1-1: Interworking between a DCMTSI client in terminal and an MTSI client in terminal

6.9.2.1.2	Interworking between a DCMTSI client in terminal and an MTSI client in terminal during BootstrapData Channel establishment

Figure 6.9.2.1.2-1 demonstrates the procedure of UE-initiated bootstrap data channel establishment between network A and UE A (a DCMTSI client in terminal), while an IMS audio call is established between UE A and UE B (an MTSI client in terminal).

Figure 6.9.2.1.2-1: Bootstrap data channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1-2.	UE #1 initiates the bootstrap DC establishment procedure along with audio media components, originating DCSF receives the DC negotiation result from IMS AS, which indicates the bootstrap DC established successfully in local network.

3-6.	The INVITE request is sent to remote network with SDP containing data channel media component for terminating UE #2. In the bootstrap DC negotiation result returned from remote network to originating DCSF, the port of bootstrap data channel between the originating DCSF and UE #2 is set to zero, which means that terminating network or UE #2 does not support or does not use IMS data channel in this session. Based on this information and DC negotiation result in step2, the DCSF determines that UE B or network B does not support data channel, at least UE B or network B does not use data channel in this session, and the data channel has been established successfully only in local network. The IMS AS sends 18X message to UE A to indicate that the data channel has been established successfully only in local network.

7.		The dedicated bearer/QCI Flow of data channel is created between UE A and its local network.

8-11.	After the precondition procedure is completed, UE A initiates SCTP association and DTLS connection establishment procedures and establishes a bootstrap data channel with the MF A. Then UE A sends the data channel application list request to DCSF A. After DCSF A responses the request, UE A downloads the application (for instance, Real-time Screen Sharing application) accordingly.

6.9.2.1.3	Interworking between a DCMTSI client in terminal and an MTSI client in terminal during Application Data Channel establishment

The following procedure uses Real-time Screen Sharing application as the example. When UE A retrieves Real-time Screen Sharing application from the DCSF A and runs the application, the following procedure shown in Figure 6.9.2.1-3 is performed to transfer UE A's mobile screen data to UE B.

Figure 6.9.2.1.3-3: Application channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1~3.	UE#1 initiates a re-INVITE request to establish an application data channel for Real-time Screen Sharing application, in which the application data channel (for instance, stream 1000) and data channel application information (for instance, application ID) are contained. The IMS AS notifies the DCSF as specified in Rel-18.

4~5.	Since the DCSF has determined that remote network or UE#2 does not support or does not use IMS data channel, the DCSF decides to start one-side data channel procedure in this IMS session and to anchor the data channel media on the MF.

6~9.	The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for the application data channels (stream 1000) on originating side and video media on terminating side. The IMS AS invokdes Nmf_MediaResourceManagement to the MF to allocate resource accordingly. The DCSF also includes the instruction of how to transform data channel media to video media which is sent to the MF via IMS AS.

Editor's note:	The details of instruction of how to transform data channel media to video media is FFS and needs coordination with SA4.

10-17.	After the resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF. UE#2 returns 200 OK for the re-NVITE. DCSF is notified about the terminating network media negotiation result.

18-21.	The IMS AS sends 200 OK for re-INVITE initiated by UE#1 and UE#1 returns ACK.

22.	UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes two application data channels (stream 1000) with the MF.

The application data channels (stream 1000) between the MF and UE#1 are associated with the RTP and RTCP video streams between the MF and UE#2. The RTP and RTCP stream of UE#1's screenshot sequence is received by UE#2 from video streams.

6.9.3	Impacts on Existing Nodes and Functionality

This solution has some requirements on the existing IMS nodes below:

DCSF:

-	Determine that the terminating network UE or the terminating network does not support data channel, and the data channel has been established only in local network

-	Instruct the IMS AS to associate the data channel media resource for originating network with the video media resource for terminating network, which can reuse existing "Nimsas_MediaControl_MediaInstruction"

operation with "Media instruction" parameter setting extension.

IMS AS:

-	Report the originating network and terminating network DC negotiation result to DCSF

-	Forward the association media instruction from DCSF to MF

MF:

-	Execute the media instruction from IMS AS and associate the data channel media resource for originating network with the video media resource for terminating network

# * * * * End of changes * * * *