SA WG2 Meeting #161	S2-2402772

February 26 – March 1, 2024, Athens, Greece

# Title:	KI#2: new solution on MWAB authentication and authorization

# Source:	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

# Document for:	Approval

Agenda Item:	FS_VMR_Ph2/19.6

Abstract:	this paper provides a solution for MBSR authentication an authorization.

1. Introduction

The Rel-19 VMR work is based on the general idea to avoid impact on the deployed NG-RAN as much as possible (ideally the reuse of a general-purpose NG-RAN would be preferred, at least for basic operation). This reduces barriers to adoption (and mobility within a PLMN). Additionally, if the MWAB-UE is as close as possible to a normal UE, this will enable the implementation of a MWAB reusing existing chipsets (with minor modifications as strictly needed). For this reason, we aim at the least impact on the Access Stratum layer (which inevitably would mean impact to NG-RAN) and attempt to build the MWAB functionality by impacting the NAS layer instead.

Therefore:

1) We do not require the NG-RAN to broadcast MWAB support indication as the goal is to enable operation with any gNB.

2) We do not require the MWAB-UE to include any MWAB indication in the AS layer as the AMF selection is not a critical aspect at all, as if the initially selected AMF was incorrect at initial registration (e.g. GUTI is not valid anymore), the correct AMF can be selected by reallocation of AMF. Thereafter the GUTI can be used for the selection of the AMF.

3) The MWAB-UE can indicate its MWAB operation intention with one or both of these options:

a)	includes a MWAB indication in the Registration request, in the UE MM Core Network Capability defined in clause 5.3.4 of TS 23.501 [2].

b) 	request a NSSAI associated with the MWAB operation. Based on the S-NSSAI, the serving network can ensure the needed access control for the MWAB-UE, including providing the connection to the 5GC serving the UEs. It is assumed the value of the S-NSSAIs in the Requested NSSAI are associated to MWAB operation only.

4) During registration, the AMF retrieves subscription data from the UDM provides to the AMF in subscription data, the MWAB authorized indication with optional location and time availability if the SUPI is authorized for MWAB operation. This MWAB subscription data is separate from that of the Rel-18 MBSR subscription data. Alternatively, if option 3.b) is used exclusively for the control, the subscription data can be simply the Slice Selection Subscription data.

5) The MWAB-UE is either accepted or rejected to register. If it is rejected it shall only register if the cause code and received information from AMF allows. If it is accepted, then if establishes a PDU session to obtain connectivity to an OAM server.

6) At any time the AMF can change authorization status for the MWAB by means of UE configuration update. If an MWAB is changing to not authorized from authorized the MWAB-gNB triggers handover of the UEs it serves to other cells of other gNBs if possible. Then MWAB-UE may release the NG-related PDU sessions. The AMF may also release the PDU sessions of the MWAB-UE if it is configured to do so after a certain amount of time. The MWAB then stops service (which may cause any remaining UEs that could not be handed over to experience RLF, unless the MWAB had released their RRC connection before going out of service.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# Proposed text

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | 2 |  |  |
| x |  | x |  |  |

# Proposed text - ALL NEW TEXT

## 6.X	Solution #x: Authorization and Change of Authorization of a MWAB and configuration of a MWAB

### 6.X.1	General

This is a Solution addressing Key Issue #2: Authorization of a MWAB and configuration of MWAB

### 6.X.2	Functional descriptions

The solution is based on this outline:

-	The MWAB-UE optionally includes a MWAB indication in the Registration request, in the UE MM Core Network Capability defined in clause 5.3.4 of TS 23.501 [2]. Alternatively, or in addition, the MWAB-UE may be configured by the HPLMN a specific (set of) S-NSSAI(s) associated with the MWAB operation, and the MWAB-UE includes the S-NSSAI in the Registration Request.

-	During registration, the AMF retrieves subscription data and the UDM provides to the AMF in subscription data, the MWAB authorized indication with optional location and time availability if the SUPI is authorized for MWAB operation.

-	The MWAB-UE is either accepted or rejected, if it is rejected it shall only register if the cause code and received information from AMF allows. If it is accepted, then if establishes a PDU session to obtain connectivity to an OAM server.

-	At any time the AMF can change authorization status for the MWAB by means of UE configuration update. If a MWAB is changing to not authorized from authorized the MWAB-gNB triggers handover of the UEs it serves to other cells of other gNBs. Then MWAB-UE may release the NG related PDU sessions. The AMF may also release the PDU sessions of the MWAB-UE if it is configured to do so after a certain amount of time. The MWAB then stops service (which may cause any remaining UEs that could not be handed over to experience RLF, unless the MWAB had released their RRC connection before going out of service).

### 6.X.3	Procedures

### 6.X.3.1	MWAB service authorization and MWAB gNB configuration

Figure 6.x.3.1-1: MWAB service authorization and initial MWAB-gNB configuration

1.	The MWAB-UE registers and optionally includes in the UE MM Core network capability defined in clause 5.3.4 of TS 23.501[2] an indication it is intending to act as MWAB. It is assumed the MWAB-UE is configured with a list of PLMNs it is not allowed to register with and any Preferred PLMNs list as usual for a UE. Alternatively, or in addition,  the MWAB-UE may be configured by the HPLMN with a (set of) S-NSSAI(s) associated with the MWAB operation and requests it in the registration procedure.

2.	The AMF retrieves the subscription data and checks whether the UE is authorized to act as MWAB by checking presence of MWAB Operation Allowed and any related location and time information. The AMF provides a default S-NSSAI for MWAB-UE if no S-NSSAI was requested by the MWAB-UE. The MWAB-UE also obtains any Configured NSSAI as applicable.

3.	If the MWAB-UE subscription allows, the MWAB-UE is accepted and an indication that MWAB operation is allowed is sent to the MWAB-UE, and the Allowed NSSAI including the S-NSSAI(s) for MWAB operation as applicable.

4. 	The MWABUE establishes a PDU session to get initial configuration information for the MWAB-gNB from the OAM server. The PDU session establishment Request may include S-NSSAI and DNN depending on the MWAB-UE configuration. If no DNN is included, the AMF determines the correct one to use for the MWAB.

5.	The MWAB-UE provides to the MWAB gNB the IP address it can use to contact the OAM server.

6.  The MWAB-gNB contact the OAM server and obtains configuration

Editor's Note: how the MWAB-gNB obtains the IP address of the OAM server is FFS

7.	The MWAB-gNB establishes, based on how it has been configured by the OAM server, any additional PDU sessions to then use the for N2 and N3 connections as necessary.

### 6.X.3.2	MWAB change of service authorization

Figure 6.x.3.2-1: MWAB change of service authorization

1.	The UDM may provide updated subscription data changing the MWAB authorization status to (not) authorized.

2.	The AMF determines whether the MWAB authorization status has changed to Allowed or Not Allowed, and if so, the AMF updates the MWAB-UE with the new MWAB authorization status by a UE configuration update procedure.

3.	The MWAB-UE acknowledges the reception of the message.

4a. If the authorization state was changed to MWAB Operation Allowed, the MWAB continues from step 4 of figure Figure 6.x.3.1-1. And all other steps are skipped.

4b.	If the authorization state was changed to MWAB Operation Not Allowed, then the MWAB-gNB hands over to other gNBs the UEs it serves.

5.	The MWAB-gNB stops serving any UE and informs OAM it is out of service.  The MWAB-gNB also releases all the NG connections to the AMFs if is connected to over the B/H PDU sessions.

6.	The MWAB-gNB informs the MWAB-UE it has to release the b/h sessions it had requested earlier to establish.

7.	The MWAB-UE releases all the b/h PDU sessions.

8. 	The AMF or the MWAB-UE may initiate deregistration procedure when there are no more b/h PDU sessions based on policy. If so, the AMF may provide a suitable cause code and location/time information.

### 6.X.4	Impacts on services, entities, and interfaces

AMF: ability to authorize and update authorization state and support MWABs as per message flows above.

MWAB (new): support the interactions with the OAM and 5GS as outlined above for authorization and change of authorization state.

OAM: configuration of MWAB.

# End of proposed text