SA WG2 Meeting #161	S2-2402797

Athens, Greece, February 26 – March 1, 2024	(was S2-240xxxx)

Source:	Charter Communications, CableLabs, Tencent, Tencent Cloud, Nokia, Nokia Shanghai Bell

Title:	KI#6: Update Solution #17: L4S in non-3GPP access networks

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: As a result of merging multiple pCRs into one pCR at SA2#160AHE, this paper proposes editorial changes to Solution #17 for ease of reading, addresses the Editor Notes, and provides several minor corrections.

# 1. Introduction

This pCR proposes editorial updates to Solution #17: L4S in non-3GPP access networks, fixes all instances of the Note related to when the UPF performs ECN marking for L4S, as well as proposals for the Editor’s Notes added during SA2#160AHE.

Wireline and wireless solutions were merged during SA2#160AHE, but the general description clause doesn’t fully align with the subsequent clauses. Clause 6.17.2.1 is updated to align with the subsequent clauses and a general diagram of non-3GPP intermediate nodes accessing a 3GPP 5G Core network was added. Also, the wireless solutions (i.e., clauses 6.17.2.4 and 6.17.2.5) is re-arranged to clearly differentiate the alternative solutions for when congestion is detected and when ECN marking for L4S is performed in the uplink and downlink.

Clauses 6.17.4.1 and 6.17.4.2 title changed to include non-3GPP to clarify that the impacted entities are related to only non-3GPP deployments support for L4S.

In addition, each Editor’s note is discussed and a proposed resolutions is provided.

In Figure 6.17.2.4.2-2 Congestion in uplink detected at UE via non-3GPP un-trusted access there is an EN copied below:

Editor's note: How the UE knows that it needs to provide congestion info and how UEs know the GRE  configuration is FFS.

The UE would need to receive a NAS-SM message from the SMF, similar to how the SMF (i.e., based on operator’s network configuration and policies) decides whether the NG-RAN or PSA UPF based ECN marking for L4S is used in R18 (see TS23.501[2] clause 5.37.3). This option would also require support in the GRE header, which is feasible since the GRE header already carries QFI value and RQI indicator for reflective QoS per PDU.

Proposal 1: Remove the above EN and add the following text to step 2 of Figure 6.17.2.4.2-2 and update clause 6.17.4.2 to indicate changes to SMF and GRE header:

Step 2 addition:

The UE would need to receive a NAS-SM message from the SMF indicating that the PSA UPF is instructed to perform ECN marking for L4S for non-3GPP untrusted access, similar to how the SMF (i.e., based on operator’s network configuration and policies) decides whether the NG-RAN or PSA UPF based ECN marking for L4S is used in R18 (see TS23.501[2] clause 5.37.3). This option would also require support in the GRE header, which is feasible since the GRE header already carries QFI value and RQI indicator for reflective QoS per PDU.

Clause 6.17.4.2 additions:

SMF:

- Update NAS-SM message to UE from SMF to include an indication that the PSA UPF is instructed to perform ECN marking for L4S for non-3GPP untrusted access and that the UE is instructed to perform congestion information monitoring and report to the N3IWF the congestion information via a GRE message to the N3IWF, which would then send the congestion information to the PSA UPF via a GTP-U message.

In Figure 6.17.2.4.1-3 Congestion in downlink detected at WLAN AP of non-3GPP untrusted access there is an EN copied below:

Editor's note: How this may be possible for an N3IWF is FFS.

One way that the N3IWF could estimate congestion on behalf of the access point, is by accumulating the number of ECN markings in the outer IP header received over a configuration window, since the intent of the ECN marking is to provide statistical congestion information, and then send this statistic to the PSA UPF via a GTP-U message. It is assumed other mechanisms are also feasible and leaves this as based on implementation.

Proposal 2: Remove the EN and correct 1st sentence in step 5 to indicate DL not UL, this was a copy paste error in original pCR.

In clause 6.17.4.1 Wireline 5G access network there are two EN’s related to the impact to the SMF and 5G-RG copied below:

Editor’s Notes: The necessaries of negotiation between SMF and 5G-RG for ECN marking for L4S is FFS.

Editor’s Notes: The impacts on SMF and 5G-RG is FFS.

In the case of wireline access, such as in cable access network, a dual-queue mechanism is used to separate queue-building traffic (non-L4S traffic) from non-queue-building traffic (L4S traffic). To correctly direct different traffic to the appropriate queues, classifiers associated with L4S Service Flows are used. For handling of UL traffic, 5G-RG is responsible for mapping the QoS rule(s) received in NAS to W-UP level QoS. In case of a 5G-CRG, QoS rules(s) are mapped to DOCSIS service flows, as indicated by the protocol stacks in the figures below.

Fig. 1 Control Plane Protocol Stacks for W-5GAN

Fig. 2 User Plan Protocol Stacks for W-5GAN

Therefore, to correctly map the L4S-enabled QoS Flow(s) to L4S-enabled W-UP resources (e.g. DOCSIS Service Flows), 5G-RG needs information from the SMF about which QoS Flow(s) are L4S enabled through NAS signalling. If the RG does not have this information from the 5GC, it could incorrectly map UL L4S traffic to non-L4S-enabled Service Flows based on QoS Rule(s) it received. Figures 3 and 4 show the 5G-CRG order of operations for an UL packet. If the 5G-RG is not aware of the dedicated L4S-enabled QoS Flow(s), as shown in Figure 3, it will determine a QFI that would map to a non-L4S Service Flow for the UL L4S traffic.

Fig. 3 Incorrect operation if 5G-RG is not aware about dedicated L4S-enabled QoS Flows

If the 5G-RG is aware about the dedicated L4S-enabled QoS Flow(s), as shows in Figure 4, it will first determine a QFI corresponding to a L4S-enabled QoS Flow for the UL L4S traffic, and then correctly map it to a L4S Service Flow.

Fig. 4 Correct operation if 5G-RG is aware about dedicated L4S-enabled QoS Flows

Therefore, it is essential for 5G-RG to receive information about L4S-enabled QoS Flow(s) via NAS signalling.

Proposal 3: Remove ENs in clause 6.17.4.1 based on the procedures defined in clause 6.17.3.1.

#### In clause 6.17.4.2. Wireless 5G access network there is an EN copied below:

Editor’s Notes: The detailed impacts caused by draft-ietf-tsvwg-ecn-encap-guidelines-22[16] are FFS, as well as the impacts of feature parity with 3GPP access. The impact for different alternatives will be further separated and clearly clarified.

This IETF draft (i.e., draft-ietf-tsvwg-ecn-encap-guidelines-22) provides guidelines for adding congestion notification to protocols that encapsulate IP and RFC 6040[a] redefines how the explicit congestion notification field of the IP header should be constructed on entry to and exit from any IP-in-IP tunnel. The impact is to copy the ECN field to the inner-most IP header (i.e., Application/PDU IP header) so that the receiving application can act accordingly.

For the different alternatives, the impact clause is updated to clearly indicate which nodes are implicated per the alternative solution.

Proposal 4: Remove EN and clause 6.17.4.2 updated to clearly indicate the impacted nodes for each alternative solution.

# 2. Proposal

This contribution proposes updates to Solution #17 in TR 23.700-70 clause 6.17.

Start of 1st changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: “Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP”.

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19] 	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	IETF TS26.522: "5G Real-time Media Transport Protocol Configurations".

[21] 	IETF ﻿draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32] 	IETF RFC 8627: " RTP Payload Format for Flexible Forward Error Correction (FEC) ".

[a] 	IETF RFC 6040: " Tunnelling of Explicit Congestion Notification".

Editor's note:	References [8], [9], [10], [16], and [21] cannot be formally referenced until published as RFC.

Start of 2nd changes

## 6.17	Solution #17: L4S in non-3GPP access networks

### 6.17.1	Key Issue mapping

This solution proposes a solution for KI #6.

### 6.17.2	Description

#### 6.17.2.1	General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [14], IETF RFC 9331 [15] and IETF RFC 9332 [19]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation. In 5G System ECN marking for L4S in the IP header is supported in either the NG-RAN (see TS 23.501 [2] clause 5.37.3.2), or in the PSA UPF (see TS 23.501 [2] clause 5.37.3.3) via NG-RAN. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows.

In extending ECN marking for L4S via non-3GPP access the same principles as above for NG-RAN are proposed in the current solution. Namely, ECN marking for L4S is enabled on a per QoS Flow basis. Dedicated non-3GPP access resources are used for carrying L4S-enabled IP traffic. For DL, intermediate non-3GPP access nodes (N3IWF, TNGF, W-AGF) map the L4S-enabled QoS Flows to the proper non-3GPP access resources. For UL UE/5G-RG maps the L4S enabled QoS Flows to proper non-3GPP access resources. An AF might request enabling ECN marking for L4S via the Nnef_AFsessionWithQoS service operation. Alternatively, ECN marking for L4S may be enabled by the SMF based on dynamic or static policies for the DNN/S-NSSAI of the PDU session. The SMF provides an indication for ECN marking for L4S for a corresponding QoS Flow in N2, N1 signalling. ECN marking for L4S might apply to GBR flows or non-GBR flows. It might also apply to uplink and/or downlink direction. QoS rules in the UE/5G-RG and PDRs in the PSA UPF determine the PDUs bound to the L4S enabled QoS Flow.

Figure 6.17.2.1-1 provides a high-level architectural view of nodes connected to a 3GPP 5G Core Network. For non-3GPP access,

ECN marking could be supported in the following nodes:

- Non-3GPP access: Non-3GPP access node(s) perform ECN marking for downlink/uplink.

- UE/5G-RG: UE/5G-RG performs ECN marking for downlink/uplink.

Figure 6.17.2.1-1 Architecture view of nodes connecting to a 3GPP 5G Core Network

For an IP packet via non-3GPP access, ECN marking could occur in one of the above components. Based on L4S behaviour as described in IETF RFC 9331 [15], congestion indication/marking by a component is performed only when congestion indication has not been set by previous component in a route. Depending on capabilities of each component, ECN marking on a route may be supported by a subset of the above, none of the above, or all the above.

The solutions provided propose that 3GPP-defined non-3GPP access nodes (i.e., N3IWF, TNGF, W-AGF, and UE/5G-RG) provide mechanisms to extend support for L4S in non-3GPP accesses. The proposed solutions have the following principles:

- Dedicated UP resources are used for carrying L4S-enabled IP traffic.

- N3IWF, TNGF, W-AGF, and UE/5G-RG maps the L4S-enabled QoS Flows to UP resources.

- Any non-3GPP intermediate node (i.e., UE/RG, W-AGF, N3IWF, & TNGF) acting as an IP-in-IP tunnel endpoint between the XR application client and server is assumed to implement encapsulation and decapsulation as specified in RFC 6040[a] and where applicable supports draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

-	Option for the UE/5G-RG to perform ECN marking when UL congestion is experienced at the UE.

The criteria based on which non-3GPP access elements including intermediate non-3GPP access nodes (N3IWF, TNGF, W-AGF) decide to mark ECN bits when congestion occurs are implementation specific.

Fig 6.17.2.1-1 L4S support in wireline access

For wireline cable access network e.g., DOCSIS, provides support for low-latency services through the dual-queue approach, by separating queue-building and non-queue-building traffic [18]. DOCSIS, such as Low Latency DOCSIS (LLD), also supports ECN marking for L4S, where L4S traffic would be managed separately from other class traffic (e.g., separate low latency service flow from other classic service flow(s)).

#### 6.17.2.2	Support of ECN marking for L4S in wireline node(s)

For wireline cable access network e.g., DOCSIS, provides support for low-latency services through the dual-queue approach, by separating queue-building and non-queue-building traffic [18]. DOCSIS, such as Low Latency DOCSIS (LLD), also supports ECN marking for L4S, where L4S traffic would be managed separately from other class traffic (e.g., separate low latency service flow from other classic service flow(s)).

Figure 6.17.2.2-1 L4S support in wireline access

In case of DL direction, when the W-AGF receives N2 requests related with PDU Session resources, the W-AGF maps the QoS profile(s) received from the 5GC to W-UP level QoS. The SMF may be instructed, based on either dynamic or predefined PCC rule, to provide an indication for L4S for a corresponding QoS Flow(s) to W-AGF. W-AGF maps the L4S-enabled QoS Flow(s) to L4S-enabled wireline QoS resource(s).

When the W-AGF receives a DL PDU via N3, it identifies the QFI from the GTP-U header, and if the QFI corresponds to a QoS Flow with L4S enabled, it determines the corresponding L4S-enabled wireline QoS resource to use for sending the DL PDU to the RG.

Details on ECN marking for L4S by wireline nodes (such as a CMTS) are outside of 3GPP scope.

In case of UL, when the 5G-RG receives NAS message related with PDU Session QoS, the 5G-RG maps the QoS rule(s) received in NAS to W-UP level QoS. The SMF may be instructed, based on either dynamic or predefined PCC rule, to provide an indication for ECN marking for L4S for a corresponding QoS Flow(s) to the 5G-RG. 5G-RG maps the L4S-enabled QoS Flow(s) to L4S-enabled wireline QoS resources(s).

When the 5G-RG transmits an UL PDU, if it determines (by using the QoS rules of the PDU Session) that the QFI corresponds to a QoS Flow with L4S enabled, it determines the corresponding L4S-enabled wireline QoS resource to use for sending the UL PDU to the W-AGF.

#### 6.17.2.3	Support of ECN marking for L4S in wireline 5G-RG deployments

5G-RG may be requested by SMF to perform ECN marking for L4S in the IP header of the user IP packets, based on its local congestion conditions, for those QoS flows for which it received an indication for L4S in the QoS rule(s) via the NAS message. The criteria based on which 5G-RG decides to mark ECN bits for L4S is 5G-RG implementation specific.

##### 6.17.2.4	Supporting of ECN marking for L4S in wireless N3IWF deployments

It is most probable that if congestion is experienced in the uplink, it is related to the wireless link (i.e., WiFi AP).  3GPP does not have responsibility for the AP specification but can relay any congestion notification that the WiFi AP provides to the inner most IP layer at the N3IWF. This is accomplished by leveraging IETF RFC 6040 [a] and IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [16], which provides Tunnelling of Explicit Congestion Notification and Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP, respectively.

Figures 6.17.2.4-1 and 6.17.2.4-2 illustrate when congestion is detected in the UL by the WLAN AP and the UE, respectively.

Figure 6.17.2.4-1 Congestion in uplink detected at WLAN AP of non-3GPP untrusted access.

Figure 6.17.2.4-1 illustrates all possible ways of how support for L4S is accomplished when congestion is detected at the WLAN AP in the uplink. Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.4-1) N3IWF performs marking: steps 1, 2, 3.

b.	option 2 (Figure 6.17.2.4-1) UPF performs marking: steps 1, 2, 4, 5 and/or 1, 2, 4, 6.

i. 	Note that if UPF is doing marking (i.e., steps 1, 2, 4, 5), it receives congestion information from the N3IWF as indicated in step 4.

- WLAN AP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- N3IWF detects outer IP header ECN field is set indicating congestion.

- N3IWF relays information to inner-most IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- N3IWF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and if the PSA UPF is responsible for performing the ECN marking in the UL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF but may not be able to provide congestion information experienced by the WiFi AP. Based on implementation it may be possible for the N3IWF to estimate congestion information relative to the access point (i.e., level of congestion of WLAN AP) and provide via GTP-U header.

- N3IWF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- N3IWF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and UPF marks ECN field as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] indicating congestion was experienced.

If congestion is experienced in the uplink by the N3IWF, not illustrated, it is capable of setting the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

If congestion is experienced in the uplink by the N3IWF and the WiFi AP, the ECN marking provided by the WiFi AP (if supported) would take precedent and relayed to the inner most IP layer for use by the receiving application. If the congestion in the uplink at the N3IWF persists, it will eventually perform the ECN marking once the congestion from the WiFi AP is removed.

A 3GPP UE may also provide uplink support for L4S when connecting to a 5GS via a non-3GPP access.

Figure 6.17.2.4-2 Congestion in uplink detected at UE via non-3GPP un-trusted access.

Figure 6.17.2.4-2 illustrates all possible ways of how support for L4S is accomplished when congestion is detected in the uplink at the UE when accessing a 5GS via a non-3GPP un-trusted access (i.e., WLAN AP + N3IWF). Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.4-2) UE performs marking: step 1.

b.	option 2 (Figure 6.17.2.4-2) UPF performs marking: steps 2, 3, 4, 5.

i.	Note that if UPF is doing marking, it receives congestion information from the N3IWF as indicated in step 4 (i.e., from the UE via GRE packet sent to N3IWF, which then copies to a GTP-U packet to the UPF).

- UE indicates via inner-most IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the UL, the UE can provide congestion information related to congestion experienced by the UE via GRE to the N3IWF. The UE would need to receive a NAS-SM message from the SMF indicating that the PSA UPF is instructed to perform ECN marking for L4S for non-3GPP untrusted access, similar to how the SMF (i.e., based on operator’s network configuration and policies) decides whether the NG-RAN or PSA UPF based ECN marking for L4S is used in R18 (see TS23.501 [2] clause 5.37.3). This option would also require support in the GRE header, which is feasible since the GRE header already carries QFI value and RQI indicator for reflective QoS per PDU.

- N3IWF detects GRE message and provides the UE’s congestion information on the GTP-U header to UPF.

- N3IWF sends GTP-U message containing congestion information to UPF.

- UPF receives congestion information from the GTP-U header and set the ECN field in the inner-most IP header accordingly.

Like uplink, it is most probably if congestion is experienced in the downlink it is related to the wireless link (i.e., WiFi AP) and 3GPP does not have responsibility for the AP specification but can relay any congestion notification that the WiFi AP provides to the inner most IP layer at the UE. This is accomplished by leveraging IETF RFC 6040 [a] and IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [16], which provides Tunnelling of Explicit Congestion Notification and Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP, respectively.

Figures 6.17.2.4-3 and 6.17.2.4-4 illustrate when congestion is detected in the DL by the WLAN AP and the N3IWF, respectively.

Figure 6.17.2.4-3 Congestion in downlink detected at WLAN AP of non-3GPP untrusted access.

Figure 6.17.2.4-3 illustrates all possible ways of how support for L4S is accomplished when congestion is detected at the WLAN AP in the downlink. Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.4-3) UE performs marking: steps 1, 2.

b.	option 2 (Figure 6.17.2.4-3) N3IWF performs marking: steps 1, 3, 4.

c.	option 3 (Figure 6.17.2.4-3) UPF performs marking: steps 1, 3, 5, 6 and/or 1, 3, 5, 7.

i.	Note that if UPF is doing marking (i.e., steps 1, 3, 5, 6), it receives congestion information from the N3IWF as indicated in step 5.

- WLAN AP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- UE detects outer IP header ECN field is set indicating congestion. UE relays information to inner IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- N3IWF detects outer IP header ECN field is set indicating congestion. How the WLAN AP copies the ECN field towards the N3IWF (i.e. UL packets) when congestion is experienced at the AP in the DL needs further investigation and may not be possible.

- N3IWF relays information to inner-most IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- N3IWF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and if the PSA UPF is responsible for performing the ECN marking in the DL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF but it may not be able to provide congestion information experienced at the WiFi AP. Based on implementation it may be possible for the N3IWF to estimate congestion information relative to the access point (i.e., level of congestion of WLAN AP) and provide via GTP-U header.

- N3IWF provides GTP-U header with congestion information to UPF and UPF marks ECN field in the inner-most header indicating congestion was experienced.

- N3IWF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and UPF marks ECN field of the inner-most IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] indicating congestion was experienced.

In the downlink the N3IWF supports the same reference point N3, as the NG-RAN and should be able to support similar L4S functionality as the NG-RAN. Such as:

- Method 1: Performing ECN marking according to IETF RFC 9330 [14]and RFC 9331 [15] for downlink in IP layer of the received packets. Also, dedicated QoS flow(s) can be used for carrying L4S enabled IP traffic.

- Method 2: If the PSA UPF performs the ECN marking in the downlink, the N3IWF can provide the congestion information via the GTP-U header. The N3IWF shall also be able to receive an indication from the SMF to report congestion information (i.e., a percentage of packets that UPF uses for ECN marking for L4S) of the QoS flow on DL direction via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL needs to be provided the N3IWF may generate an UL Dummy GTP-U packet for such reporting. That is, if congestion is experienced in the downlink by the N3IWF, it can set the IP header ECN field of the inner IP as well as provide congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

Figure 6.17.2.4-4 Congestion in downlink detected at N3IWF of non-3GPP untrusted access.

Figure 6.17.2.4-4 illustrates how support for L4S is accomplished when congestion is detected at the N3IWF in the downlink.  Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.4-4) N3IWF performs marking: steps 1.

b.	option 2 (Figure 6.17.2.4-4) UPF performs marking: steps 1, 2, 3.

i.	Note that if UPF is doing marking, it receives congestion information from the N3IWF as indicated in step 2.

- N3IWF indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the DL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF via GTP-U header.

- UPF receives congestion information from the GTP-U header and set the ECN field accordingly.

##### 6.17.2.5	Supporting of ECN marking for L4S in wireless TNAN deployments

Trusted (i.e. TNAN) access support for L4S is similar to un-trusted, although 3GPP has responsibility for specifying NWt we can leverage IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] as described in clause 6.17..2.4.

Figure 6.17.2.5-1 Congestion in uplink detected at TNAP of non-3GPP trusted access.

Figure 6.17.2.5-1 illustrates all possible ways of how support for L4S is accomplished when congestion is detected in the uplink at the TNAP. Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.5-1) TNGF performs marking: steps 1, 2, 3.

b.	option 2 (Figure 6.17.2.5-1) UPF performs marking: steps 1, 2, 4, 5 and/or 1, 2, 4, 6.

i.	Note that if UPF is doing marking, it receives congestion information from the TNGF as indicated in step 4.

- TNAP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- TNGF detects outer IP header ECN field is set indicating congestion.

- TNGF relays information to inner IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- TNGF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and if the PSA UPF is responsible for performing the ECN marking in the UL, the TNGF can provide congestion information related to congestion experienced by the TNGF but may not be able to provide congestion information experienced by the TNAP. Based on implementation it may be possible for the TNGF to estimate congestion information relative to the access point (i.e., level of congestion of TNAP) and provide via GTP-U header.

- TNGF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- TNGF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and UPF marks ECN field as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] indicating congestion was experienced.

If congestion is experienced in the uplink by the TNGF, not illustrated, it is capable of setting the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

If congestion is experienced in the uplink by the TNGF and the WiFi AP, the ECN marking provided by the WiFi AP (if supported) would take precedence and relayed to the inner most IP layer for use by the receiving application. If the congestion in the uplink at the TNGF persists, it will eventually perform the ECN marking once the congestion from the TNAP is removed.

Figures 6.17.2.5-2 and 6.17.2.5-3 illustrate when congestion is detected in the DL by the TNAP and the TNGF, respectively.

Like uplink, it is most probably if congestion is experienced in the downlink it is related to the wireless link (i.e., TNAP) and 3GPP does not have responsibility for the AP specification but can relay any congestion notification that the TNAP provides to the inner most IP layer at the UE. This is accomplished by leveraging IETF RFC 6040 [a] and IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [16], which provides Tunnelling of Explicit Congestion Notification and Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP, respectively.

Figure 6.17.2.5-2 Congestion in downlink detected at TNAP of non-3GPP trusted access.

Figure 6.17.2.5-2 illustrates all possible ways of how support for L4S is accomplished when congestion is detected at the TNAP in the downlink. Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.5-2) UE performs marking: steps 1, 2.

b.	option 2 (Figure 6.17.2.5-2) TNGF performs marking: steps 1, 3, 4.

c.	option 3 (Figure 6.17.2.5-2) UPF performs marking: steps 1, 3, 5, 6 and/or 1, 3, 5, 7.

i.	Note that if UPF is doing marking (steps 1, 3, 5, 6), it receives congestion information from the TNGF as indicated in step 5.

- TNAP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- UE detects outer IP header ECN field is set indicating congestion. UE relays information to inner IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- TNGF detects outer IP header ECN field is set indicating congestion.

- TNGF relays information to inner IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

- TNGF relays information to outer IP header towards UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and if the PSA UPF is responsible for performing the ECN marking in the DL, the TNGF can provide congestion information related to congestion experienced by the TNGF but may not be able to provide congestion information experienced by the TNAP. Based on implementation it may be possible for the TNGF to estimate congestion information relative to the access point (i.e., level of congestion of TNAP) and provide via GTP-U header.

- TNGF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- TNGF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] and UPF marks ECN field of inner-most IP header as specified in IETF RFC 6040 [a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16] indicating congestion was experienced.

In the downlink the TNGF supports the same reference point N3, as the NG-RAN and should be able to support similar L4S functionality as the NG-RAN. Such as:

- Method 1: Performing ECN marking according to IETF RFC 9330 [14] and RFC 9331 [15] for downlink in IP layer of the received packets. Also, dedicated QoS flow(s) can be used for carrying L4S enabled IP traffic.

- Method 2: If the PSA UPF performs the ECN marking in the downlink, the TNWF can provide the congestion information via the GTP-U header. The TNGF shall also be able to receive an indication from the SMF to report congestion information (i.e., a percentage of packets that UPF uses for ECN marking for L4S) of the QoS flow on DL direction via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL needs to be provided the TNGF may generate an UL Dummy GTP-U packet for such reporting. That is, if congestion is experienced in the downlink by the TNGF, it can set the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

Figure 6.17.2.5-3 Congestion in downlink detected at TNGF of non-3GPP trusted access.

Figure 6.17.2.5-3 illustrates how support for L4S is accomplished when congestion is detected at the TNGF in the downlink.  Which steps are executed depends on which option is followed from the following alternatives:

a.	option 1 (Figure 6.17.2.5-3) TNGF performs marking: steps 1.

b.	option 2 (Figure 6.17.2.5-3) UPF performs marking: steps 1, 2, 3.

i.	Note that if UPF is doing marking, it receives congestion information from the N3IWF as indicated in step 2.

- TNGF indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the DL, the TNGF can provide congestion information related to congestion experienced by the TNWF via GTP-U header.

- UPF receives congestion information from the GTP-U header and set the ECN field accordingly.

A 3GPP UE may also provide uplink support for L4S when connecting to a 5GS via a non-3GPP access.

Figure 6.17.2.4.3-1 Congestion in uplink detected at UE via non-3GPP un-trusted access.

Figure 6.17.2.4.3-1 illustrates how support for L4S is accomplished when congestion is detected in the uplink at the UE when accessing a 5GS via a non-3GPP un-trusted access (i.e., WLAN AP + N3IWF).

- UE indicates via inner-most IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the UL, the UE can provide congestion information related to congestion experienced by the UE via GRE to the N3IWF.

Editor's note: How the UE knows that it needs to provide congestion info and how UEs know the GRE  configuration is FFS.

- N3IWF detects GRE message and provides the UE’s congestion information on the GTP-U header to UPF.

- N3IWF sends GTP-U message containing congestion information to UPF.

- UPF receives congestion information from the GTP-U header and set the ECN field in the inner-most IP header accordingly.

In summary, Figure 6.2.2.4.3-1 illustrates the following option for ECN marking when congestion is detected in the UE in an untrusted deployment:

1.	UL

a.	option 1 (Fig 6.2.2.4.3-1) UE performs marking: step 1.

b.	option 2 (Fig 6.2.2.4.3-1) UPF performs marking: steps 2, 3, 4, 5.

i.	Note that if UPF is doing marking, it also provides config info.

### 6.17.3	Procedures

#### 6.17.3.1	Procedures in wireline 5G access network

Fig. 6.17.3.1-1 High-level procedure of the solution

- Steps 1-2a specified in TS 23.316 [17] clause 7.3.1.1, with the difference that the 5G-RG 5GSM Core Network Capability included in the PDU Session Establishment Request within the N1 SM container indicates whether the 5G-RG supports ECN marking for L4S.

2a. The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: For each QoS Flow, an ECN marking for L4S indicator to W-5GAN in the case of ECN marking for L4S in W-5GAN.

The N1 SM container that the AMF shall provide to the 5G-RG contains the: For each QoS Flow, an ECN marking for L4S indicator to 5G-RG in the case of ECN marking for L4S in the 5G-RG, if the 5G-RG indicated support for ECN marking for L4S.

2b. The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to W-AGF to establish the access resources for this PDU Session.

3. 	Based on the QoS flows and QoS parameters received in the previous step, W-AGF determines the corresponding L4S-enabled wireline QoS resource needed for the PDU session.

4.	The W-AGF sets up the W-UP resources for the PDU session.

5.	After all W-UP resources are established, the W-AGF shall forward to 5G-RG via the W-CP signalling connection the PDU Session Establishment Accept message (including the ECN marking for L4S indicator) received in step 2b.

6.	The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response including: established QoS Flows status (active/not active) for ECN marking for L4S in wireline access.

7.	All steps specified in TS 23.502 [3] clause 4.3.2.2.1 after step 14 are executed according to the PDU Session Establishment procedure over 3GPP access.

### 6.17.4	Impacts on services, entities and interfaces

#### 6.17.4.1	Wireline non-3GPP 5G access network

SMF:

- Indication of ECN marking for L4S to 5G-RG over N1

- Configuration of QoS profiles to W-AGF including L4S handling

W-AGF:

- Mapping of L4S-enabled QoS profile to L4S-enabled W-UP resource

- Supports ECN marking for L4S like NG-RAN node as specified in TS 23.501[2]

5G-RG:

- Mapping of L4S-enabled QoS rule to L4S-enabled W-UP resource

- ECN marking for L4S in the IP header of the user IP packets

- Indication of support for ECN marking for L4S during PDU Session Establishment Request

#### Wireless non-3GPP 5G access network

Table 6.17.4.2-1 summarizes the alternative solutions described in clauses 6.17.2.4 and 6.17.2.5

Table 6.17.4.2-1: Summary of alternative solutions

| Congestion Detected in |  | ECN Marking for L4S performed by |  |  |  |  |  |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE |  |  | N3IWF |  | TNGF |  | PSA UPF |  |  |  |
| UL @ UE |  | Figure 6.17.2.4-2 step 1 |  |  |  |  |  |  | Figure 6.17.2.4-2 steps 2,3,4,5 |  | For UL congestion detected at the UE, the UE can directly set the ECN codeword in the application IP header and can also relay congestion information to the PSA UPF via GRE/GTP-U message if the PSA UPF is instructed to perform ECN marking for L4S. |  |
| UL @ AP |  |  |  |  | Figure 6.17.2.4-1 steps 1,2,3 |  | Figure 6.17.2.5-1 step 1,2,3 |  | Figure 6.17.2.4-1 steps 1,2,4,6 or steps 1,2,4,5 Figure 6.17.2.5-1 steps 1,2,4,6 or steps 1,2,4,5 |  | For UL congestion detected at the AP, the N3IWF or TNGF can perform ECN marking for L4S. If PSA UPF is instructed to perform ECN marking for L4S on behalf of the AP/IWF either solution is acceptable. |  |
| UL @ IWF |  |  |  |  |  |  |  |  |  |  | TBD |  |
| DL @ IWF |  |  |  |  | Figure 6.17.2.4-4 step 1 |  | Figure 6.17.2.5-3 step 1 |  | Figure 6.17.2.4-4 steps 2,3 Figure 6.17.2.5-3 steps 2,3 |  | For DL congestion detected at the IWF, the N3IWF or TNGF can perform ECN marking for L4S. If PSA UPF is instructed to perform ECN marking for L4S on behalf of the IWF. |  |
| DL @ AP | Figure 6.17.2.4-3 steps 1,2 Figure 6.17.2.5-2 steps 1, 2 | Figure 6.17.2.4-3 steps 1,3,4 | Figure 6.17.2.5-2 step 1,3,4 | Figure 6.17.2.4-3 steps 1,3,5,6 or steps 1,3,5,7 Figure 6.17.2.5-2 steps 1,3,5,6 or steps 1,3,5,7 | For DL congestion detected at the AP, the N3IWF or TNGF can perform ECN marking for L4S. If PSA UPF is instructed to perform ECN marking for L4S on behalf of the AP/IWF either solution is acceptable. |
| DL @ UE |  |  |  |  |  |  |  |  |  |  | TBD |  |

SMF:

- Configures QoS profiles to N3IWF/TNGF as to NG-RAN for L4S handling in all alternative solutions described above.

- Update NAS-SM message to UE from SMF to include an indication that the PSA UPF is instructed to perform ECN marking for L4S for non-3GPP untrusted access and that the UE is instructed to perform congestion information monitoring and report to the PSA UPF the congestion information via a GRE message to the UPF, which would then send the congestion information to the PSA UPF via a GTP-U message.

UPF:

- Same functionality/support for L4S as R18 NG-RAN in all alternative solutions described above when UPF performs ECN marking for L4S.

- Supports RFC 6040[a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

N3IWF:

- N3IWF supports L4S handling like NG-RAN/gNB as N2/N3 termination point including ECN marking for L4S in all alternative solutions described above when N3IWF is deployed.

- Supports RFC 6040[a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

TNGF:

- TNGF supports L4S handling like NG-RAN/gNB as N2/N3 termination point and support ECN marking for L4S in all alternative solutions described above when TNGF is deployed.

- Supports RFC 6040[a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

UE:

- Support ECN marking for L4S as illustrated in Figure 6.17.2.4-2 step 1; Figure 6.17.2.4-3 steps 1; Figure 6.17.2.5-2 steps 1, 2

- Supports RFC 6040[a] and draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [16].

NOTE: 	Clauses 6.17.4.1 and 6.17.4.2 include impact for several alternatives together and it is assumed only one alternative will be supported.

End of changes