3GPP TSG-WG SA2 Meeting #161	S2-2402831

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Terms for DualSteer

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: The contribution is proposed to provide the definition of terms to be used in DualSteer aspect of FS_MASSS.

1. Introduction/Discussion

The DualSteer aspect of FS_MASSS is mainly about how to enable a DualSteer Device to perform DualSteer traffic steering and DualSteer traffic switching based on policies from HPLMN. In order to make the key issues clear in this study, we propose to define the terminology.

The definition of DualSteer Device is lifted from TS 22.261, while the definition of DualSteer traffic steering and DualSteer traffic switching is revised based on the discussion in SA2#160-AHE meeting. We proposed to keep the word “registered” in the definition of DualSteer traffic steering and DualSteer traffic switching for the time being, but this may need to be removed if the solutions allow for these operations to be performed without prior registration.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change * * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

DualSteer Device: A device supporting Dualsteer traffic steering and DualSteer traffic switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two 3GPP access networks, or it is two separate UEs in case of simultaneous data transmission over the two 3GPP access networks.

DualSteer traffic steering: the procedure whereby a DualSteer device selects a registered 3GPP access network for a new service and transfers all the traffic of that service over the selected 3GPP access network.

DualSteer traffic switching: the procedure whereby a DualSteer device moves all traffic of a service from one registered 3GPP access network to another registered 3GPP access network in a way that minimizes service interruption.

# * * * * End of changes * * * *