SA WG2 Meeting #161	S2-2402851

# Feb 26th – Mar 2nd, 2024; Athens

Source: 	Samsung

Title: 	KI #1, New Sol: Enhancements to PDU Session establishment for UPF selection for additional functionalities

Agenda Item: 	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract of the contribution: New solution for KI#1 describing overall impacts on PDU session establishment procedure for UPF selection based on desired functionalities.

# 1	Discussion

KI#1 description as specified in TS 23.700-63 is as follows:

Currently a UPF advertises the features it supports in the NF profile of UPF registered in NRF as specified in e.g. clause 6.1.6.2.13 of TS 29.510 [5] and/or over PFCP as defined in TS 29.244 [6]. An SMF leverages the exposed UPF information for proper UPF discovery and selection. A UPF implementation may support more optional functionalities like NAT, Packet Inspection, etc which are not fully specified by 3GPP. The following should be studied:

-	Whether there is a need and how to extend the existing UPF advertising capabilities.

-	Whether and how the input parameter(s) for UPF selection by SMF should be enhanced to support selection of UPF supporting specific user plane functionalities, e.g. NAT, Packet Inspection, etc.

# 2 	Proposal

It is proposed to adopt the following changes to TR 23.700-63.

*** Start of the change***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 |  |  |  |
| x | x |  |  |  |

*** Next change (all new text) ***

## 6.X	Solution #X: Enhancements to PDU Session establishment for selecting UPF supporting additional functionalities

### 6.X.1	Description

This solution describes the high level procedure for enhancement to UPF selection based on selected UPF functionalities during PDU Session establishment

Following can potentially impact the SMF decision on selecting a particular UPF for a PDU Session:

SM subscription data

- A particular DNN,S-NSSAI combination may be preferred to use particular UPF functionalities

SM-PCF policies

- A particular PDU session or the SDF flows through that PDU Session may require additional functionalities fro the UPF

List of UPFs received from the NRF

- NF profile stored in the UPF may store the additional functionalities supported by the UPF

- SMF may prefer those UPF which support the desired UPF functionalities (either received  from the PCF or configured in SMF)

### 6.X.2	Procedures

Figure 6.X.2-1 UPF registration for additional functionalities

- PDU Session establishment request UE to AMF as described in clause

- PDU Session establishment request AMF to SMF

- Subscription data retrieval from UDM

UDM provides the PDU Session subscription data to SMF, which may contain specific information regarding what UPF functionalities are required/UE is subscribe for the particular PDU Session (identified by S-NSSAI/DNN combination).

Note: 	What exact parameters, such as what UPF functionalities are used in the subscription are assumed to be covered by other solutions.

- SMF receives policies received from PCF

PCF  provides the PDU Session related policies and PCC rules to SMF, which may contain specific information regarding what UPF functionalities are required/ the particular PDU Session and or SD flows.

Note: 	What exact parameters, such as what UPF functionalities are used in the polices are assumed to be covered by other solutions.

- UPF discovery (optional, via NRF) and selection

Note: 	What exact parameters, such as what UPF functionalities are used to query NRF are assumed to be covered by other solutions.

- N4 session establishment

After SMF has performed SMF selection, it can request UPF to activate particular functionalities for the PDU Session and/or specific SDF.

Note: 	What exact parameters, such as what UPF functionalities are used in the N4 messages are assumed to be covered by other solutions.

- PDU Session Establishment accept to UE as described in TS 23.502 [2].

### 6.X.4	Impacts on services, entities and interfaces

PCF impacts

- PCF policies can be configured to prefer the use of specific UPF functionalities for a specific (S-NSSAI, DNN) or for a specific SD flow.

UPF impacts

- UPF can advertise its functionalities to NRF/ SMF and SMF may use N4 procedures to ask UPF to enable particular functionalities

SMF impacts

- SMF can receive polices related to specific UPF functionalities from PCF

- SMF to discover UPF supporting specific functionalities by providing NRF specific parameters during NRF discovery

UDM impacts

- UDM subscription data to be updated to prefer the use of specific UPF functionalities for a specific (S-NSSAI, DNN)  for a PDU Session

*** End of the changes ***