3GPP TSG-WG SA2 Meeting #161	S2-2402915

# February 26 – March 1, 2024, Athens, GR		(revision of S2-2401451)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5220 | rev | 4 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBSR mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson] Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 has specified the MBSR migration procedures in rel-18 TS 38.401 with additional information provided in S2-2400065/R3-238043. It needs to be further reflected in SA2 specs. Rev2: Resolve EN related to the MBSR HO procedure without PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of MBSR mobility with reference to TS 38.401. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing certain aspects of the MBSR mobility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.3.1, 5.35A.3.2, 5.35A.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.35A.3.1	UE mobility between a fixed cell and MBSR cell

For UEs in RRC_IDLE and RRC_INACTIVE state when a MBSR goes out-of-service, procedure for cell (re-)selection as specified in TS 38.304 [50] for RRC_IDLE and RRC_INACTIVE is used between MBSR cell and other cells.

If the MBSR goes out-of-service (due to e.g. MBSR moves to an area where the MBSR is not allowed to provide the relay service), the NG-RAN triggers the handover procedures to the neighbouring cells for UEs in RRC_CONNECTED state served by the MBSR. Any change of authorization state of the MBSR is also handled according to clause 5.35A.4.

The IAB-donor-CU triggers handover procedure when it is possible for the UEs accessing emergency service and being served by the MBSR, if MBSR is about to become unavailable to provide the services.

# * * * * Next change * * * *

#### 5.35A.3.2	UE mobility between MBSR cells

Similar to the behaviours described in clause 5.35A.3.1, UEs and NG-RAN use existing procedures as specified in clause 8.23 of TS 38.401 [42], TS 23.502 [3], or TS 38.304 [50] to handle the mobility between MBSR cells.

# * * * * Next change * * * *

#### 5.35A.3.x	MBSR mobility

The mobility of MBSR between different IAB-donor gNBs is described in clause 8.23 of TS 38.401 [42].

If a MBSR operates without PDU sessions, the NG-RAN triggers the handover procedure for a dummy PDU session and includes dummy PDU session information in the Handover Required message, as specified in TS 38.413 [34] towards the AMF of the MBSR (IAB-UE). The AMF shall ignore the PDU Session information it receives since the MBSR (IAB-UE) UE context has no PDU session information stored, The AMF shall then perform the handover procedures as specified in clause 4.9.1 of TS 23.502 [3] without involving any SMF and it includes in the Handover Request message towards the target NG-RAN an indication to cause the target NG-RAN to ignore the received PDU session information (No PDU Session Indication IE defined in TS 38.413 [34]) .  If the AMF is reallocated during the handover, the target AMF checks whether the UE context for the MBSR (IAB-UE)  includes any PDU sessions related information and if none is detected it continues the handover without involving any SMF and it includes in the Handover Request message towards the target NG-RAN an indication to cause the target NG-RAN to ignore the received PDU session information (No PDU Session Indication IE defined in TS TS 38.413 [34])

# * * * * End of changes * * * *