3GPP TSG-SA WG2 Meeting #S2-161	S2-2402934

Athens, Greece, Feburary 26  –  March 1, 2024

Source: 	Samsung

Title: 	new Solution for KI#2: provisioning of subscribers allowed to access CAG cell

Document for: 	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2 on how to provision subscribers allowed to access CAG cell.

- Introduction

For KI #2 we study the following aspects:

- How to enable the CAG owner or an authorized administrator to provision/update CAG information to the network for 5G Femto access control. The provisioning/updating of CAG info to the network that 5G Femto serves and the network that the UE has subscription will be considered.

In this paper, we focus on how to provision subscribers allowed to access CAG cell by the CAG owner or an authorized administrator.

2 Proposal

It is proposed to add the following contents to TR 23.700-45.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  | O |
| #2 |  |  |

# * * * * Second change (new text)* * * *

6.X	Solution #X: provisioning of subscribers allowed to access CAG cell

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

CAG owner or administrator may provision to 5GC subscribers allowed to access a CAG cell.

In principle provisioning CAG information to UDM is based on the clause 4.2.4.2 of TA 23.502 and provisioning the updated CAG information is based on the clause 4.15.6.2 of TA 23.502.;

- AF (CAG owner or administrator) provides UE ID (e.g. SUPI, MSISDN or GPSI),  and allowed Cell information (e.g. cell ID(s), FEMTO device ID) or CAG information, including Validity Time (for temporary users) per UE via Nnef_ParameterProvision_Create/Update/Delete procedures.

- NEF authorizes the request from AF, determines SUPI from GPSI/MSISDN if needed, may derive corresponding CAG information per UE and then provision into UDM.

- And UDM may provide CAG information to the corresponding UEs via UCU procedures via AMF.

- The result of UE provisioning can be reported to AF

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The Figure 6.x.2-1 shows the procedure for how the AF provision CAG subscriptions allowed to access CAG cell:

Figure 6.x.2-1: Procedures for AF to provision CAG subscriptions allowed to access CAG cell.

The steps of Figure 6.x.3-1 are described as follows:

1.	The AF provides UE ID (e.g. SUPI, MSISDN or GPSI), allowed Cell information (e.g. cell ID(s), FEMTO device ID) or CAG information including Validity Time (for temporary users) per UE via Nnef_ParameterProvision_Create/Update/Delete procedures.

2.	NEF authorizes the request from AF, determines SUPI from GPSI/MSISDN if needed and, may derive corresponding CAG information or allowed Cell information per UE and then provisioned into UDM.

3/5. UDM is provisioned CAG information and can be updated to UDR also.

6.	UDM sets the CAG information update indicator to confirm its update.

7-10. CAG information is provisioned.

11-12. CAC information update result is notified to the AF via NEF.

6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

UDM: to use CAG information update indicator in order to confirm UE provisioning.

AMF: updated information on CAG information are to be provided to gNB and UE.

NEF: to receive and authorize CAG owner or administrator's input (UE ID, allowed Cell information or CAG information) and, if required, change them into 5GS term(e.g. CAG information) and provide UDM.

AF: to provide UE ID and allowed Cell information or CAG information to 5GC.

# * * * * End of changes * * * *