SA WG2 Meeting #161	S2-2402976

February 26 – March 1, 2024, Athens, Greece

Source:	InterDigital Inc.

Title:	KI#2, New Solution, UAV flight route tracking and DAA using ground-based stations

Document for:	Approval

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution supporting ground based DAA using ground-based stations

# 1	Discussion

This contribution proposes a solution for ground-based DAA using ground-based stations addressing key issue #2.

# 2	Proposal

It is proposed to adopt the following new solution in TR 23.700-59.

* * * * Start of Changes (All Next Text) * * * *

## 6.X	Solution #X: UAV flight route tracking and DAA using ground-based stations

### 6.X.1	Key Issue mapping

This solution addresses KI#2, “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information”.

### 6.X.2	Description

In this solution, the UAV flight route tracking for DAA using ground-based stations (GS) is initiated from the USS/UTM and sent to the UAS-NF in 5GC. The GS have UE capabilities. The request includes UAV tracking related information such as planned route information and target UAV identifier. The UAS NF selects the ground-based stations along the route and send UAV monitoring request to the serving AMFs.

The serving AMF may perform the following actions:

- Sends an UAV Monitoring Request Message to a GS.

- Receive an UAV Monitoring Response Message from an GS and can forward it to the UAS NF.

- Receive an UAV Monitoring Report Message from a GS and can forward it to the UAS NF.

### 6.X.3	Procedures

Figure 6.X.3-1: UAV flight route tracking for DAA using ground-based stations

- The USS sends a UAV Tracking Request to the 5GS (e.g. UAS NF), using 5GS network. The request includes the tracking related information such as planned route information and target UAV identifier.

- The UAS NF choses ground-based stations.

- The UAS NF constructs a UAV Monitoring Request message and sends it to the serving AMFs.

- The AMF sends an DL NAS Transport message including UAV Monitoring Request Message to each GS.

- The AMF receives an UL NAS Transport message including UAV Monitoring Response Message from each GS and can forward it to the UAS NF.

- For tracking and monitoring of a Target UE, the GS can communicate with the Target UE, both;

  - 3GPP communication is supported; and

  - non-3GPP communication is supported.

- Receive an UL NAS Transport message including UAV Monitoring Report Message from an GS and can forward it to the UAS NF.

- UAS NF sends a tracking report to USS.

- USS may perform DAA deconflicting based on received tracking report via GS.

### 6.X.4	Impacts on services, entities and interfaces

UAS NF:

- Supports selection of grand-based stations

- Supports construction of UAV flight route monitoring request and forwarding the request to other NFs (e.g. AMF).

- Supports receiving a UAV flight route monitoring response and report.

- Support forwarding a UAV tracking report to USS.

AMF:

- Supports requesting UAV flight route monitoring to a ground-based station

- Supports receiving an UAV flight route monitoring response and report from a ground-based station.

- Support forwarding UAV flight route monitoring response and report to the UAS NF.

UE:

- Supports communication with the target UE and UAV flight route monitoring.

* * * * End of Changes * * * *