3GPP TSG-SA WG2 Meeting #161	S2-2403007

# Athens, Greece, 26 February – 01 March, 2024							   (revision of S2-2312176)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0189 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on EAS address and DNAI mapping information stored in UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current specification, DNAI information can be provided as part of EAS Deployment Information as given in Table 6.2.3.4-1 by AF or can be configured by OAM as part of Table 6.8.1-1. As the NEF is in charge of the management of EAS Deployment Information to be stored in UDR, NEF should enable coherency for the record on EAS address and DNAI Mapping information stored in UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 6.2.3.4.2 to clarify that NEF ensures collision is avoided for the record on EAS address and DNAI mapping information stored in UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned data in UDR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2 provides clarification on when NEF ensures collision is avoided. |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 6.2.3.4.2	EAS Deployment Information Provision from AF via NEF

The AF provides non-PDU Session specific EAS Deployment information to 5GC via the procedure defined in this clause.

Figure 6.2.3.4.2-1 EAS Deployment Information management in the AF procedure

1.	The AF invokes the Nnef_EASDeployment_Create/Update/Delete service operation.

2.	NEF checks whether the AF is authorized to perform the request, and authorised to provision the EAS Deployment Information based on the operator policies. The NEF derives DNN and S-NSSAI from the AF Service Identifier if not received explicitly and translates received External Application Identifier to Application Identifier known inside MNO domain. NEF also ensures that any EAS IP address range Information per DNAI in EAS Deployment Information does not contradict with the existing information configured via O&M.

3.	The NEF invokes the Nudr_DM_Create/Update/Delete to the UDR if it is authorized.

4.	The UDR stores/updates/removes the corresponding information (and responds a Nudr_DM_Create/Update/Delete Response to the NEF.

5.	The NEF sends Nnef_EASDeployment_Create/Update/Delete Response to the AF.

END OF CHANGES