3GPP TSG-WG SA2 Meeting #161	S2-2403020

# Athens, Greece, Feb 26 – Mar 1, 2024		(revision of S2-2401509)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4700 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the AMF service operation for 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon], Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.2.2.4.4 of TS 23.502, the Namf_MT_EnableGroupReachability is specified. However, there are some issues: - Some of the parameters are missing: it is highlighted in TS 23.247 clause 7.2.5.2 and TS 29.518 clause 6.3.6.2.7 that the most demanding ARP and 5QI should be included as the optional parameters. - The parameter “UE reachability Notification” is put in a wrong place. This parameter is an optional input and should be put into “Inputs, Optional” field rather than the “Input, Required” part. It is proposed to correct the above-mention parts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ARP and 5QI as the optional inputs to the service. Move UE reachability Notification from mandatory input to option input. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect service operation description in stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R02: At SA2#160AHE, S2-2401510 (CR 4700r01) with the same changes was agreed under the 5MBS_Ph2 WI, although the changes do not relate to any objective of the Rel-18 work but improve 5MBS text from Rel-17. Thus the WI is changed and changes are made applicable from Rel-17 onwards. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.2.2.4.4	Namf_MT_EnableGroupReachability service operation

Service operation name: Namf_MT_EnableGroupReachability.

Description: NF Service Consumer uses this service operation to request paging towards a group of UEs.

Inputs, Required: Multicast Session ID, UE list.

Inputs, Optional: MBS service area, ARP, 5QI, Associated PDU Session ID list, UE reachability Notification Address.

Outputs, Required: None.

Outputs, Optional: List of UEs in connected state (out of the input list).

This service operation is used in 5G MBS as specified in TS 23.247 [78].

# * * * * End of changes * * * *