3GPP SA WG2 Meeting #161	S2-2403153

Athens, Greece, February 26 – March 1, 2024

Source:	Ericsson

Title:	TR 23.700-63: Solution to KI#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS Ph2 / Rel-19

Abstract of the contribution: The contribution proposes a solution to KI#1 for enhancement of UPF selection.

# Introduction/Discussion

Solution proposal por KI#1

The following problems are identified:

•	Current information elements based on basic flags to expose the UPF capabilities are not flexible enough for complex deployments where several UPFs with different capabilities (potentially from different vendors or for special purporse e.g for the Edge) coexist with:

•	UPFs with different hardware configurations (e.g CPU or NIC (Network Interface Controller) that can bring better capacity or latency characteristics).

•	UPFs that do not support fully a standardized feature but partially.

•	There are non-standard features implemented in UPF, such as e.g NAT, firewalling or advanced non standardized reporting or forwarding features, which cannot be mapped to current information elements as capabilities.

# 2. Text Proposal

It is proposed to introduce the following changes vs. TR 23.700-63.

# * * * * First change (all new) * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 |  |
| Solution #X: Selection on UPF with extended user plane capabilities | X |  |  |  |

# * * * * Next change (all new) * * * *

## 6.X	Solution #X: Selection on UPF with extended user plane capabilities

### 6.X.1	Description

#### 6.X.1.1	Solution Description

This solution proposes to improve current UPF selection by extending the baseline UPF capabilities announced in N4 and UPF NF profile in NRF by defining an open and generic value (e.g. octet string) which can be used for non-standard or partially supported features:

-	The content of this new information element shall be configured by operator at UPF, and it represents the characteristics of this NF, which could be beyond current 3GPP standardized capabilities.

-	The content syntax could be just a  string or a sequence of bits values. This is for stage 3 to define what is the best syntax.

-	SMF can use this information to choose right UPF according to the capabilities of UPF and the needs of the PDU session.

-	This new information element is to be considered in the selection/re-selection of UPF (see clause 6.3.3.3 in TS 23.501 [1]).

The proposed solution can be used for addressing many scenarios. As an example, the new information element can be defined to correspond to:

-	Different hardware configurations (e.g CPU or NIC (Network Interface Controller) that brings better capacity or latency characteristics).

-	Partial support of a standardized feature.

-	Non-standard features implemented in UPF, such as e.g NAT, firewalling or advanced non standardized reporting or forwarding features that cannot be mapped to current information elements as capabilities.

And combinations of the above, simplifying management of complex deployments where several UPFs (potentially from different vendors and/or for special purpose e.g tuned for the Edge) coexist.

The new information element proposed is referred to as AdditionalSupportInfo in following clauses.

### 6.X.2	Procedures

The solution enhances the following procedures:

-    It extends the PFCP Association Setup (see clause 4.4.3.1 with “N4 Association Setup Procedure” and clause 4.4.3.2 “N4 Association Update Procedure” in TS 23.502 [3]) adding new capabilities for supported functionalities. This si for stage 3 to define, for example extending current IE Supported-Features IE in PFCP with one more bit and adding a new AdditionalSupportInfo IE with a new octet string that contains the information configured locally in UPF (as described in 6.X.1).

-    It extends UPF registration in NRF (clause 4.17.1	“NF service Registration” and clause 4.17.2	“NF service update” in TS 23.502 [3]) by adding in NF profile for UPF new capabilities. This is for stage 3 to define, for example extending SupportedPfcpFeatures in UpfInfo element in Nnrf_NFManagement NFRegister Request message with one more bit and adding a new AdditionalSupportInfo IE that contains the information configured locally in UPF (as escribed in 6.X.1).

-    It enhances UPF discovery  assisted by NRF (see discovery procedures, e.g. clause 4.17.4	 “NF/NF service discovery by NF service consumer in the same PLMN” in TS 23.502 [3]). The Consumer may include in Nnrf_NFDiscovery Request message the desired value of new  AdditionalSupportInfo parameter in the NFProfile.

-    It enhances UPF selection by the SMF. As part fo PDU Session Management, SMF must select the UPF for the PDU Session. As indicated in clause 6.3.3.3	“Selection of an UPF for a particular PDU Session” in TS 23.501 [1], the capability of the UPF including the new AdditionalSupportInfo IE and the functionality required for the particular PDU session shall be considered so an appropriate UPF can be selected by matching the functionality and features required.

See below in figure 6.X.2-1 how this solution enhances UPF Selection during PDU Sesison Establishment (based on clause 4.4.3 in TS 23.502 [3]).

When this procedure is triggered, UPF has already provided its capabilities to SMF over N4 or SMF gets them from NRF as described above. As shown in figure 6.X.2-1, it is in the step 8 of this procedure where SMF performs UPF selection. For such selection, the UPF new capabilities (including AdditionalSupportInfo IE) are considered by SMF in addition to the baseline ones so an appropriate UPF can be selected by matching the functionality and features required

Editor's Note: it is FFS if there is any existing IE in PFCP that can be used

How the SMF determines information about the user plane network topology, also including this new AdditionalSupportInfo, is based on operator configuration (as indicated in NOTE1 in 6.3.3.3	Selection of an UPF for a particular PDU Session in TS 23.501

AMF

P

C

F

UDM

(R)AN

UE

7

b.

SM

Policy

Association

Establishment

or

SMF initiated

SM

Policy

Association

Modification

10

a

.

N

4

Se

ssion

Establishment

/Modification

Request

1.

PDU Session

Establishment

Request

UPF

SMF

10

b

.

N

4

Session Establishment

/Modification

Response

9

. SMF initiated SM Policy Association Modification

1

1

.

Namf_Communication_N1N2MessageTransfer

1

3

.

AN

-

specific resource setup

(PDU Session Establishment Accept)

1

2

. N

2

PDU Session

Requ

est

(NAS msg)

1

4

. N

2

PDU Session

Re

sponse

1

5

. Nsmf

_

PDUSession_

UpdateSM

Context

Request

1

6

a

.

N

4

Session

Modification

Request

1

6

b

.

N

4

Session

Modification

Response

1

9

. IPv6 Address Configuration

First Uplink Data

First Downlink Data

8

. UPF selection

:

(new) SMF consider UPF AddditionSupport Info

2. SMF selection

1

7

. Ns

mf_

PDUSession_

UpdateSM

Context

Response

7

a. PCF selection

DN

6

.

PDU Session authentication/authorization

3

.

Ns

mf_

PDUSession_

CreateSMContext

Request

4. Subscription retrieval

/ Subscription

for updates

2

1

.

Unsubscription

5

. Ns

mf_PDUSession_CreateSMContext Response

1

8

.

Nsmf_

PDUSession

_

SMContext

Status

Notify

16c. Registration

20. SMF initiated SM Policy Association Modification

Figure 6.X.2-1 Example of UE-requested PDU Session Establishment for non-roaming and roaming with local breakout (TS 23.502 [3] clause 4.3.2.2)

### 6.X.3	Impacts on existing nodes and functionality

UPF, SMF, NRF are impacted.

Impacts results from adding the AdditionalSupport Info information in UPF configuration and in the procedures for:

-	PFCP Association Setup.

-	UPF registration in NRF.

-	UPF discovery  assisted by NRF.

-	UPF selection by the SMF.

As described in 6.X.1.

# * * * * End of changes * * * *