3GPP TSG-WG SA2 Meeting #161 	S2-2403295

Athens, Greece, Feb 26 – Mar 01, 2024

Source:	NTT DOCOMO

Title:	KI#3: New Solution for UE-satellite-UE communication in the context of LEO satellite constellations with ISL

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: A solution for UE-satellite-UE communication of IMS voice and video where satellites are part of LEO satellite constellation and communicate via ISL

# 1. Discussion

In TR 23.700-29 v0.3.0, it is proposed to study the support of UE-satellite-UE communication as below:

-	Architectural enhancement to support UE-satellite-UE communication, including minimum necessary set of 5GC network functions and IMS components (if any) onboard the satellite(s), including:

- 	How to ensure routing of user plane traffic (for MMTEL and MC applications) remains in the satellite.

-  Potential impacts on IMS procedures and functions to manage UE-satellite-UE communication

- 	Conditions (such as eligibility, restrictions, authorization …) for realizing UE-satellite-UE communication for UEs.

In this paper, we present a possible architecture that facilitates UE-satellite-UE communication via a group of LEO satellites. These LEO satellites form a satellite constellation with ISL. The architecture introduces dedicated equipment for satellite constellation communications. These include gNB, UPF and IMS AGW on board the satellites and their representative ground-based entities i.e., gNB (ground), UPF (ground), and IMS AGW (ground). Such an architecture ensures that the infrastructure provided by satellite communication operators can be used and shared by MNOs. It can also hide complexities related to satellite communication from MNOs. The ground-based entities which are representative of onboard entities are treated as such and shared by the AMF, SMF, and P-CSCF owned by MNOs.

NOTE:	In this paper, we present how the user plane path is established between UE and IMS AGW. When two UEs are served by the same satellite constellation, the user plane traffic is routed between IMS AGWs on board LEO satellite(s) of the same satellite constellation by using an existing mechanism.

Figure 1-1: Basic concept of this solution depicted based on Figure C-3 of TR 23.700-29

To make the solution work, the following aspects need to be addressed:

- How the nodes on the satellite and NFs on the ground discover and select each other.

- How 5GC discovers/recognizes the UEs when call is being setup and associates with corresponding entities. Interactions between the ground and satellite-based entities are required.

# 2. Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change* * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 Support of Regenerative-based satellite access | 2 Support of Store and Forward Satellite operation | 3 Support of UE-satellite-UE communication |  |
| X |  |  | X |  |

# * * * * Second change (All new text) * * * *

## 6.X	Solution X: UE-satellite-UE communication in the context of LEO satellite constellations with ISL

### 6.X.1	Description

This solution resolves Key Issue #3: Support of UE-satellite-UE communication. The focus is on architectural enhancements to support UE-satellite-UE communication particularly for IMS voice and video service via a satellite constellation of LEO satellites. Multiple LEO satellites form a satellite constellation and are connected via ISL. To achieve this, it is necessary from an architectural perspective to place gNB, UPF and IMS AGW onboard the satellites. Additionally, their representative entities (gNB (ground), UPF (ground), and IMS AGW (ground)) are ground-based. These entities refer to the satellite astronomical calendar based on satellite ephemeris to determine the opposing nodes.

This solution is based on the following assumptions:

-	The satellite onboard entities i.e., onboard gNB, UPF, and IMS AGW are owned and operated by a Satellite network operator and are used by multiple MNOs in shared manner.

-	The ground-based entities i.e., gNB (ground), UPF (ground), and IMS AGW (ground) are owned and operated by a Satellite network operator and are used by multiple MNOs in shared manner.

AMF and SMF treats the gNB (ground) and UPF (ground) respectively as a usual gNB and UPF. The P-CSCF treats IMS AGW (ground) as a usual IMS AGW. Satellite constellations are identified by Satellite constellation IDs.

Similarly, the gNB (ground) behaves as an AMF to onboard gNB, the UPF (ground) behaves as SMF to onboard UPF, and the IMS AGW (ground) behaves as a P-CSCF to onboard IMS AGW.

NOTE: 	These ground-based representative entities serve as control plane entities and hence the routing of the user plane traffic remains in the satellite.

Architectural assumptions explained above is depicted based on Figure C-3 as follows:

Figure 6.X.1-1: Basic concept of architectural assumptions of this solution

This solution is based on the Rel-18 approach agreed in TS 23.228, TS 24.229 and TS 23.502 related to PDU Session Establishment, IMS registration signaling, and the IEs included in the messages.

Editor's note:	The HO procedures when the satellite moves is FFS.

### 6.X.2	Procedures

#### 6.X.2.1	Association of ground entities representative of satellite constellations

The registration procedure for the gNB (ground), UPF (ground), and the IMS AGW (ground) with AMF, SMF, and P-CSCF respectively is as illustrated in Figure 6.X.2.1-1. The identifier for the constellations “satellite constellation ID” is used.

All the ground-based representative entities of satellite constellations establish a connection with each of their counterpart nodes (onboard) deployed on each of the LEO satellites that form a part of the satellite constellation and that can be connected from the ground at a specific time period derived from the satellite astronomical calendar based on satellite ephemeris.

Figure 6.X.2.1-1: Procedure for association of ground entities representative of satellite constellations

1. The ground-based gNB (ground) sends the NG setup request including the satellite constellation ID to the AMF.

2. This is followed by the response message.

3. The ground-based UPF (ground) sends the N4 Association setup request message including the satellite constellation ID to the SMF.

4. This is followed by the response message.

5. The ground-based IMS AGW (ground) sends the Registration request message including the satellite constellation ID to the P-CSCF.

6. This is followed by the response message.

#### 6.X.2.2	Registration Procedure

Figure 6.X.2.2-1: Registration for UE-satellite-UE communication.

- When a UE initiates a call, an RRC connection is set up between the UE and its serving gNB (onboard). The gNB then refers to the list of [start time, end time, to-be-connected gNB (ground)] created based on the satellite ephemeris and selects the appropriate gNB (ground).

Editor's note: How the list of [start time, end time, to-be-connected gNB (ground)] is created based on satellite ephemeris is FFS.

2. The gNB forwards the GUAMI in its Initial UE message. The inclusion of GUAMI in the Initial UE message is essential for the gNB (ground) to select the appropriate AMF.

3. gNB (ground) sends Initial UE message which includes the Registration request to the selected AMF.. Registration procedure continues according to clause 4.2.2.2.2 of TS 23.502.

4. The AMF sends the Registration accept response via Downlink NAS Transport message to the gNB (ground).

5. gNB (ground) sends the Registration accept response via Downlink NAS Transport message to the onboard gNB.

6. The onboard gNB sends the Registration accept message to the UE.

#### 6.X.2.3 	IMS PDU Session Establishment Procedure

Figure 6.X.2.3-1: IMS PDU session establishment procedure.

1-3. AMF receives PDU session establishment request for IMS DNN via the gNB (ground). Based on the locally stored information created by the procedure in Section 6.x.2.1, the AMF recognizes that the request comes from a UE served by a particular satellite constellation.

4-5. AMF sends the Nsmf_PDUSession_CreateSMContext request message including a satellite constellation ID to SMF. SMF selects a satellite UPF GW corresponding to the satellite constellation ID

6-8. SMF sends N4 session establishment request to UPF (ground). This includes the PDU session ID, which is used so that the context in UPF related to the PDU session ID can be easily found. The UPF (ground) refers to the list of [start time, end time, to-be-connected UPF] created based on the satellite ephemeris and selects the onboard UPF. N4 Session Establishment request/response is exchanged between UPF (ground) and onboard UPF followed by N4 Session Establishment response sent to SMF.

NOTE 1: 	PDU session ID to find context in UPF can be used in the procedure for possible HO procedures between LEO satellites.

Upon session establishment, the SMF sets IP address of P-CSCF tailored to treat satellite communication in Extended protocol configuration options (ePCO). The SMF may allocate an IP address to the UE from the IP address range of that particular satellite constellation ID.

NOTE 2: 	Since the SMF allocates IP address to UE from a pool of IP address range belonging to a satellite constellation ID, the IP address of the UE does not change during HO process when a satellite moves.

9. 	Namf_Communication_N1N2MessageTransfer including the P-CSCF address in ePCO is sent from SMF to AMF.

10-12. PDU Session Establishment accept is sent from AMF to onboard gNB via gNB (ground).

11. PDU Session Establishment accept is sent to UE.

#### 6.X.2.4	IMS registration and voice call setup procedure

Figure 6.X.2.4-1: IMS registration and voice call setup procedure.

1. The UE sends Register to the P-CSCF IP address available to it from the ePCO (see Section 6.X.2.3).

2. P-CSCF sends Register to S-CSCF.

3. Registration of S-CSCF in HSS.

4. Nhss_ImsSDM_Get request/response between S-CSCF and HSS.

5-6. 200 OK messages from S-CSCF to P-CSCF and from P-CSCF to UE.

7. A voice establishment request message (SIP INVITE) is received at the P-CSCF.

The P-CSCF selects the IMS AGW (ground) belonging to the satellite constellation ID based on UE’s IP address and the corresponding satellite constellation ID.

8-10. Message for controlling the access of the voice media is sent to the selected IMS AGW (ground). The IMS AGW (ground) refers to the list of [start time, end time, to-be-connected IMS AGW] created based on the satellite ephemeris and selects the onboard IMS AGW. Allocation request is sent to reserve a resource of a terminating point in its own equipment for data communication with the callee side, followed by response.

11. SIP INVITE is transferred to S-CSCF. Interaction with the terminating network continues.

12. SIP 183 Session Progress response is sent to S-CSCF.

13-15. Request/response to set a terminating point (Terminating UE IP address).

16-18. Allocation request to set a terminating point in the caller and to reserve a resource of a terminating point in its own equipment with regards to data communication with the caller.

19. SIP 183 Session Progress message is sent to UE.

20. Npcf_PolicyAuthorization_Create request message is sent from the P-CSCF to the PCF. The PCF which recognized the PDU session ID from PDU session establishment procedure sends the PDU session ID in response to the P-CSCF.

21-23. Context update request/response messages exchanged between P-CSCF and IMS AGW via the IMS AGW (ground). This includes the PDU session ID.

NOTE: 	This is a newly introduced message exchange. PDU session ID can be used in possibleprocedures for HO between LEO satellites.

### 6.X.3	Impacts on services, entities, and interfaces

gNB on board:

-	Capability to use satellite ephemeris and to discover an appropriate Satellite gNB GW.

gNB (ground):

-	A new network node that behaves as a gNB to AMF on the ground and as an AMF to gNB on board.

-	Capability to use satellite ephemeris and to discover an appropriate gNB on board.

UPF (ground):

-	A new network node that behaves as a UPF to SMF on the ground and as an SMF to UPF on board.

-	Capability to use satellite ephemeris and to discover an appropriate UPF on board.

IMS AGW (ground):

-	A new network node that behaves as an IMS AGW to P-CSCF on the ground and as a P-CSCF to IMS AGW on board.

-	Capability to use satellite ephemeris and to discover an appropriate IMS AGW on board.

AMF:

-	Capability to identify a request coming from gNB (ground) and to set satellite constellation ID to a message to SMF.

SMF:

-	Capability to select UPF (ground) based on satellite constellation ID.

-	Capability to assign IP address to UE from a pool of IP address range belonging to a satellite constellation ID.

P-CSCF:

-	Capability to identify the need to select IMS AGW (ground) on board based on UE IP address.

-	Capability to select IMS AGW (ground) based on satellite constellation ID.

-  Capability to retrieve PDU session ID from PCF and to send it to IMS AGW (ground).

# * * End of changes * * * *