3GPP TSG-SA WG2 Meeting #S2-162	                                   S2-2404623

Changsha, China, April 15  –  19, 2024

Source: 	Samsung

Title: 	Update for Solution #4

Document for: 	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes remove the EN’s .

- Introduction

CAG information(e.g. allowed CAG list) in the subscription data is defined per PLMN.

When a CAG owner or administrator in the vPLMN wants to let the authorized UE use the Femto service in the vPLMN, under assumption that the AF is authenticated&authorized with vPLMN (via v-NEF), the AF may update the CAG information for the vPLMN in the UDM subscription data.

2 Proposal

It is proposed to add the following contents to TR 23.700-45.

# * * * * First change * * * *

## 6.4	Solution #4: provisioning of subscribers allowed to access CAG cell using CAG subscription update from AF

### 6.4.1	Description

CAG owner or administrator may provision to 5GC subscribers allowed to access a CAG cell.

In principle provisioning CAG information to UDM is based on the clause 4.2.4.2 of TA 23.502 and provisioning the updated CAG information is based on the clause 4.15.6.2 of TA 23.502.;

- AF (CAG owner or administrator) provides UE ID (e.g. SUPI, MSISDN or GPSI), and allowed Cell information (e.g. cell ID(s), FEMTO device ID) or CAG information per PLMN ID, including Validity Time (for temporary users) per UE via Nnef_ParameterProvision_Create/Update/Delete procedures.

- NEF authorizes the request from AF, determines SUPI from GPSI/MSISDN if needed, may derive corresponding CAG information per UE and then provision into UDM.

- And UDM may provide CAG information to the corresponding UEs via UCU procedures via AMF.

- The result of UE provisioning can be reported to AF.

NOTE: when an AF in a visited PLMN wants to let the UE use the Femto service in the vPLMN, the AF should provision the CAG information to UDM as above.

### 6.4.2	Procedures

The Figure 6.4.2-1 shows the procedure for how the AF provision CAG subscriptions allowed to access CAG cell:

Figure 6.4.2-1: Procedures for AF to provision CAG subscriptions allowed to access CAG cell.

The steps of Figure 6.4.3-1 are described as follows:

1.	The AF provides UE ID (e.g. SUPI, MSISDN or GPSI), allowed Cell information (e.g. cell ID(s), FEMTO device ID) or CAG information including Validity Time (for temporary users) per UE via Nnef_ParameterProvision_Create/Update/Delete procedures.

2.	NEF authorizes the request from AF, determines SUPI from GPSI/MSISDN if needed and, may derive corresponding CAG information or allowed Cell information per UE and then provisioned into UDM.

3/5. UDM is provisioned CAG information and can be updated to UDR also.

6.	UDM sets the CAG information update indicator to confirm its update.

7-10. CAG information is provisioned.

11-12. CAG information update result is notified to the AF via NEF.

### 6.4.3	Impacts on services, entities and interfaces

UDM: CAG information for UE(s) are provisioned from AF via NEF and notify the result of provisioning to AF.

NEF: to receive and authorize CAG owner or administrator's input (UE ID, allowed Cell information or CAG information) and, if required, change them into 5GS term (e.g. CAG information) and provide UDM.

AF: to provide UE ID and allowed Cell information or CAG information to 5GC.

# * * * * End of changes * * * *