SA WG2 Meeting #162  	  S2-2403996

15 April-19 April 2024, Changsha,China

Source:	ZTE

Title:	TR 23.700-84: KI#1 – Solution #3 update

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: This contribution proposes label for LMF-side model training.

# 1. Discussion

As we agreed on the definition of ‘label’ in previous meeting, this paper does terminology alignment at first by changing ‘ground truth data’ to ‘label’. Besides, this paper proposes to use PRU or GNSS information as label for model training because both of them are precise UE locations. In the case that one of them is temporarily unavailable or biased, the other one can be used by NWDAF for LMF-side model training. Both of them are available in core so coordination with RAN on label is not needed.

# 2. Proposal

This paper proposes to update solution #3 for Key Issue #1 to TR23.700-84 as follows.

---------- Start of change ----------

6.3.1	Functional Description

A request of  LMF-side model that outputs location of a specific UE is provided by the service consumer. After training of the ML model, the NWDAF provides the trained ML model to the service consumer (e.g. LMF) and the service consumer can perform inference afterwards.

Although the service consumer can obtain UE location from AMF/LCS, but the accuracy may not be enough. Therefore, an enhancement of the existing NWDAF is needed, to train the ML model with measurement data and provide a more accurate location estimate of an UE.

6.3.1.1	Input of model training

| Information | Source | Description |
| --- | --- | --- |
| Measurements | TBD |  |
| >Measurement data |  | The measurement data collected by LMF. |
| >Positioning method |  | The positioning method corresponding to the measurement data. |
| >Time stamp |  | Time stamp of measurement data |
| Label |  | The label for the LMF-side model training |
| >PRU | LMF | The PRU information stored in LMF. |
| >GNSS location information | AF | The GPS information of UE. |

Table 6.3.1.1-1: Data Collected for LMF-side model training.

The information may collect by the NWDAF for LMF-side model training is defined in Table 6.x.1.1-1.

Editor's note: 	It is FFS to determine what and how data to be collected for LMF-side model training. Coordination with RAN WG is needed.

Editor's note: 	How to collect the UE location label is FFS.

The trained LMF-side model shall be able to calculate UE location.

---------- End of change ----------