3GPP TSG-SA2 Meeting #161	S2-2404093

Athens, GREECE, February 26 – March 01, 2024 	(revision of S2-2402895)

Source:	CATT

Title:	KI#3, New Sol: Support of No Transmit Zones

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3

Abstract of the contribution: This paper is to propose a new solution for key issue#3.

1. Introduction

It is proposed to agree the new solution for key issue#3 into TR 23.700-59.

This pCR updates the unhandled pCR (S2-2402895) in the previous SA2 meeting. The main changes include adding solution applies to EPS and adding solution to scenario B described in clause 2 below.

2. Discussion

Based on the deployment, to support NTZ, different scenarios shown in Figure 1 may happen.

Figure 1: Scenarios to support NTZ

The description of scenarios in Figure 1 is as follows:

-	Scenario A: in urban area, small cells are deployed. The geographical area of NTZ has overlap coverage with multiple cells. The time period of UAV UE to fly across the NTZ geographical area is similar to the time period of it to fly across the overlapping cells on the flight path. Based on the frequency band announced by the cell(s) overlapped with NTZ, this scenario can be further classified into two scenarios:

-	Scenario A.1: All the cells overlapped with NTZ only announce the Protected Frequency Band.

-	Scenario A.2: Part of the cells overlapped with NTZ announce the Non-Protected Frequency Band.

-	Scenario B: in rural area, large cells are deployed. The geographical area of NTZ is a small area inside a large cell. The time period of UAV UE to fly across the NTZ geographical area is very short compared with the time period of it to fly across the large cell. Based on the frequency announced by the cell overlapped with NTZ, this scenario can be further classified into two scenarios:

-	Scenario B.1: The cell overlapped with NTZ only announces the Protected Frequency Band.

-	Scenario B.2: The cell overlapped with NTZ announces the Non-Protected Frequency Band.

Because the UE behaviour in scenario B is related to the answer to the question sent to ECC in S2#160AHE (S2-2401813), the solution for scenario B is for further study. This pCR focuses on the solution to scenario A.

Here take Scenario A.2 as an example to describe the UE behaviour in this solution. The frequency announced by cells in Scenario A.2 is shown in Figure 2. Frequency f1 and f2 are Protected Frequency Band. Frequency f3 is Non-Protected Frequency Band. The UE receives the identifier of the cell(s) overlapped with NTZ and the Protected Frequency Band from RAN.

Figure 2: Example for frequencies announced in different cells in Scenario A.2

Based on the flight path of UAV UE, it will fly over cell1, cell2, cell3 and cell4. The UE behaviour in different cells are as follows:

-	When the UAV UE flies into cell1 or cell2, because the cell is the cell(s) overlapped with NTZ and only announce the Protected Frequency Band, the UAV UE stays in Any cell state.

-	When the UAV UE flies into cell3 which is the cell(s) overlapped with NTZ, because cell3 announces the f3 which is the Non-Protected Frequency Band, the UAV UE selects the f3.

-	When the UAV UE flies into cell4 which is not the cell(s) overlapped with NTZ, the UAV UE selects the frequency based on the existing mechanism.

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[3]	ECC Decision (22)07 (cept.org) https://docdb.cept.org/download/4240: "Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[x]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

* Second change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #3> | <Key Issue #y> |
| #x | X |  |
| #y |  |  |

* Third change *

6.X	Solution #X: Support of No Transmit Zones

6.X.1	Key Issue mapping

This solution address key issue #3: Support of No Transmit Zones.

6.X.2	Description

6.X.2.1	Scenarios

Based on the deployment, to support No-Transmit Zone (NTZ), different scenarios shown in Figure 6.x.2.1-1 may happen.

Figure 6.x.2.1-1: Scenarios to support NTZ

The description of scenarios in Figure 6.x.2.1-1 is as follows:

-	Scenario A: in urban area, small cells are deployed. The geographical area of NTZ has overlap coverage with multiple cells. The time period of UAV UE to fly across the NTZ geographical area is similar to the time period of it to fly across the overlapping cells on the flight path. Based on the frequency band announced by the cell(s) overlapped with NTZ, this scenario can be further classified into two scenarios:

-	Scenario A.1: All the cells overlapped with NTZ only announce the Protected Frequency Band.

-	Scenario A.2: Part of the cells overlapped with NTZ announce the Non-Protected Frequency Band.

-	Scenario B: in rural area, large cells are deployed. The geographical area of NTZ is a small area inside a large cell. The time period of UAV UE to fly across the NTZ geographical area is very short compared with the time period of it to fly across the large cell. Based on the frequency announced by the cell overlapped with NTZ, this scenario can be further classified into two scenarios:

-	Scenario B.1: The cell overlapped with NTZ only announces the Protected Frequency Band.

-	Scenario B.2: The cell overlapped with NTZ announces the Non-Protected Frequency Band.

6.X.2.2	Support of NTZ in 5GS

6.X.2.2.1	NTZ information distribution

OAM is configured with the NTZ information which includes the Protected Frequency Band and a geographical area. The OAM is responsible to determine the TA(s) overlapped with NTZ and the cell(s) overlapped with NTZ.

The OAM provides the following NTZ information to AMF/MME and RAN (i.e. gNB) separately:

-	NTZ information to AMF:

-	TA(s) overlapped with NTZ and the geographical area of the NTZ.

-	NTZ information to RAN:

-	Cell ID(s): the identity of cell(s) overlapped with NTZ.

-	Geographical area of the NTZ.

-	Protected Frequency Band: the frequency band is not allowed to be operated by UAV UEs in the NTZ, e.g. 703-733 MHz, 832-837 MHz, 2500-2570 MHz/2570-2620 MHz.

When AMF receives the NTZ information, the AMF provides the NTZ information to UAV UE as follows:

-	During Initial Registration procedure, if the UE is an UAV UE which supports NTZ capability and the registration area includes the TA(s) overlapped with NTZ received from OAM, the AMF provides the NTZ information including the TA(s) in the registration area overlapped with NTZ and the related geographical area to the UAV UE.

-	When the NTZ information is updated by the OAM, e.g. new NTZ is deployed or the geographical area of the existing NTZ is changed, if the NTZ information sent to UAV UE needs to be updated, the AMF update the NTZ information sent to UAV UE by sending UE Configuration Update Command.

6.X.2.2.2	NTZ support mechanism

When the UAV UE receives the NTZ information from AMF, if the UAV UE detects that it is approaching the geographical area of NTZ, it sends a request to RAN to notify that the UAV UE is approaching the geographical area of NTZ. Based on the NTZ geographical area and NTZ information received from OAM, RAN determines that the NTZ geographical area covers several cells or inside a cell:

-	If the NTZ geographical area covers several cells (i.e. scenario A), RAN provides the identity of cell(s) overlapped with NTZ and Protected Frequency Band to UE. When the information is received, the UE behaviour is as follows:

-	When the UAV UE (re)selects a cell, based on the information provided by RAN, if the suitable cell(s) announces the Non-Protected Frequency Band, the UE selects the Non-Protected Frequency Band. If the suitable cell(s) only announces the Protected Frequency Band, the UE stays in Any cell state.

-	For UE in CM-CONNECTED state, based on the measurement report, RAN determines that the target cell only announces the Protected Frequency Band, RAN releases RRC connection.

-	If the NTZ geographical area is inside of a large cell (i.e. scenario B), the RAN behaviour is as follows:

-	If the cell announces the Non-Protected Frequency Band and the UAV UE is not using the Non-Protected frequency band, the RAN configures the UAVs UE to use the non-protected frequency band.

-	Otherwise, the RAN determines a silence period based on the geographical area and flight path of UAV UE and sends the period to UAV UE. When UAV UE receives the period, the UE does not send uplink data and uplink signaling during the period.

6.X.2.3	Support of NTZ in EPS

6.X.2.3.1	NTZ information distribution

OAM is configured with the NTZ information which includes the Protected Frequency Band and a geographical area. The OAM is responsible to determine the TA(s) overlapped with NTZ and the cell(s) overlapped with NTZ.

The OAM provides the following NTZ information to MME and RAN (i.e. eNB) separately:

-	NTZ information to MME:

-	TA(s) overlapped with NTZ and the geographical area of the NTZ.

-	NTZ information to RAN:

-	Cell ID(s): the identity of cell(s) overlapped with NTZ.

-	Geographical area of the NTZ.

-	Protected Frequency Band: the frequency band is not allowed to be operated by UAV UEs in the NTZ, e.g. 703-733 MHz, 832-837 MHz, 2500-2570 MHz/2570-2620 MHz.

When MME receives the NTZ information, the MME provides the NTZ information to UAV UE as follows:

-	During Initial Attach procedure and Tracking Area Update procedures, if the UE is an UAV UE which supports NTZ capability and the Tracking Area List includes the TA(s) overlapped with NTZ received from OAM, the MME provides the NTZ information including the TA(s) overlapped with NTZ and the related geographical area to the UAV UE.

-	When the NTZ information is updated by the OAM, e.g. new NTZ is deployed or the geographical area of the existing NTZ is changed, if the NTZ information sent to UAV UE needs to be updated, the MME sends a Detach Request message with the Detach Type be set to re-attach. When the UAV UE re-attaches at the end of the detach process, the MME provides the updated NTZ information to the UAV UE.

6.X.2.3.2	NTZ support mechanism

When the UAV UE receives the NTZ information from MME, if the UAV UE detects that it is approaching the geographical area of NTZ, it sends a request to RAN to notify that the UAV UE is approaching the geographical area of NTZ. Based on the NTZ geographical area and NTZ information received from OAM, RAN determines that the NTZ geographical area covers several cells or inside a cell:

-	If the NTZ geographical area covers several cells (i.e. scenario A), RAN provides the identity of cell(s) overlapped with NTZ and Protected Frequency Band to UE. When the information is received, the UE behaviour is as follows:

-	When the UAV UE (re)selects a cell, based on the information provided by RAN, if the suitable cell(s) announces the Non-Protected Frequency Band, the UE selects the Non-Protected Frequency Band. If the suitable cell(s) only announces the Protected Frequency Band, the UE stays in Any cell state.

-	For UE in CM-CONNECTED state, based on the measurement report, RAN determines that the target cell only announces the Protected Frequency Band, RAN releases RRC connection.

-	If the NTZ geographical area is inside of a large cell (i.e. scenario B), the RAN behaviour is as follows:

-	If the cell announces the Non-Protected Frequency Band and the UAV UE is not using the Non-Protected frequency band, the RAN configures the UAV UE to use the non-protected frequency band.

-	Otherwise, the RAN determines a silence period based on the geographical area and flight path of UAV UE and sends the period to UAV UE. When UAV UE receives the period, the UE does not send uplink data and uplink signaling during the period.

6.X.3	Procedures

6.X.3.1	Support of NTZ in 5GS

6.X.3.1.1	NTZ information provisioning via NAS procedure

During registration procedure, if the UAV UE supports NTZ capability and the registration area includes the NTZ(s), AMF is responsible to provide the NTZ information to UAV UE. The NTZ information includes TA(s) in the registration area overlapped with NTZ and the related geographical area. The registration procedure in clause 4.2.2.2.2 of TS 23.502 [4] applies with the following additions:

-	In step 1, the UAV UE provides NTZ capability in UE MM Core Network Capability in Registration Request message.

-	In step 21, the AMF determines that the registration area of UE includes the TA(s) overlapped with NTZ in the NTZ information provided by OAM and the UAV UE supports NTZ capability. The AMF provides the NTZ information to UE. The NTZ information includes TA(s) in the registration area overlapped with NTZ and the related geographical area.

When the NTZ exists in the registration area of UAV UE changes, the AMF is responsible to provide the updated NTZ information to UAV UE. The UE Configuration Update procedure in clause 4.2.4.2 of TS 23.502 [4] applies with the following additions:

-	In step 0, the AMF determines to update the NTZ information sent to UE.

-	In step 1, the AMF provides the NTZ information to UE.

6.X.3.1.2	NTZ information provisioning via RRC procedure and NTZ support mechanism

Based on the NTZ information (i.e. TA(s) in the registration area overlapped with NTZ and the related geographical area) received via NAS procedure, when the UAV UE detects that it is approaching the geographical area of NTZ, it sends a request to RAN to notify that the UAV UE is approaching the geographical area of NTZ. RAN provides the UAV UE with the information to support NTZ. The procedure is shown in Figure 6.x.3.1.-1

Figure 6.x.3.1-1: NTZ information provisioning via RRC procedure

1.	UE sends a RRC request to RAN to notify that it is approaching NTZ.

2.	RAN sends RRC response to UE. Based on the serving cell of UAV UE and the OAM configured NTZ information, the information provided by RAN to UE is different.

If the NTZ geographical area covers several cells (i.e. scenario A), the RRC Response includes the identity of cell(s) overlapped with NTZ and Protected Frequency Band. In this case, to support NTZ, the UAV UE behaviour is as follows:

-	When the UAV UE (re)selects a cell, based on the information provided by RAN, if the suitable cell(s) announces the Non-Protected Frequency Band, the UE selects the Non-Protected Frequency Band. If the suitable cell(s) only announces the Protected Frequency Band, the UE stays in Any cell state.

-	For UE in CM-CONNECTED state, based on the measurement report, RAN determines that the target cell only announces the Protected Frequency Band, RAN releases RRC connection.

If the NTZ geographical area is inside of a large cell (i.e. scenario B), RAN configures the UAV UE to use the non-protected frequency band if the cell announces the non-protected frequency band. Otherwise, RAN determines a silence period based on the geographical area and flight path of UAV UE and includes the silence period in the RRC response. In this case, to support NTZ, when UAV UE receives the period, the UE does not send uplink data and uplink signalling during the period.

6.X.3.2	Support of NTZ in EPS

6.X.3.2.1	NTZ information provisioning via NAS procedure

During attach procedure, if the UAV UE supports NTZ capability and the TAI List includes the NTZ(s), MME is responsible to provide the NTZ information to UAV UE. The NTZ information includes TA(s) in the TAI List overlapped with NTZ and the related geographical area. The attach procedure in clause 5.3.2.1 of TS 23.401 [x] applies with the following additions:

-	In step 1, the UAV UE provides NTZ capability in UE Core Network Capability in Attach Request message.

-	In step 17, the MME determines that the TAI List of UE includes the TA(s) overlapped with NTZ in the NTZ information provided by OAM and the UAV UE supports NTZ capability. The MME provides the NTZ information to UE. The NTZ information includes TA(s) in the registration area overlapped with NTZ and the related geographical area.

During Tracking Area Update procedure, if the UAV UE supports NTZ capability and the TAI List includes the NTZ(s), MME is responsible to provide the NTZ information to UAV UE. The NTZ information includes TA(s) in the TAI List overlapped with NTZ and the related geographical area. The Tracking Area Update procedure in clause 5.3.3.1 of TS 23.401 [x] applies with the following additions:

-	In step 1, the UAV UE provides NTZ capability in UE Core Network Capability in Attach Request message.

-	In step 20, the MME determines that the TAI List of UE includes the TA(s) overlapped with NTZ in the NTZ information provided by OAM and the UAV UE supports NTZ capability. The MME provides the NTZ information to UE. The NTZ information includes TA(s) in the registration area overlapped with NTZ and the related geographical area.

6.X.3.2.2	NTZ information provisioning via RRC procedure and NTZ support mechanism

Same mechanism as 5GS, i.e. the procedure in clause 6.X.3.1.2, applies.

6.X.4	Impacts on services, entities and interfaces

OAM:

-	Determine the TA(s) overlapped with NTZ and cell(s) overlapped with NTZ.

-	Provide to AMF/MME the NTZ information including TA(s) overlapped with NTZ and the geographical area of NTZ.

-	Provide to RAN (gNB in 5GS and eNB in EPS) the NTZ information including identifier of the cell(s) overlapped with NTZ and the Protected Frequency Band.

AMF:

-	Receive the NTZ information including TA(s) overlapped with NTZ from OAM.

-	Send the TA(s) in the registration area overlapped with NTZ and the related geographical area to the UAV UE via Registration Accept or UE Configuration Update Command.

MME:

-	Receive the NTZ information including TA(s) overlapped with NTZ from OAM.

-	Send the TA(s) in the TAI List overlapped with NTZ and the related geographical area to the UAV UE via Attach Accept or TAU Accept.

-	If the NTZ information sent to UAV UE needs to be updated, the MME sends a Detach Request message with the Detach Type be set to re-attach.

UE:

-	Receive the TA(s) overlapped with NTZ from AMF/MME.

-	Send the notification to RAN when approaching the geographical area.

-	Receive the identifier of the cell(s) overlapped with NTZ and the Protected Frequency Band or a silence period from RAN:

-	The information received from RAN includes identifier of the cell(s) overlapped with NTZ and the Protected Frequency Band: Based on the information received from RAN, if the suitable cell(s) for (re)selection announces the Non-Protected Frequency Band, the UE selects the Non-Protected Frequency Band. If the suitable cell(s) only announces the Protected Frequency Band, the UE stays in Any cell state.

-	The information received from RAN includes a silence period: When UAV UE receives the period, the UE does not send uplink data and uplink signaling during the period.

RAN (gNB in 5GS and eNB in EPS):

-	Receive the NTZ information including the identifier of cell(s) overlapped with NTZ, geographical area of the NTZ and the Protected Frequency Band from OAM

-	Provide the identifier of cell(s) overlapped with NTZ and the Protected Frequency Band or a silence period to UE based on the UE notification.

-	For UE in CM-CONNECTED state, based on the measurement report, if the target cell only announces the Protected Frequency Band, RAN releases RRC connection.

* End of changes *