3GPP TSG-WG SA2 Meeting #162 	S2-2404109

Changsha, China, April 15 – April 19, 2024	(revision of S2-240xxxx)

Source:	China Mobile

Title:	New Solution on MTLF-based model performance monitoring for AI/ML positioning

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1.

1. Discussion

As indicated in Key Issue #1: Enhancements to LCS to support Direct AI/ML based Positioning, the following contents will be studied to support AI/ML based Positioning for Cases 2b, 3b:

-	Study whether and how an AI/ML model for direct AI/ML positioning (i.e. case 2b/3b) is handled:

-	Which entity trains the model for direct AI/ML positioning and if the entity that train the model and the consumer are different, how the model consumer get the trained AI/ML model;

-	Which entity acts as the model consumer that will use the trained model to perform inference and/or derive UE location;

-	Define procedures for data collection with objective to train AI/ML models for direct AI/ML positioning.

-	Whether and how to support direct AI/ML positioning at LMF with additional 5GC enhancements.

-	How to monitor model performance for ML models used for direct AI/ML based positioning.

According to the definition of TS23.288[5], an NWDAF may have the accuracy checking capability for Analytics and/or ML Models. The NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes. The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate Analytics Accuracy Monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The NWDAF containing MTLF may collect location information for a target UE or a group of target UEs from the LCS system. MTLF determines ML Model degradation based on newly collected test data and retrains or reprovisions the existing AI/ML positioning Model. LMF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or re-provisioning of the AI/ML positioning model.

Thus, this contribution proposes to define the procedures of MTLF-based model performance monitoring for AI/ML positioning.

2. Text Proposal

It is proposed to agree the following solution into TR 23.700-84.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |  |  | Use cases (optional) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 |
| #1 | X |  |  |  |  |  |  |  |  |  |
| #2 | X |  |  |  |  |  |  |  |  |  |
| #3 | X |  |  |  |  |  |  |  |  |  |
| #4 | X |  |  |  |  |  |  |  |  |  |
| #5 | X |  |  |  |  |  |  |  |  |  |
| #6 | X |  |  |  |  |  |  |  |  |  |
| X | X |  |  |  |  |  |  |  |  |  |

# * * * * Second change * * * *

6.X	Solution #X: MTLF-based model performance monitoring for AI/ML positioning

6.X.1	Description

This solution addresses Key Issue #1 "Enhancements to LCS to support Direct AI/ML based Positioning" regarding how to monitor model performance for ML models used for direct AI/ML based positioning. And assumes LMF with an AI/ML model for inference capability.

According to the definition of TS23.288[5], an NWDAF containing MTLF has the accuracy checking capability for Analytics and/or ML Models.

The LMF may provide the AI/ML positioning inference to consumers when requested by consumer. The LMF may provide inference data to MTLF for model monitoring. The MTLF performs AI/ML positioning model monitoring based on requests from LMF.

The MTLF may collect UE location information from t from UE, RAN, AMF, GMLC and AF as described in TS 23.273 [7]. MTLF monitors AI/ML positioning Model performance based on newly collected UE data and re-trains or re-provisions the existing AI/ML positioning Model.

6.X.2	Procedures

Figure 6.X.2-1: Procedure of MTLF-based model performance monitoring for AI/ML positioning

1.	The AF/Consumer NF subscribes to the LMF to request AI/ML Positioning.

2.	The LMF performs UE location data collection from UE, RAN, AMF, GMLC and AF as described in TS 23.273 [7].

3.	The LMF uses the trained ML Model for direct AI/ML based positioning.

4.	The LMF notifies AI/ML Positioning inference to the AF/Consumer NF, and may send the inference to MTLF for monitoring model performance.

5.	The MTLF performs UE location data collection for monitoring model performance of AI/ML positioning inference.

6.	The MTLF obtains model performance based on the collected UE location data and may decide to re-train/re-provision the ML Model for LMF.

7.	The MTLF provides a model performance report to LMF. When the newly generated or re-trained ML Model is ready, the MTLF sends a new or re-trained ML Model to the LMF.

6.X.3	Impacts on services, entities and interfaces

MTLF:

-	Supports to monitor AI/ML positioning model performance.

-	Supports to generate new or re-trained AI/ML positioning models.

LMF:

-	Supports to provide inference of AI/ML positioning to MTLF.

# * * * * End of changes * * * *