SA WG2 Meeting #162	S2-2404465

Apr.15th – Apr 19st, 2024; Changsha, China

Source:	China Mobile

Title:	Solution KI #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

Document for:	Approval

Agenda Item: 	X.X

Work Item / Release:	FS_UIA_ARC /Rel-19

# 1. Introduction

This solution addresses the key issue on identifying non-3GPP Devices Connecting behind a UE or 5G-RG

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-32.

# * * * * First change (all new) * * * *

## 6.X	Solution #X: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

### 6.x.1	Key Issue Mapping

Table 6.x.1-1: Mapping of Solutions to Key Issues

|  |  | Key Issues |  |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #X |  |  |  | √ |

### 6.X.2	Description

This solution addresses key issue #4 “Identifying non-3GPP Devices Connecting behind a UE or 5G-RG”, including what information is stored as part of the non-3gpp Devices’ User Identity profile, and how non-3gpp Devices’ User Identity Profiles are configured, stored, and updated.

Introduce a new type of parameter identifying the non-3gpp device’s User ID. When a UE/5G-RG requests a non-3gpp device’s User ID-related service, the UE/5G-RG shall provide both UE/5G’s SUPI and non-3gpp device’s User ID to the network.

### 6.X.3	Procedures

The figure 6.X.3-1 below shows a high-level flow of the actions needed for the Configuration and Usage of non-3gpp devices’ User Identifiers.

Non-3GPP Devices

UE/5G-RG

NG-RAN

5GC

DN

AAA

1A: Pre-configuration

1B: Pre-configuration

2. UIA info update during the registration procedure or UE/5G-RG parameter update procedure

3. Uplink Data

5． Secondary Authentication (Optional)

6． UE/5G-RG SUPI+Non-3gpp device UID status activation and Concurrent active Non-3gpp device’s number check

8． Data transfer with subscribed QoS

9. Downlink Data

10． PDU release procedure

11. UE/5G-RG SUPI+Non-3gpp device UID status deactivation

4. PDU session related request, including UE/5G-RG SUPI and non-3gpp device’s User ID

7. PDU session-related response, including non-3gpp device’s User ID subscribed QoS

Figure 6.X.3-1: high-level flow of Configuration and Usage of non-3gpp devices’ User Identifiers

The procedure includes the following steps:

1A: The UE/5G-RG is pre-configured with non-3GPP device access info.

1B: The AAA server is pre-configured with non-3gpp device access information.

2: When UIA info is updated, the network can update the UIA info during the registration procedure or UE/5G-RG parameters Update procedure. The application layer approach, which may be an alternative, is out of the 3GPP scope.

3. Non-3gpp Device sends uplink data to UE/5G-RG.

4: When a non-3GPP device triggers the data service, the associated UE/5G-RG may send the PDU session-related request message to the network, including the UE/5G-RG SUPI and Non-3gpp device’s User ID.

5: During the PDU session procedure, the SMF may trigger the Secondary authentication using credentials.

6: When the non-3gpp device’s User ID has completed the authentication, the network enables the connectivity between UE/5G-RG and DN. Meanwhile, the network (e.g. UDM) shall mark the UE/5G-RG SUPI/Non-3GPP device’s User ID status as “active”. At the same time, the network will check the concurrent non-3GPP device number with the UE/5G-RG SUPI. If the concurrent non-3GPP device number is larger than the UE/5G-RG subscribed concurrent number, the network will reject the PDP session request with the appropriate cause code.

7: The network enables the connectivity between UE/5G-RG and DN with subscribed dedicated QoS.

8: The UE/5G-RG transmits the data with connectivity between UE/5G-RG and DN. UE/5G-RG must support multiple PDU sessions launched by different non-3GPP devices simultaneously.

9: the UE/5G-RG transmits the data to non-3GPP devices.

10: the non-3GPP device de-activates the connection, which will trigger the relevant PDU session between UE/5G-RG and 5GC to be released.

11：The network shall mark the UE/5G-RG SUPI/Non-3gpp device’s user ID status as “inactive”, which means for the UE/5G-RG (SUPI), this Non-3gpp device’s User ID is not being active.  Furthermore, abnormal non-3GPP user ID deactivation should be considered too. For example, the network may set an active duration timer or inactivity timer for each non-3gpp device’s user ID, When the timer expires, the network triggers to release the connectivity to allow other access.

### 6.X.4	Impacts on services, entities and interfaces

UE/5G-RG impacts:

-	Correct Non-3GPP User ID selection for PDU session based on non-3GPP device access info

-  Manage multiple Non-3GPP device’s User IDs

AMF impacts:

-	impact to the PDU session establishment/modification procedure based on the SUPI+ additional Non-3GPP device’s User ID.

SMF impacts:

-	impact to the PDU session establishment/modification procedure based on the SUPI+ additional Non-3GPP device’s User ID.

Charging impacts:

-	impact to the CDR procedure based on the SUPI+ additional Non-3GPP device’s User ID.

UDM/UDR impacts:

-	The UDM supports multiple non-3GPP device’s user profiles per UE/5G-RG SUPI -- the Non-3GPP device’s user ID(s) list associated with the one UE/5G-RG SUPI, including subscribed dedicated QoS info, SUPI/User identify status, and so on.

-	Capability to reject or accept the non-3gpp device’s user ID provisioning request based on the existing concurrent active number and subscribed concurrent active number

# * * * * End of changes * * * *