3GPP TSG SA WG2 Meeting #162	S2-2404474

Changsha, China, April 15 – 19, 2024	(was S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1084 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to procedures of e2e data volume transfer time analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.502 clause 4.15.4.5.1 and clause 5.2.26.2 defines that a consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. However the procedure for AnalyticsID on E2E data volume transfer time analytics, that collects the QoS flow Packet delay is not aligned with the UPF event exposure described in 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the procedure for AnalyticsID on E2E data volume transfer time analytics to show that to collect QoS Monitoring event, the SMF subscribes to UPF via N4 i/f and the notification is sent by UPF to NWDAF directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect analytics procedure for the procedure for AnalyticsID on E2E data volume transfer time analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 6.18.4	Procedures

The NWDAF may provide E2E data volume transfer time analytics to a 5GC NF (e.g. AF, or NEF).

Figure 6.18.4-1: Procedure for E2E data volume transfer time analytics

1.	The Consumer NF, e.g. AF or NEF, requests or subscribes to analytics for E2E data volume transfer time analytics from NWDAF (possibly via NEF in case the consumer NF is an untrusted AF) and provides the input information as specified in clause 6.18.1 to 5GC.

2a-b.	The NWDAF may subscribe to the service data from AMF as defined in Table 6.18.2-2 using Namf_EventExposure_Subscribe service operation for collecting UE location(s) for a UE or a group of UEs using an Internal-Group-Id.

NOTE:	If NWDAF requires UE location information with finer granularity than TA/cell level, then NWDAF collects the location data from GMLC instead of AMF.

2c.	The NWDAF may subscribe to service data from SMF as defined in Table 6.18.2-2 by invoking Nsmf_EventExposure_Subscribe service operation (Event ID, SUPI(s) or Application ID).

2d	In order to provide QoS flow Packet delay data, the SMF subscribe to QoS Monitoring information from UPF as defined in Table 6.18.2-2 using N4 Session Reporting Rule.

2e.	The subscribed event is triggered in the UPF.

2f.	The UPF notifies the subscribed event report directly to the  NWDAF.

2g.	The NWDAF may subscribe to the input data from the OAM as defined in the Table 6.18.2-1 according to the data collection principles described in clause 6.2.3.

2h.	The NWDAF may subscribe to the service data from as defined AF in the Table 6.18-3 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe (Event ID = E2E data volume transfer time information, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) service as defined in TS 23.502 [3].

3.	The NWDAF derives the requested analytics, in the form of E2E data volume transfer time statistics and/or predictions.

4.	The NWDAF provides the requested E2E data volume transfer time analytics to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the consumer NF subscribed to E2E data volume transfer time analytics in step 1, once the NWDAF generates new analytics for E2E data volume transfer time, it provides a notification using Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

*** End of Changes ***