SA WG2 Meeting #162	S2-2404554

15 - 19 April, 2024, Changsha, China

Source:	vivo, China Telecom

Title:	New Sol: Association of registrations

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Solution for subscription and registration.

1	Discussion

This paper proposes a new solution for KI#1.1 subscription and KI#1.2 registration of the FS_MASSS SID (SP-240467).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | Key Issue #1.1 | Key Issue #1.2 | Key Issue #1.3 | Key Issue #1.4 |
| #1.A | X | X |  |  |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |
| --- | --- | --- |
| Solution# | Key Issue #2.1 | Key Issue #2.2 |
| #2.1 | X |  |
| #2.2 |  | X |
| #2.3 | X |  |
| #2.4 | X |  |
| #2.5 | X |  |
| #2.6 |  | X |
| #2.7 |  | X |
| #2.8 |  | X |

* * * * Second Change (all new text) * * * *

### 6.1.A	Solution #A: Association of registrations

6.1.A.1	Description

This solution addresses the Key Issue#1.1 "Subscription aspects to support DualSteer" and Key Issue#1.2 "Registration and mobility management for DualSteer".

6.1.A.2	Procedures

6.1.A.2.1	Information flow

Figure 6.1.A.2.1-1: Association of registrations

1-2.	The DualSteer device to perform 5GS registration procedure with SUPI#1 towards serving network#1, it initiates Registration Request (SUCI#1/5G-GUTI#1) towards AMF#1. During the registration procedure, the AMF#1 registers with UDM or UDM+HSS for SUPI#1.

3-4.	The DualSteer device decides to perform dual registration with SUPI#2, it initiates 5GS initial registration procedure towards AMF#2 or EPS Attach procedure towards MME in serving network#2. During the registration procedure, the AMF#2 registers with UDM or MME registers with UDM+HSS for SUPI#2.

5.	The UDM associates the two registration procedures as dual-registration from a DualSteer device.

6.1.A.2.2	Subscription data

In order to support dual registration of a DualSteer device, new subscription data types, i.e., DualSteer Pair Data, is used to accommodate the management information for a DualSteer device.

To associate two subscriptions/SUPIs for DualSteer, the DualSteer Pair Data includes two SUPIs that share the same subscription profile.

Following tables 6.1.A.2.2-1 and 6.1.A.2.2-2 show the subscription enhancement for DualSteer.

Table 6.1.A.2.2-1: DualSteer Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Pair Data | DualSteer Pair Identifier | Identifiers of the DualSteer Pair or the DualSteer device that the DualSteer Pair Data belongs to. |
|  | SUPI list | Corresponding SUPI list, i.e., 2 SUPIs |
|  | DualSteer data | Subscription data for supporting DualSteer, which includes SUPIs. (NOTE 1) |
| NOTE 1: The content of Dualsteer data is subject to the solutions to other KIs on DualSteer, considering the subscription information related to access and mobility, session management, and policy aspects, for example the DNN an S-NSSAI specific parameters to support DualSteer. |  |  |

Table 6.1.A.2.2-2: DualSteer Subscription data types keys

| Subscription Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| DualSteer Pair Data | DualSteer Pair Identifier | - |

The benefit of introducing the DualSteer Pair Data is to facilitate the management of the two SUPIs when they act as one DualSteer device and the subscription information related to the DualSteer device. They could be managed using the share data feature, for example, the network could assign the DNN an S-NSSAI specific parameters to support DualSteer, or the PCF could update policies that targeting the pair.

Meanwhile, if the UE itself is not capable of DualSteer feature, e.g., only perform single registration, it can still use the common subscription data of one SUPI, i.e., the network and UE do not need to consider any DualSteer related information.

6.1.A.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UDM/UDR:

-	Subscription data is enhanced to support DualSteer.

* * * * End of Changes * * * *