SA WG2 Meeting #162	S2-2404556

15 - 19 April, 2024, Changsha, China

Source:	vivo

Title:	New Sol: URSP provisioning for DualSteer traffic steering

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new solution for URSP provisioning for DualSteer traffic steering.

1	Discussion

This paper proposes a new solution for KI#1.1 subscription, Key Issue#1.2, and KI#1.4 policy of the FS_MASSS SID (SP-240467).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | Key Issue #1.1 | Key Issue #1.2 | Key Issue #1.3 | Key Issue #1.4 |
| #1.C | X | X |  | X |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |
| --- | --- | --- |
| Solution# | Key Issue #2.1 | Key Issue #2.2 |
| #2.1 | X |  |
| #2.2 |  | X |
| #2.3 | X |  |
| #2.4 | X |  |
| #2.5 | X |  |
| #2.6 |  | X |
| #2.7 |  | X |
| #2.8 |  | X |

* * * * Second Change (all new text) * * * *

### 6.1.C	Solution #C: URSP provisioning for DualSteer traffic steering

6.1.C.1	Description

This solution addresses the Key Issue#1.1 "Subscription aspects to support DualSteer", Key Issue#1.2 "Registration and mobility management for DualSteer", and Key Issue#1.4 "Policy enhancements for DualSteer".

In this solution, a (DNN, S-NSSAI) combination is configured subject to only one SUPI for supporting DualSteer traffic steering,

6.1.C.2	Procedures

6.1.C.2.1	Information flow

Figure 6.1.C.2.1-1: URSP provisioning for DualSteer traffic steering

1.	The DualSteer device performs registration procedure with SUPI#1, it initiates Registration Request message towards the AMF#1 with UE Policy Container, which contains DualSteer capable indication.

2.	The AMF#1 invokes Npcf_UEPolicyControl_Create Request (UE Policy Container) towards the PCF#1.

3.	If the DualSteer capable indication is received, the PCF#1 invokes Nudr_DM_Subscribe (SUPI#1, Policy Data, UE context Policy Control data) towards UDR to subscribe data modification notification of UE context Policy Control data of SUPI#1.

Before step 3, the PCF#1 may invoke Nudr_DM_Create/Update service operation towards UDR as described in clause 4.16.11 of TS 23.502 [4].

4.	The registration procedure is continued.

5.	The DualSteer device decides to perform dual registration with SUPI#2, steps 1-4 are repeated for SUPI#2.

6.	The UDM updates the UE context policy control data of SUPI#1 and SUPI#2 to include DualSteer Pair ID in the DualSteer Ready IE.

7.	The UDR notifies the updated UE context policy control data to PCF#1 and PCF#2, which includes the DualSteer Pair ID.

8.	The PCF#1 and PCF#2 invoke Nudm_DM_Query (DualSteer Pair ID, Subscription Data, DualSteer Pair Data) towards the UDR to obtain the DualSteer Pair Data as described in table 6.1.C.2.2-2.

9.	The PCF#1 and PCF#2 determine to update the URSP based on the received DualSteer subscription data. The PCF#1 and PCF#2 deliver the updated URSP to the DualSteer device as described in clause 4.2.4.3 of TS 23.502 [4].

6.1.C.2.2	Subscription data

The subscription data for this solution is based on the subscription data proposed in solution #1.A with the following additions.

Table 6.1.C.2.2-1: UE context Policy Control Subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| UE context policy control subscription data | … | … |
|  | DualSteer Ready | Indicates whether the SUPI is ready to be involved for DualSteer. If ready, it indicates the DualSteer Pair Identifier. |
|  | … | … |

Table 6.1.C.2.2-2: DualSteer Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Pair Data | DualSteer Pair Identifier | Identifiers of the DualSteer Pair or the DualSteer device that the DualSteer Pair Data belongs to. |
|  | SUPI list | Corresponding SUPI list, i.e., 2 SUPIs |
|  | Policy related information in DualSteer data: |  |
|  | Zero or more entries: |  |
|  | Steering Descriptor | Indicates the RAT type, PLMN ID, and/or SUPI. |
|  | For each Steering Descriptor, one or more entries: |  |
|  | (DNN, S-NSSAI) combination list | Indicates the (DNN, S-NSSAI) combination list for DualSteer traffic steering. |

6.1.C.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UDM/UDR:

-	Subscription data is enhanced to support DualSteer.

(H-)PCF:

-	Determine URSP based on subscription data related to DualSteer.

DualSteer device:

-	Indicate DualSteer capable indication to PCF.

* * * * End of Changes * * * *