SA WG2 Meeting #162	S2-2404740

April 15 – April 19 , 2024, Changsha

Source:	China Telecom

Title:	KI#1.4, New Sol: Policy for DualSteer steering and switching

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS/Rel-19

Abstract of the contribution: This paper proposes a new Solution to address key issue #1.4 of the FS_MASSS.

1	Discussion

This paper proposes a new Solution to address Key Issue #1.4 of the FS_MASSS.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * 1st Change * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> | <Key Issue #1.3> | <Key Issue #1.4> |
| #X | X |  | X |  |

* * * 2nd Change * * * (All new text)

### 6.1.X	Solution #X: Policy for DualSteer steering and switching

#### 6.1.X.1	Description

This solution addressed Key Issue #1.4 “Policy enhancement for DualSteer”.

Policy for steering:

During the registration procedure of the DualSteer device, the AMF  may initiate UE Policy Association procedure to provide policies (e.g. DualSteer rule or URSP rule) for DualSteer traffic steering. In the UE policy association procedure, AM-PCF can retrieve the DualSteer related subscription data from UDR, e,g, DNN and S-NSSAI specific parameter assigned for DualSteer. Then the PCF can include the combination of DNN and S-NSSAI in the UE policy (e.g. DualSteer rule or URSP rule).

Policy for switching:

After the establishment of PDU Session for DualSteer, some specific service data may be steered over one 3GPP access. Considering the requirement of some specific service, e.g. large bandwidth, application can provide guidance to the network for UE policy (e.g. DualSteer rule or URSP rule) determination via NEF, so that the service data on one 3GPP access can be switched to the other 3GPP access.

Structure of Subscription data for DualSteer Pair:

To associate two subscriptions/SUPIs for DualSteer, the term “DualSteer Pair” may be introduce to indicate two SUPIs that share the same subscription profile and has corresponding subscription data stored in UDM/UDR as in Table 6.1.x.1for example:

Table 6.1.x.1: DualSteer Pair Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Pair | DualSteer Pair Identifier | Identifies the pair of SUPIs that share the same subscription profile that supports DualSteer. |
|  | SUPI list | Corresponding SUPI list, i.e., 2 SUPIs |
| DualSteer Pair Data | DualSteer Pair Identifier | Identifiers of the DualSteer Pair that the DualSteer Pair Data belongs to. |
|  | DualSteer data | (NOTE 1) |
| NOTE 1: The content of DualSteer data is subject to the solutions to other KIs on DualSteer, considering the subscription information related to access and mobility, session management, and policy aspects, for example the DNN an S-NSSAI specific parameters to support DualSteer. |  |  |

In this solution, DualSteer Pair ID can be used to retrieve DualSteer data from UDR, so that PCF can make policy decision for DualSteer based on the DualSteer data and/or the application influence information.

Structure of UE Policy for DualSteer Pair:

Table 6.1.x.2: UE Policy for DualSteer Pair

| Information name | Description | Category | PCF permitted to modify | Scope |
| --- | --- | --- | --- | --- |
| DualSteer Rule | 1 or more Dualsteer Rule as specified in Table 6.1.x.3 | Optional | Yes | UE context |

Editor's Note: To define new DualSteer Rule or to expand URSP Rule is left to normative phase.

Table 6.1.x.3: DualSteer Rule

| Information name | Description | Category | PCF permitted to modify | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the dualsteer rule is enforced in the DualSteer device | Mandatory | Yes | UE context |
| Indication for reporting DualSteer Rule enforcement | Determines the need for reporting the DualSteer rule enforcement in the DualSteer device. | Optional | Yes | UE context |
| Traffic descriptor (maybe the same as the Traffic descriptor in Table 6.6.2.1-2 in TS23.503 for URSP Rule) | This part defines the Traffic descriptor components for the DualSteer rule. (NOTE 1) | Mandatory |  |  |
| List of Steering Description |  | Optional (NOTE2) |  |  |
| List of Switching Description |  | Optional (NOTE2) |  |  |
| NOTE 1: The Traffic descriptor for DualSteer Rule might be the same as the traffic descriptor in Table 6.6.2.1-2 in TS23.503 for URSP Rule. NOTE 2: The DualSteer Rule might have List of Steering Description only, or List of Switching Desciprion only, or both. |  |  |  |  |

Editor's Note: the details of List of Steering Description and List of Switching Description is FFS.

Editor's Note: it is FFS that the UE policy for DualSteer recieved by SUPI1 and SUPI2 is different as separate UEs or is the same as DualSteer Pair.

#### 6.1.X.2	Procedures

6.1.X.2.1 Procedures for Policy for DualSteer Traffic Steering

Figure 6.1.x.1-1 Policy for DualSteer traffic steering

- The DualSteer device uses SUPI1 to send registration request including DualSteer capability and UE Policy Container to AMF-1 over 3GPP aceess1.

- The AMF-1 establishes UE Policy Association with the PCF when a UE Policy Container is received from the UE.

- The AMF-1 sends a Npcf_UEPolicyControl Create Request with SUPI1 and the capability of DualSteer device to PCF. PCF retrieves UE context policy control subscription data for SUPI1 which may contain DualSteer Pair ID.

- After receiving the DualSteer Pair ID, PCF sends Nudr_DM_Query including Data key “DualSteer Pair ID”, Data Set “subscription data” and Data Subset “DualSteer Pair data” to retrieve DualSteer data from UDR.

- PCF generates UE policy based on the received DualSteer data, e.g. DNN and S-NSSAI specific parameter assigned for DualSteer, and send Npcf_UEPolicyControl Create Response to AMF-1.

- The remaining registration procedure described in clause 4.2.2.2 of TS23.502.

- The UE policy association procedure for SUPI2 is the same as SUPI1 as described above.

- When the DualSteer device detects a new application, it follows the received traffic steering rule to associates all the application flows to the DualSteer PDU Session of SUPI1 or SUPI2

NOTE：If the new application does not match any traffic descriptor in the Dualsteer Rule, or only one of the DualSteer Pair is registered, the UE will handled the application according to current URSP rule.

6.1.X.2.2 Procedures for Policy for DualSteer Traffic Switching

Figure 6.1.x.1-2 Policy for DualSteer traffic switching

- After the establishment of PDU Session for DualSteer, some specific service data may be steered on 3GPP access-1 as in clause 6.1.X.2.1.

- According to network situation change or service requirement from AF, PCF may decide whether to update the policy to guide the switching of the service data, i.e. switching from 3GPP access-1 (e.g. E-UTRAN) to 3GPP access-2 (e.g. NG-RAN).

For cases that are influenced by service requirement, AF may send guidance to UDR for UE policy (e.g. DualSteer rule) determination via NEF. And then, PCF may receive the update on the AF guidance information once it has subscribed to the information using Nudr_DM_Subscribe. Based on the received guidance information and the DualSteer data from UDR, PCF may decide whether to update the policy.

- The policy generated by PCF is sent to DualSteer device by UE Configuration Update procedure described in clause 4.2.4.3 of TS23.502.

- Based on the policy, device may decide to switch the traffic of the service data to an existing PDU Session over 3GPP access-2, or initiate a new PDU Session for switching if there is no proper existing PDU Session over the other access.

If a new PDU Session for switching is needed, the anchor UPF which should be the same with the UPF used in the PDU Session before switching needs to be selected during the establishment procedure.

#### 6.1.X.3	Impacts on services, entities and interfaces

PCF:

- Generating PCC rules for DualSteer service based on DualSteer data and/or application data

UDM/UDR:

- 	Providing DualSteer related subscription data to PCF for DualSteer related policy.

* * * * End of Changes * * * *