3GPP TSG-WG SA2 Meeting #162	S2-2404783

Changsha, China, April 15th – 19th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and Conclusion for KI#6 Standalone Data Channel

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_ph2 / Rel-19

Abstract: This pCR add the evaluation and conclusion for KI#6 in TR 23.700-77

# 1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

## 7.x Evaluation Related to Key Issue #6

There are six solutions for KI#6, which are solution #14, #15, #16, #19, #20 and #21.

This section evaluates the differences among those solutions based on the following criteria:

-	Whether additional subscription for standalone data channel is needed.

-	Whether additional media negotiation for standalone data channel is needed.

-	Whether an MMTel session can be downgrade to a standalone data channel session, and/or, whether a standalone data channel session can be upgraded to an MMTel session.

|  | Subscription | Media Negotiation | Upgrade/Downgrade |
| --- | --- | --- | --- |
| Sol#14 | No | No | Yes |
| Sol#15 | NA | NA | No |
| Sol#16 | NA | NA | NA |
| Sol#19 | Yes | Yes | No |
| Sol#20 | No | No | Yes |
| Sol#21 | Yes | Yes | NA |

# * * * * Second change * * * *

## 8.x Conclusions Related to Key Issue #6

The following conclusions are agreed for standalone data channel:

-	The IMS data channel is considered as one kind of media supported by MMTel session, and IMS standalone data channel is considered as a kind of MMTel session.

-	UE and IMS network do not negotiate support of standalone IMS data channel service. When receiving an SDP offer with standalone data channel, the network or UE that does not support standalone data channel shall return SIP 400 Bad Request response to inform the offerer that it does not recognize the syntax of the standalone DC.

-	The UE establishes an IMS session for a standalone bootstrap data channel, standalone application data channel or a combination of standalone bootstrap data channel and standalone application data channel(s). Simplified call establishment procedures may be applied as there is no need to use SIP preconditions.

-	The use of standalone IMS Data Channel shall be included as a part of the IMS Data Channel subscription information, which stored in the transparent data in HSS.

-	The standalone IMS data channel session is terminated by the existing session release procedure specified in TS 23.228 [5].

-	UE should support multiple Standalone IMS data channel sessions simultaneously.

-	The SDP may include version control information in the SDP during media negotiation for standalone IMS data channel session. This information may be obtained by UE during the download of a given DC app and shall be cached in the UE if the given DC app is cached in the UE. The version control information is used to guarantee that both UEs have version compatible DC app.

NOTE:	The version control information needs to be defined in SA4. Further collaboration with SA4 is needed.

-	If terminating UE reject the standalone application data channel due to lack of the version compatible DC app, the terminating UE should include the reason of rejection in the SDP answer.

# * * * * End of changes * * * *