3GPP TSG-WG SA2 Meeting #162	S2-2404792

Changsha, China, April 15th – 19th, 2024	(revision of S2-24xxxxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, Update of Solution #22: Removal of EN

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This contribution update Solution #22 by removing EN and provide the Control of UE energy consumption requirement for KI#2 in TR 23.700-66.

# 1. Introduction

This contribution update Solution #22 by removing EN and provide the Control of UE energy consumption requirement for KI#2 in TR 23.700-66

# 2. Proposal

It is proposed to capture the following changes vs TR 23.700-66 v0.4.0.

# * * * * First change * * * *

## 6.22	Solution #22: Energy related UE subscription information

### 6.22.1	Key Issue mapping

This solution resolves KI#2 for energy related UE subscription information.

### 6.22.2	Functional Description

UE subscription information needs to be consolidated to support energy saving effectively. The proposed energy related UE subscription information is structured in Table 6.22.2-1 and the details are as follows.

-	Energy Efficiency (EE) threshold: Specifies the threshold of energy efficiency for energy saving related procedures such as energy related information exposure, policy control and etc.

-	Energy Consumption (EC) Requirements: Specifies the requirements for the UE's maximum energy consumption in a specific period of time or space, or the requirements for the UE's maximum energy credit limit.

-	Renewable energy ratio threshold: Specifies the threshold of renewable energy ratio for energy saving related procedures such as energy related information exposure, policy control and etc.

Table 6.22.2-1: Energy related UE subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| Energy related subscription data | Energy Efficiency (EE) threshold | Energy Efficiency threshold |
| (data needed for considering EE, EC, | Energy Consumption (EC) Requirements | Energy consumption related requirements |
| renewable energy usage) | Renewable energy ratio threshold | Renewable energy ratio threshold |

### 6.22.3	Procedures

### 6.22.3.1	Energy saving operations based on energy related UE subscription data

The energy related UE subscription information is maintained in UDM/UDR and can be retrieved by other NFs for energy saving related operations such as AM/SM policy control, PDU session establishment/modification and etc.

NOTE:	The proposed UE subscription information is general to other energy saving related solutions. We reuse the procedures of solution #7 to show how to use the proposed UE subscription information.

Figure 6.22.3.1-1: UE subscription data based energy saving operations

1.	The energy related UE subscription data is provisioned in UDM/UDR. This provisioning process can occur during the initial user contract phase, or dynamically via UDM services.

2.	The same procedure of Solution #7 step 1-2.

3.	EECF retrieves the energy related UE subscription data from UDR.

4.	The same procedure of Solution #7 step 7-10 to release the PDU session.

#### 6.22.3.2	Energy control operations based on energy related UE subscription data

Figure 6.22.3.2-1: UE subscription data based UE's energy consumption requirements control operations

- The same procedure of step 1 of 6.22.3.1.

- For a UE subject to control of energy consumption (EC) requirements, during the PDU session establishment procedure, PCF checks and retrieves UE’s energy consumption requirements based on the subscription data with UDR/UDM.

- PCF provides PCC rule(s) with indication(s) for the control of energy consumption (EC) requirements related to the UE's maximum energy consumption in a specific period of time, or the UE's maximum energy credit limit or both to the SMF.

- SMF configures the UE's maximum energy consumption limit, or the UE's maximum energy credit limit with CHF for the control of energy consumption (EC) requirements.

NOTE 1: The procedure of the control of UE current energy consumption for the energy consumption (EC) requirements related to UE's maximum energy consumption in a specific period of time is presented in Solution #X (S2-2404798).

NOTE 2: The procedure of the control of UE energy credit for the energy consumption (EC) requirements related to UE's maximum energy credit limit is presented in Solution #Y (S2-2404799).

### 6.22.4	Impacts on existing services, entities and interfaces

UDM/UDR:

-	Should support the proposed energy related UE subscription data.

PCF/AMF/SMF/CHF/EECF:

-	Should support the energy related UE subscription data retrieval and/or processing.

# * * * * End of changes * * * *