SA WG2 Meeting #162	S2-2404848

April 15 - 19, 2024 Changsha, China	(revision of)

Source:	Ericsson

Title:	Preliminary evaluation and conclusions on applying User id towards IMS

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC/Rel-19

Abstract of the contribution: proposes preliminary evaluation and conclusions on applying User id towards IMS re-using IMS subscription currently defined.

# 1. Introduction

TS 22.101 [2] has the following requirements regarding support for User identifiers in the context of IMS  :

The 3GPP system shall be able to provide a User Identifier to a user. The User Identifier shall be independent of existing identifiers relating to subscription (e.g. IMSI, MSISDN, IMPI, IMPU, SUPI, GPSI). The User Identifier may be provided by some entity within the operator’s network or by a 3rd party.

The 3GPP system shall support a mechanism to perform authentication of a User Identity, regardless of the access, the UE and its HPLMN as well as the provider of the User Identifier.

The 3GPP system shall be able to store or link user specific service settings and parameters with the User Identifier. Those shall include network parameters (e.g. QoS parameters), IMS service (e.g. MMTEL supplementary services) and operator deployed service chain settings.

The above implies that users wanting to use a device associated to an IMS subscription in HSS, can be allocated an identifier that is independent of the IMS subscription associated to the device and supported identifiers in the IMS subscription and be able  as well to have their own unique service profiles. These users can also have their own IMS subscription tied to their own device.

# 2. Discussion

IMS currently support the following data model in TS 23.228:

The relation of a shared Public User Identity (Public-ID-2) and Private User Identities

With the following explanation to the Figure based on TS 23.228:

An IMS subscription may support multiple IMS users.

NOTE 1:	The Public User Identity sharing mechanism described above is not intended to support sharing of identities across large numbers of Private User Identities, since this would result in all these users being forced to be associated with the same IMS subscription and hence the same S-CSCF.

NOTE 2:	Subscription data is assumed to indicate which Public User Identities within a subscription are shared and which are not.

All Service Profiles of a user shall be stored in the same HSS, even if the user has one or more shared Public User Identities.

IMS subscription is an abstraction for a static association of one or more private IMPI(s), where each IMPI represents a UE,  to a single IMS subscription for administrative purposes. This abstraction also enables HSS, as specified in TS 23.228, to share multiple IMPUs amongst multiple devices (IMPIs), but this sharing is static.

To support dynamic sharing of identities among any devices out of e.g., a family subscription, requires the ability to either create dynamically an IMS subscription linkage between these devices, or remove some of the current restrictions in IMS such as the use of the same S-CSCF for a registered IMPU. Such dynacimy needs to be thoroughly studied as the implications can be quite extensive. This cannot be addressed in the timeframe for Release 19 for this SI.

It is thus proposed to restrict sharing devices in Release 19 to members, of e.g. a family, using the same IMS subscription.

# 3. Conclusion

It is proposed that in Release 19 only IMS subscriptions allowing more than one IMPI and multiple IMPUs, e.g. family subscription, is supported, e.g. allowing members of a family to share devices with their own identity.

# 4.	Proposal

It is proposed to update TR 23.700-32 v0.2.0 with the following:

Release 19 supports only IMS subscriptions, as currently defined in TS 23.228, allowing more than one IMPI and multiple IMPUs, e.g. allowing members of a family to share devices with their own identity.

* * * Start of Changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles".

[3]	3GPP TS 22.115: "Service aspects; Charging and billing".

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TR 22.904: "Study on user centric identifiers and authentication (Release 16)".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	3GPP TS 23.228: "IM Multimedia Subsystem (IMS); Stage 2".

* * * Next Change * * * *

# 7	Overall Evaluation

Editor's note:	This clause provides evaluations of different solutions.

## 7.X. Preliminary evaluation of KI#1

There is already support in TS 23.228 [x] for an IMS subscription enabling HSS to share multiple IMPUs amongst multiple devices (IMPIs); this sharing is static. This can be used for service differentiation of IMS services.

* * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X. Preliminary conclusion of KI#1

## Service differentiation of IMS service is based on IMS subscription as currently specified in TS 23.228 [], clause 4.3.3.4.

* * * End of changes * * * *