3GPP TSG-WG SA WG2 Meeting #162	S2-2404910

Changsha, China, 15th –19th April, 2024                        (revision of S2-2402735)

Source:	CATT, ETRI

Title:	New solution for KI#1.1 for Subscription aspects to support DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new solution for FS_MASSS KI#1.1.

1	Discussion

This paper proposes a new solution for KI#1.1: Subscription aspects to support DualSteer.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | Key Issue #1.1 | Key Issue #1.2 | Key Issue #1.3 | Key Issue #1.4 |
| #X | X |  |  |  |

* * * * Next Changes(all new next) * * * *

6.1.X	Solution #X: The UE subscription data for DualSteer in the UDM/UDR

6.1.X.1	Description

This solution addresses the Key Issue #1.1: Subscription aspects to support DualSteer.

This solution shows how the UDM/UDR identifies and associates the two subscriptions/SUPIs for the DualSteer Device in the UE subscription data.

This solution is based on the following assumptions:

-	The DualSteer Device has two UEs which identified by SUPI1 and SUPI2 separately.

The following table 6.1.X.1-1shows the example of new added “DualSteer Subscription data” in the UDM/UDR for the DualSteer Device.

Table 6.1.X.1-1 Example of DualSteer Subscription data

| DualSteer Subscription data | UE ID | DualSteer Allowed | Associated SUPI/IMSI | Primary /Leading Device | DualSteer for Emergency service indicator | Access type/RAT Allowed |
| --- | --- | --- | --- | --- | --- | --- |
|  | SUPI 1 | True | SUPI 2 | Yes | Yes |  |
|  | SUPI 2 | True | SUPI 1 | No | No |  |

The above parameters in the table 6.1.x.1-1are described as follows:

-	UE ID: Identifier for the UE which can be identified by SUPI/IMSI, GPSI/MSISDN, etc.

-	DualSteer Allowed: Indicate whether the UE is allowed to be used for DualSteer.

-	Associated SUPI/IMSI: Indicate the associated UE identifier sharing a DualSteer device, which can be identified by SUPI/IMSI.

-	Primary/Leading Device: Optionally, only if when the DualSteer Authorization Allowed is true. Indicate whether the UE is the primary/leading device, that is, the preferred device within the HPLMN.

-	DualSteer Device for Emergency service indicator: Optionally, only if when the DualSteer device indicator is true. Indicate whether the UE belonging to the DualSteer device applies for emergency service.

-	Access Type/RAT Allowed: Indicate which Access Type/RAT can be accessed by the DualSteer Device.

NOTE:	The DualSteer Subscription data can be a single Subscription data type in the UDM/UDR or a part of Access and Mobility Subscription data and/or Session Management Subscription data.

After provisioning the DualSteer Subscription data in the UE subscription data, the UDM in the 5GS can identify and associate the two subscriptions/SUPIs for the DualSteer Device if it has two single UEs.

6.1.X.2	Procedures

The procedure of AMF/SMF retrieving the UE subscription data from the UDM as defined in clause 4.2.2.2 and clause of 4.3.2.2 of TS 23.502[4] can be reused.

6.1.X.3	Impacts on services, entities and interfaces

The solution impacts the following network functions:

UDM/UDR:

-	Provision DualSteer Subscription data to identify and associate the two subscriptions/SUPIs for DualSteer Device.

UE:

-	The USIM profile should also be configured to align with the DualSteer Subscription data in the UDM/UDR.

* * * * End of Changes * * * *