SA WG2 Meeting #162	S2-2404986

Changsha, China, April 15 –19, 2024

Source:	CableLabs, Charter Communications, Nokia

Title:	KI#6: Interim conclusions

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This contribution proposes interim conclusions for KI#6.

# 1. Description of Key Issue #6

The following is described in TR 23.700-70 v0.4.0:

## 5.6 Key Issue #6: L4S for non-3GPP access networks and intermediate 5GS nodes ### 5.6.1 Description As the use-cases and applications for XRM are not limited to 3GPP access, XRM devices and applications may use non-3GPP access as a means of communication. The objective of this Key Issue is to extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes. The following aspects should be studied: - How to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S: - Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF). - Support L4S in wireline access (e.g. W-AGF). NOTE: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

# 2. Proposal

This contribution proposes conclusions for Key Issue #6 to be captured in TR 23.700-70.

Start of changes, all new text

8.X Conclusions for Key Issue #6

The following aspects are concluded as principles for the normative work:

- Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g., IPsec Child SAs) are used for carrying L4S enabled IP traffic.

- For wireline access –

  - ECN marking for L4S is supported in W-AGF. It is controlled via N2 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled W-UP resource(s).

  - ECN marking for L4S is supported in 5G-RG. It is controlled via N1 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s).

- For untrusted/trusted access –

  - ECN marking for L4S is supported in N3IWF/TNGF. It is controlled via N2 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled IPsec Child SAs.

  - ECN marking for L4S is supported in UE.

End of changes