3GPP TSG-WG SA2 Meeting #162 	S2-2404998

# 15 – 19 April 2024, Changsha, China	     	  (revision of S2-24xxxxx)

Source:	Google

Title:	(KI#2/#4) Update Solution#11 for multiplexed multiple media types traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes update to resolve Editor’s Notes for solution#11 in TR23.700-70.

# 1. Discussion

This paper proposes resolve the following Editor’s note to enable support of multiple media types multiplexed in one RTP session for a QUIC connection for Sol#11: RTP over QUIC based Encrypted Traffic Detection, Identification, and QoS flows mapping.

Editor's note: It FFS whether and how to support multiple media types multiplexed in one RTP session for a QUIC connection. Each media type would require different QoS requirements.

The principles of this proposal are based on the following two IETF RFCs.

- IETF RFC 8860: "Multiple media types within one RTP stream" [X].

- Use of SSRC (32 bits) in RTP header to distinguish RTP streams

- The RTP session used for the original streams can include multiple RTP streams, and those RTP streams can use multiple media types.

- Within an RTP session where multiple media types have been configured for use, an SSRC can only send one type of media during its lifetime (i.e., it can switch between different audio codecs, since those are both the same type of media, but it cannot switch between audio and video).

- IETF RFC 8108: "Sending Multiple RTP Streams in a Single RTP Session" [Y].

- Define a flow identifier to distinguish multiple RTP flows, RTCP flows, and non-RTP flows that are carried on the same QUIC connection.

# 2. Proposal

It is proposed to resolve the above Editor’s note and capture the following changes in TR 23.700-70.

# * * * * First change * * * *

## 6.11	Solution #11: RTP over QUIC based Encrypted Traffic Detection, Identification, and QoS flows mapping

### 6.11.1	Key Issue mapping

This solution addresses Key Issue #2: "Support PDU Set information identification for end-to-end encrypted XRM traffic", and Key Issue #4: Traffic detection and QoS flow mapping for multiplexed data flows.

### 6.11.2	Description

#### 6.11.2.1	Introduction

This solution is proposed to enable the support of PDU Set related handling for end-to-end encrypted traffic using RTP over QUIC (RoQ) [8]. RTP over QUIC allows RTP packets to be encapsulated within QUIC packets via QUIC streams and datagrams to transport real-time data within a QUIC connection for a specific IP flow (represented by IP 5-tuple).

As shown in Figure 6.11.2-1, the RTP over QUIC uses nest encapsulation that encapsulates RTP packets in QUIC payload. The necessary PDU Set Information contained in the RTP Extension header in [20] is encrypted and become undetectable by the 5G network for traffic detection and PDU Set identification. The UDP Option in [21] is a suitable tool to provide in-band metadata for the encrypted QUIC packet with encapsulated RTP packets.

Figure 6.11.2-1. IP packet with nest encapsulation for RTP over QUIC and UDP Option

#### 6.11.2.2	RTP session with one media type traffic

For RTP session with one media type traffic, as shown in Figure 6.11.2-1, different RTP sessions can be transmitted using different QUIC connections or using different QUIC streams within one QUIC connection.

The principles of the solution are as follows:

-	One QUIC packet can encapsulate one or more RTP packets that share the same QUIC connection and RTP session properties, e.g. media frame type (I/P/B type), QUIC connection and QUIC stream, PDU Set, Data burst, PDU Set Importance, fragmented media frames (of the same type).

-	Different QUIC streams which are synchronous can be used to transmit RTP packets with different media frame type.

-	The metadata that contains necessary RTP session information includes:

-	Correlation ID: a fixed length hash value generated by the application, which is used to associate the QUIC Connection ID that may be changed throughout the lifetime of the QUIC connection.

-	Stream ID: Stream ID or a mapped value of Stream ID contained in the QUIC header of the QUIC packet.

-	Priority of the QUIC stream based on the media properties, e.g. media types, media frame types.

-	Timestamp: to provide time instance information of the last encapsulated RTP packet. RTP header extension element for the RTP packet already can carry the timestamp of the media frame. This information can help in band delay measurement.

-	QUIC packet information: includes information of RTP extension header in [20] for the last RTP packet and the following:

-	Number of RTP packets in the QUIC packet: can be used by the NG-RAN to calculate the dropping rate of the RTP packets if dropping an QUIC packet.

NOTE:	The Correlation ID and Stream ID can be generated based on one or more components of the traffic descriptions: IP flows (each is represented by an IP 5-tuple), QUIC connections, and QUIC streams, e.g. to be associated to a specific QoS requirement of an QoS flow.

-	The AF sends AF request message including QoS requirement and assistance information for the media traffic to the NEF/PCF including:

-	QoS requirement contains PDU Set based QoS parameters.

-	Traffic description includes IP 5-tuple.

-	Additional traffic description that is included in the metadata:

-	Transport Connection Correlation ID.

-	Transport Connection Associated Stream ID(s).

-	Protocol Description: indicate the real-time transport layer protocol applied for the encapsulation layer for user plane traffic over N6 [20].

-	Additional Transport layer Protocol Description: indicate transport layer protocol that encapsulates upper layer packets, including: QUIC over UDP, UDP-Option.

-	PDU Set Handling List of Stream ID(s) that require PDU Set Handling within the QUIC connection.

Editor's note:	It FFS whether and how to also support for UL direction.

#### 6.11.2.3	RTP session with multiplexed multiple media types traffic

To enable QoS provisioning for a service data flow with multiplexed multiple media types traffic, there requires application to properly manage RTP streams that multiplexed different media types within one RTP session for an QUIC connection. For example,

- The RTP session can include multiple RTP streams, and those RTP streams can use multiple media types. Within an RTP session where multiple media types have been configured for use, an SSRC can only send one type of media during its lifetime (i.e., it can switch between different audio codecs, since those are both the same type of media, but it cannot switch between audio and video) [X], [Y].

- RTP packets with different media types in one RTP session are transmitted via different QUIC streams within one QUIC connection, in which each QUIC stream is corresponding to different media type traffic multiplexed in one RTP session. That is, one RTP stream with a specific media type within one RTP session is mapped to one QUIC stream within one QUIC connection.

Therefore, for an RTP session with multiplexed multiple media types traffic, each media type traffic can be transmitted using different QUIC streams within one QUIC connection. The AF needs to provide QoS requirement per QUIC stream level granularity.

The principles of the solution are as follows:

- The application uses SSRC information in RTP packet header to differentiate media type of a RTP stream within a RTP session (with a specific flow identifier, FID) and then use different QUIC stream for different media type of the RTP stream in the RTP session.

- QoS requirement: for different media types, the QoS requirements of a media traffic is associated to the QUIC stream that carries a RTP stream within a RTP Session in the QUIC connection.

- The PCF/SMF/UPF need to be enhanced to handle multiple PCC rules for multiple QoS flows with different parameters associated with different media types for multiple correlated QUIC streams within one service data flow. The UPF based on the N4 rules and metadata over N6 to identify a QUIC stream that requires a corresponding QoS parameters.

- Metadata for identifying a RTP stream within a QUIC connection:

- Correlation ID: a fixed length hash value generated by the application, which is used to associate the QUIC Connection ID that may be changed throughout the lifetime of the QUIC connection.

- Media Stream ID: unique within a QUIC Connection to identify a QUIC stream that encapsulates “RTP stream” with a specific media type in a QUIC connection, which can be set based on QUIC Stream ID within the associated QUIC Connection, as contained in the QUIC header of the QUIC packet or based on SSRC, as contained in the RTP header of the RTP packet encapsulated in the QUIC packet or based on a fixed length random number, e.g. using a hash function, generated by the application.

- The AF sends AF request message including QoS requirement and assistance information for the media traffic to the NEF/PCF including:

- QoS requirement for the QUIC connection (based on IP 5-tuple) and additional QoS requirements for the media traffics within a QUIC connection including QoS requirements and corresponding QoS correlation ID for associating additional traffic description.

- Traffic description includes IP 5-tuple and additional traffic description includes QoS Correlation ID, including Transport Connection Correlation ID, and Transport Connection Associated Stream ID. The QoS Correlation ID is indicated in the metadata for identifying the QUIC stream that requires different QoS parameters.

- Protocol Description indicates RTP/SRTP with RTP extension header defined in TS26.522 [20] for the applicable QUIC connection and QUIC stream. This IE indicates the real-time transport layer protocol applied for the encapsulation layer for user plane traffic over N6 [5].

- Additional Transport layer Protocol Description can indicate transport layer protocol that encapsulates upper layer packets, including: QUIC over UDP, UDP-Option, RTP. The UPF can use this information to perform corresponding traffic detection, PDU Set information identification.

- PDU Set Handling List of Stream ID(s) that require PDU Set Handling within the QUIC connection. This information indicates to the PCF/SMF/UDF for the need for PDU Set handling. Some QUIC streams that carry control signaling does not need PDU Set handling.

### 6.11.3	Procedures

#### 6.11.3.1	PDU Set based QoS handling for RTP over QUIC based encrypted traffic

Figure 6.11.3.1-1: Procedure for PDU Set based QoS handling for RTP over QUIC-based encrypted traffic

The process includes the following steps:

-	Step0: PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502 [3]) are performed. A network slice type for XR service can be used for such a PDU Session.

-	Step1, AF->NEF/PCF: the AF sends AF request, e.g Nnef_AFsessionWithQoS_Create request as defined in clause 4.15.6.6 of TS 23.502, to the 5GC via NEF/PCF to provide PDU Set based QoS requirement of the End-to-end encrypted media traffic and assistant information for traffic detection, PDU Set identification and marking on the End-to-end encrypted media traffic.

The AF provided information can be used in determining PCC Rules by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [4] and for identifying the PDU Set information from the End-to-end encrypted media traffic by the PSA UPF.

-	Step2, PCF->SMF: based on assistance information and PDU Set based QoS requirement from the AF directly or via NEF, the PCF determines that PDU Set based QoS Handling is to be performed for End-to-End encrypted traffics from a service data flow or one or more media streams within a service data flow for an application, the PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration to generates PCC rule(s), containing the PDU Set QoS parameters and assistance information for the media traffic from a service data flow or one or more media streams within a service data flow. In step 2b, the PCF sends PCC rules to the SMF.

-	Step3, SMF: According to the PCC rules from PCF, the SMF performs QoS flow mapping to map a service data flow to a QoS flow, determines a QoS Profile for the QoS Flow or map one or multiple specific media streams within a service data flow to one or multiple QoS flows, and determines N4 rules including QoS Enforcement Rule with PDU Set based QoS parameters and Packet Detection Rule with assistance information included in Packet Detection Information. Alternatively, the SMF may be configured to support PDU Set QoS handling without receiving PCC rules from a PCF.

-	Step 3a: the SMF sends the N4 rules including QER and PDR with Packet Detection Information for a service data flow or one or more media streams within a service data flow to the PSA UPF.

-	Step 3b: the SMF sends the QoS profiles and QoS rules containing in the NAS message to the NG-RAN via AMF.

-	Step 3c: the SMF sends the QoS rules in a NAS message to the UE via AMF and NG-RAN.

-	Step4, AS/AF: the application encapsulates RTP packets in QUIC packets and transmits the QUIC packet with UDP option containing metadata within a QUIC connection containing one or more QUIC streams over N6 interface to PSA UPF.

-	Step5, UPF: When receiving IP packets from Application server over N6, the PSA UPF performs traffic detection based on configured PDR received in step 3a and PDU Set Identification based on the metadata included in the UDP-Option and the assistance information for the media traffic of a service data flow or one or more media streams within a service data flow received from the SMF.

-	Step 5a: based on the PDR with assistance information configured by the SMF via N4 session in step 3b and the metadata included in the UDP-Option from an IP packet over N6 interface, the PSA UPF can identify a PDU that belongs to a specific PDU Set and performs PDU Set based QoS handling according to QER included in N4 rule.

-	Step 5b: UPF->NG-RAN over N3: the PSA UPF marks the identified PDU with PDU Set Information in GTP-U header to provide PDU Set Information to the NG-RAN.

-	Step6, NG-RAN: Based on the PDU Set Information in GTP-U header, RAN performs PDU Set based QoS handling, and maps the QoS flow to one or more DRB.

-	Step 6a: the NG-RAN transmits DRBs over NR-Uu to the UE.

-	Step 6b: the UE maps the received DRBs to the QoS flows based on QoS rules and then forward Downlink packet to XRM application.

### 6.11.4	Impacts on services, entities and interfaces

AF:

- enhance to support IP flow with multiplexed media types

- provide QoS requirements and assistance information per QUIC stream level granularity

PCF:

- generate multiple PCC rules for multiple QoS flows with different parameters associated with different media types for multiple correlated QUIC streams within one service data flow.

SMF:

- generate multiple QoS flows based on PCC rules

- configures N4 rules per each QoS flow.

UPF:

- enhanced to support multiple N4 rules for a service data flow

- enhanced to perform PDU Set Identification based on metadata over N6 using corresponding transport level protocol.

- based on the N4 rules and metadata over N6 to identify a QUIC stream that requires a corresponding QoS parameters

# * * * * 2nd change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issue # |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <KI #1> | <KI#2> | <KI#3> | <KI#4> | <KI#5> | <KI#6> | <KI#7> | <KI#8> | <KI#9> |
| #1: PDU Set content ratio awareness at RAN | X |  |  |  |  |  |  |  |  |
| #2: Discarding of redundant PDUs (FEC) and reporting | X |  |  |  |  |  |  |  |  |
| #3: FEC mechanism and PSI based PDU Set QoS Handling Enhancement | X |  |  |  |  |  |  |  |  |
| #4: PDU Set FEC-based PDU Set QoS Handling | X |  |  |  |  |  |  |  |  |
| #5: PDU Set Handling and Information marking …for PSDB/PSER/PSIHI | X |  |  |  |  |  |  |  |  |
| #6: Enhanced Alternative QoS Profiles for PDU set based QoS handling | X |  |  |  |  |  |  |  |  |
| #7: Enhancing alternative QoS profile …PDU set QoS parameters | X |  |  |  |  |  |  |  |  |
| #8: Consistent PDU Set Handling between AF and 5GS | X |  |  | X |  |  |  |  |  |
| #9: PDU Set information identification for encrypted traffic |  | X |  | X |  |  |  |  |  |
| #10: PDU Set information identification based on MoQ (CMCC) |  | X |  |  |  |  |  |  |  |
| #11: RTP over QUIC based Encrypted Traffic …QoS flows mapping (Google) |  | X |  | X |  |  |  |  |  |
| #12: Obfuscated Metadata to Classify Payload in Encrypted Media Packets (Futurewei) |  | X |  | X | X |  |  |  |  |
| #13: Multiple DSCP markings per QoS Flow (Intel) |  |  | X |  |  |  |  |  |  |
| #14: Extending Packet Filter … within a single transport connection |  |  |  | X |  |  |  |  |  |
| #15: Traffic Detection and QoS mapping for XR and Media services |  |  |  | X |  |  |  |  |  |
| #16: AS based trigger of data boost handling with reflective QoS |  |  |  |  | X |  |  |  |  |
| #17: L4S in non-3GPP access networks |  |  |  |  |  | X |  |  |  |
| #18: PDU Set handling in wireline/wireless non-3GPP access |  |  |  |  |  |  | X |  |  |
| #19: Alternative PDU Set QoS parameters to support differentiated QoS handling and ... Exposure | X |  |  |  |  |  |  |  | X |
| #20: Nominal PSDB | X |  |  |  |  |  |  |  |  |
| #21: Enhancing PDU Set QoS Handling with Dynamic FEC Related Information Marking in GTP-U | X |  |  |  |  |  |  |  |  |
| #22: The handling UL PDU Set QoS parameters | X |  |  |  |  |  |  |  |  |
| #23: PDU set discard based on PDU sets correlation info from AS/AF | X |  |  |  |  |  |  |  |  |
| #24: PDU set identification ...fully encrypted using a tunneled connection over N6 |  | X |  |  |  |  |  |  |  |
| #25: Preconfigured N6 tunnelling and GTP-U header extension for ... PDU Set-related information |  | X |  |  |  |  |  |  |  |
| #26: PDU Set identification for end-to-end encrypted traffic |  | X |  |  |  |  |  |  |  |
| #27: Differentiated Handling for Transporting Encrypted XRM traffics Using Metadata over N6 (Google) |  | X | X | X |  |  |  |  |  |
| #28: QoS Flow Mapping Considering the PSI for Multiplexed Data Flows |  |  |  | X |  |  |  |  |  |
| #29: Support for multiplexed media traffic using RTP header inspection |  |  |  | X |  |  |  |  |  |
| #30: Support of dynamic change of traffic burst size |  |  |  |  | X |  |  |  |  |

# * * * * 3rd change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[33]	IETF RFC 7656: "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources".

[34]	S2-2210181: "Reply LS on further details on XR traffic".

[35]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[36]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[37]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[38]	IETF RFC 9298: "Proxying UDP in HTTP".

[39]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[40]	IETF draft-ietf-masque-quic-proxy-01: "QUIC-Aware Proxying Using HTTP".

[41]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[42]	IETF RFC 5764:" Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[43]	IETF RFC 7983: "Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)".

[44]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[45]	IETF RFC 9443: "Multiplexing Scheme Updates for QUIC".

[46]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[X] 	IETF RFC 8860: "Multiple media types within one RTP stream".

[Y] 	IETF RFC 8108: "Sending Multiple RTP Streams in a Single RTP Session".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

# * * * *End change * * * *