3GPP TSG-SA WG2 Meeting #162	S2-2405050

April 15 – April 19, 2024; Changsha, China	(revision of S2-2403956)

Source: 	Samsung

Title: 	23.700-77: KI#8, New Sol: Support of IMS DC based avatar communication

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This paper proposes solution for KI#8.

1 Introduction

This paper proposes new solution for KI#8 in TR 23.700-77.

2 Proposal

It is proposed to approve following content in TR 23.700-77.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| 18 |  | X |  |  |  |  |  |  |
| 19 |  |  |  |  |  | X |  |  |
| 20 |  |  |  |  |  | X |  |  |
| 21 |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  | X |  |
| 23 |  |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  |  |  | X |
| 27 |  |  |  |  |  |  |  | X |
| X |  |  |  |  |  |  |  | X |

# * * * * Second change (all text is new)* * * *

6.X	Solution #X: Support of IMS DC based avatar communication

6.X.1	Description

This solution proposes how to support Avatar communication with the avatar data repository in the IMS DC network. The avatar data stored in the network can be accessed by the IMS network only for the authorized UE.

6.X.1.1	General

Figure 6.X.1.1: Example of enhanced IMS DC Architecture to support avatar store and access

To support of IMS avatar communications, the architecture is enhanced as below:

Avatar Data Repository (ADR):

-	The Avatar Data Repository is used by the UE and the DCSF/MF to store and access to the list of Avatar IDs and its corresponding Avatar data.

-	ADR can be located within the PLMN e.g. as a new network function, as a repository connected to DCSF (like DC Application Repository), or as a trusted/untrusted application server (like DC Application Server).

The Avatar Data Repository stores and manages the information elements as following:

Avatar data:

-	Avatar data is composed of a Base Avatar, and the related information.

-	Base Avatar is the 3D representation which is generated from the inputs such as captured data from camera and other sensors information. The Base Avatar is non-timed and static data, and unique to the user.

-	Base Avatar related information is the metadata of the Base Avatar such as the format, codec, version (e.g. data of creation), associated application service(s), and other information.

-	Avatar data is accessible via an Avatar ID.

List of Avatar IDs:

-	The list of Avatar IDs is provided to the authorized UE.

-	The UE downloads the list of Avatar IDs from the DC AS during Bootstrap DC session. The UE selects an Avatar ID within the list of Avatar IDs to use for avatar session, and requests with Application DC session.

-	To download the List of Avatar IDs from the DC AS, the UE requests Bootstrap DC session with the UE ID.

Animation data:

-	The data such as facial feature points, pose, gesture that can be generated from the camera, microphones, motion capturing devices, etc. The Animation data is timed and dynamic data.

Animated avatar media:

-	The media generated with the Base Avatar and the Animation Data, which will be used in the rendering process.

NOTE :	The above information elements will be keep updated with coordination with SA4.

6.X.2	Procedures

#### 6.X.2.1	Authorization of the UE to access avatar data stored in the network

Based on Figure 5.1 in clause 5.2.2 of TS 23.228 [5], Authorize the UE for using the avatar data stored in Avatar Data Repository accessed by the IMS network for avatar communication with following differences:

-	The UE includes a media feature tag in the Contact header field in REGISTER request that indicating UE supports avatar communication.

-	The S-CSCF and HSS/UDM check the subscription data whether the user is allowed to use the avatar communication service which means the network can access to the Avatar Data Repository and the network is allowed to transfer the list of Avatar ID, and the Avatar data for this user.

-	The S-CSCF includes a Feature-Caps header field which indicating its avatar communication capability in 200 OK response to the UE.

-	The S-CSCF indicates whether the user is allowed to use the avatar communication service based on subscription data in 200 OK response to the UE.

-	After this procedure, the UE will establish IMS data channel sessions.

#### 6.X.2.2	Bootstrap data channel establishment for retrieval of identities of avatar data stored in the network

Based on the Figure AC.7.1-1 of TS 23.228 [5], the bootstrap data channel establishment for retrial of Avatar ID List can be performed as described in 6.X.2.2-1.

Figure 6.X.2.2-1: Bootstrap Data Channel set up Signalling Procedure for avatar communication

- UE sends the INVITE with the SDP offer for audio/video, and bootstrap DC.

- IMS AS validates user subscription data to determine whether it is allowed for this user that the network can access to the Avatar Data Repository and can transfer the list of Avatar ID, and the Avatar data. IMS AS decides DC Routing and DCSF discovery.

- IMS AS sends session event control notification based on the information included in the INVITE message. The IMS AS indicates that it is allowed to access to the Avatar Data Repository for this UE.

In case of DC routing to the remote network in step 2, the indication that it is allowed to access to the Avatar Data Repository behalf of the UE#1 is added in step 11. By this information, the Avatar ID List of UE#1 can be downloaded in terminating network/UE#2 for UE#2 centric avatar communication.

- DCSF decides the provision of DC, and determines the DC control policy.

- DCSF creates originating and terminating side media information.

  - Upon reception of the indication from IMS AS that it is allowed to access to the Avatar Data Repository for this UE, DCSF retrieves the Avatar ID List for the UE#1 from Avatar Data Repository, the User ID of UE#1.

  - Avatar data Repository checks whether the UE#1 is allowed to download the list of Avatar IDs with the User ID of UE#1 that DCSF provided. If the authorization is successful, then the Avatar Data Repository provides the list of Avatar IDs to the DCSF.

- DCSF returns the list of Avatar IDs to the IMS AS, which will be used by the MF when it downloads the list of Avatar IDs via MDC1.

- MF/MRF discovery and selection is performed.

- IMS AS requests MF/MRF to reserve originating and terminating side media resource. MF/MRF allocates resources for originating and terminating MDC1.

- IMS AS responses to DCSF.

- DCSF responses to IMS AS.

- The INVITE message is sent to S-CSCF.

- The INVITE message is sent to the terminating network/UE#2.

- The terminating network negotiation is performed.

- 18X message may be exchanged.

- The terminating network/UE#2 sends 200 OK to S-CSCF.

- 200 OK is forwarded to IMS AS.

- DCSF responses to IMS AS.

- 200 OK is forwarded to the UE#1.

- Step 20 to 23 of Figure AC.7.1-1 of 23.228 [5] are performed. The list of Avatar IDs for UE#1 is downloaded from Avatar Data Repository to the UE#1 via originating MF/MRF through the established bootstrap data channel.

- Subsequent procedures can be performed. After this procedure, the UE#1 initiates the application data channel with certain Avatar ID(s) of UE#1 which is selected among the Avatar ID List by the UE#1 (as described Solution #17).

6.X.3	Impacts on Existing Nodes and Functionality

This solution may have the following impacts to exiting entities and interfaces:

S-CSCF:

- S-CSCF checks whether the UE is subscribed to the avatar communication service which means the network can access to the Avatar Data Repository and the network is allowed to transfer the list of Avatar ID, and the Avatar data for this user..

IMS AS:

- IMS AS validates user subscription data to determine whether it is allowed for this user that the network can access to the Avatar Data Repository and can transfer the list of Avatar ID, and the Avatar data.

- For allowed UE, IMS AS indicates to DCSF indicates that it is allowed to access to the Avatar Data Repository for this UE.

DCSF:

- Upon reception of the indication from IMS AS that it is allowed to access to the Avatar Data Repository for this UE, DCSF retrieves the list of Avatar IDs for the UE from Avatar Data Repository.

- DCSF provides the list of Avatar IDs for the UE to MF/MRF via bootstrap data channel.

Avatar Data Repository (ADR):

- ADR stores the Avatar data, and provides to the authorized UE via bootstrap data channel.

- ADR can be located within the PLMN e.g. as a new network function, as a repository connected to DCSF (like DC Application Repository), or as a trusted/untrusted application server (like DC Application Server).

MF/MRF:

- MF/MRF provides the list of Avatar IDs for the UE to UE via bootstrap data channel.

UE:

- UE downloads the list of Avatar IDs via bootstrap data channel.

- UE selects an Avatar ID within the list of Avatar IDs, and initiates the application data channel session with the selected Avatar ID to use avatar communication.

# * * * * End of changes * * * *