SA WG2 Meeting #162	S2-2405054

Changsha, China, April 15 – 19, 2024	(revision of S2-24004347)

Source:	ZTE

Title:	KI #3, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: The paper gives primary evaluation and conclusion for KI #3.

1. Discussion

KI #3 addresses IMS data channel interworking with MTSI UE. The contribution evaluates the solutions for KI #3 and proposes some primary conclusion for this key issue.

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	KI #3 Interim Conclusion

For KI#3 on "IMS data channel interworking with MTSI UE" the following interim conclusions are agreed:

- When the originating DCMTSI UE initiates IMS DC session to the terminating MTSI UE, the originating DCSF knows the target UE does not support DC based on the SDP answer from the terminating side and determines DC interworking based on DC subscription.

- When the originating DCMTSI UE establishes application DC, the originating DCSF decides to start one-side data channel procedure in this IMS session and to anchor the data channel media on the MF. The DCSF instructs the IMS AS to create resources and association for the application data channels and how to transcode data channel media to video media for the terminating UE

- Editor's note:	The details for transmission from UE b to UE a and required transcoding is FFS.

- The MF associates the data channel media resource for originating network with the video media resource for terminating network and transform data channel media to video media.

Editor's note:	The details of instruction of how to transform data channel media to video media is FFS and needs coordination with SA4.

- Alternatively, messaging service can be used to send hyperlink for accessing the shared information for MTSI UE.

* * * * End of Changes * * * *