3GPP TSG-WG SA2 Meeting #162	S2-2405365

# Changsha, China, Apr 15 – Apr 19, 2024		(revision of S2-2404103)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4774 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the PDTQ policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in clause 6.1.2.7 of TS 23.503: The PCF may request threshold reporting or periodic reporting. If periodic reporting is received, the PCF shall determine, based on the latest analytics information and previously selected PDTQ policy, whether the PDTQ policy should be updated or not. If the PCF determines to update the PDTQ policy, the PCF may try to re-negotiate the affected PDTQ policies with AFs that accepted PDTQ policy re-negotiation. However, in the current procudure for PDTQ warning notification, the PCF can only request threshold reporting, which is not aligned with TS 23.503. There is a wording issue in the last sentence of step 11, “but no PDTQ policy and the previously negotiated PDTQ policy shall be kept” raising confusion whether the PDTQ policy shall be kept. Actually, when the AF doesn't select any of the new PDTQ policies, the previously negotiated PDTQ policy shall be kept, and the AF response only includes PDTQ reference ID, but no PDTQ policy. Steps 7 and step 8 in Figure 4.16.15.2.2-1 are incorrectly placed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the PCF can also request periodic reporting for the Network Performance analytics or DN Performance analytics. Editorial change on step 11. Update Figure 4.16.15.2.2-1. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCF can only request threshold reporting for the NWDAF analytics. It’s unclear whether the PDTQ policy shall be kept when the AF doesn't select any of the new PDTQ policies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.15.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.15.2.2	Procedure for PDTQ warning notification

UDR

H-PCF

NEF

AF

2. Network performance has degraded

3. Nudr_DM_Query request

4. Nudr_DM_Query response

7. Npcf_PDTQPolicyControl_Notify

8. Nnef_PDTQPolicyNegotiation_Notify

9. AF checks PDTQ policies in the candidate list

10. Step 9-14 from section 4.16.15.2.1 are executed

11. Step 9-12 from section 4.16.15.2.1 are executed

Alternative B: AF does not select any PDTQ policy from the candidate list

Alternative A: AF selects a PDTQ policy from the candidate list

1. Negotiation for PDTQ policies is complete

5. Identifying affected PDTQ policies

6. Identifying new candidate PDTQ policies

Figure 4.16.15.2.2-1: The procedure for PDTQ warning notification

1.	The negotiation for PDTQ policy as described in clause 4.16.15.2.1 is completed. In addition, the PCF has subscribed to analytics on "Network Performance" or "DN Performance" from NWDAF for the area of interest and time window of a PDTQ policy following the procedures and services described in TS 23.288 [50], including a Reporting Threshold or periodic reporting in the Analytics Reporting information. The value for Reporting Threshold is set by the PCF based on operator configuration.

2.	The PCF is notified with the Network Performance analytics or DN Performance analytics in the area of interest from the NWDAF when the NWDAF determines that the Network Performance or DN Performance reaches the Reporting Threshold or according to periodic reporting, as described for the Network Performance analytics or DN Performance analytics in TS 23.288 [50]. In case of periodic reporting, the PCF shall determine, based on the latest analytics information and previously selected PDTQ policy, whether the PDTQ policy should be updated or not. If the PCF determines to update the PDTQ policy in case of periodic reporting, or the PCF receives the threshold reporting, the following steps are executed.

3.	The H-PCF requests from the UDR the stored PDTQ policies using Nudr_DM_Query (Policy Data, Planned Data Transfer with QoS requirements) service operation.

4.	The UDR provides all the PDTQ policies together with the relevant information received from the AFs (as defined in clause 6.1.2.7 of TS 23.503 [20]) to the H-PCF.

5.	The H-PCF identifies the PDTQ policies affected based on the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the PDTQ traffic will be influenced by the degradation of network Performance or DN Performance and which requested the H-PCF to send the notification. The PCF then performs the following steps for each of the determined ASPs, i.e. Steps 6 - 11 can occur multiple times (i.e. once per ASP).

6.	The PCF decides based on operator policies, whether a new list of candidate PDTQ policies can be calculated for the ASP. If the PCF does not find any new candidate PDTQ policy, the previously negotiated PDTQ policy shall be kept, no interaction with that ASP shall occur and the procedure stops for that PDTQ policy.

NOTE:	The PDTQ policies of an ASP which did not request to be notified are kept and no interaction with this ASP occurs.

7.	The PCF sends the notification to the NEF by invoking Npcf_PDTQPolicyControl_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

8.	The NEF sends the PDTQ warning notification to the AF by invoking Nnef_PDTQPolicyNegotiation_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

9.	The AF checks the new PDTQ policies included in the candidate list in the PDTQ warning notification.

10.	If the AF selects any of the new PDTQ policies, the steps 9-14 from clause 4.16.15.2.1 are executed with the difference that the AF has to respond as well when only one PDTQ policy was provided by the PCF and the PCF removes the no longer valid PDTQ policy from the UDR for the corresponding PDTQ Reference ID.

11.	If the AF doesn't select any of the new PDTQ policies, the steps 9-12 from clause 4.16.15.2.1 are executed, with the AF indicating that none of the candidate PDTQ policies is acceptable. In this case, the AF response only includes PDTQ reference ID (without PDTQ policy), and the previously negotiated PDTQ policy shall be kept.

The AF can send a Stop notification by invoking Nnef_PDTQPolicyNegotiation_Update service, when the AF requests not to receive the PDTQ warning notification anymore. Then, the NEF invokes Npcf_PDTQPolicyControl_Update service in order to provide this information for the H-PCF.

# * * * * End of changes * * * *