SA WG2 Meeting #162	S2-2405373

Changsha, China, April 15 –19, 2024	(was S2-2405103)

Source:	CableLabs, Charter Communications, Nokia, Tencent, Tencent Cloud

Title:	KI#6: Interim conclusions

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This contribution proposes interim conclusions for KI#6.

# 1. Description of Key Issue #6

The following is described in TR 23.700-70 v0.4.0:

## 5.6 Key Issue #6: L4S for non-3GPP access networks and intermediate 5GS nodes ### 5.6.1 Description As the use-cases and applications for XRM are not limited to 3GPP access, XRM devices and applications may use non-3GPP access as a means of communication. The objective of this Key Issue is to extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes. The following aspects should be studied: - How to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S: - Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF). - Support L4S in wireline access (e.g. W-AGF). NOTE: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

# 2. Proposal

This contribution proposes conclusions for Key Issue #6 to be captured in TR 23.700-70.

Start of changes, all new text

8.X Conclusions for Key Issue #6

The following aspects are concluded as principles for the normative work:

- Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g., IPsec Child SAs) are used for carrying L4S enabled IP traffic.

- For wireline access –

  - ECN marking for L4S is supported in W-AGF. It is controlled via N2 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled W-UP resource(s).

  - ECN marking for L4S is supported in 5G-RG in UL. It is controlled via N1 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s).

- For untrusted/trusted access –

  - ECN marking for L4S is supported in N3IWF/TNGF. It is controlled via N2 signaling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled IPsec Child SAs.

  - N3IWF/TNGF in UL shall support and UE in DL can support the IP-in-IP tunnel behavior of copying ECN bits between outer and inner headers as per IETF RFC 6040 [x].

NOTE: To support this functionality, the UE needs to support L4S feedback as described in IETF RFC 9330 [14] which is not in the scope of 3GPP.

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[33]	IETF RFC 7656: "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources".

[34]	S2-2210181: "Reply LS on further details on XR traffic".

[35]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[36]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[37]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[38]	IETF RFC 9298: "Proxying UDP in HTTP".

[39]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[40]	IETF draft-ietf-masque-quic-proxy-01: "QUIC-Aware Proxying Using HTTP".

[41]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[42]	IETF RFC 5764:" Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[43]	IETF RFC 7983: "Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)".

[44]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[45]	IETF RFC 9443: "Multiplexing Scheme Updates for QUIC".

[46]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[x]	IETF RFC 6040: " Tunnelling of Explicit Congestion Notification".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

End of changes