3GPP SA WG2 Meeting #162	S2-2405547

Changsha, China, 15 – 19 April 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #3: New Sol: Third party provisioning for no transmission zone and band restriction for Aerial vehicle

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a solution to KI#3: Support of No Transmit Zones

# Introduction

This contribution proposes third-party provisioning aspects for controlling UE to honour the NTZ area, which is related to the introduction of No Transmit Zones for aerial UEs by CEPT decision 22(07) [3]. The ECC Decision asserts that a mechanism is necessary to ensure that aerial UEs respect no-transmit zones in order to protect incumbent radio systems from potential interference from aerial UEs.

# Discussion

The NTZ area is demarcated as a geographical location where UE is not to transmit any radio signal. One such example is the upward-looking beam, which might cause interference due to UE being close/within the visible path of an ATC communicating with planes. The drones can still fly at an allowed level where upward-looking beams from the base station do not interfere with the ATC radio system.

Figure 1 NTZ application based on UAVs height where the ATC can have interference

- How to provision and control Aerial vehicles about the no-transmission zone in its pre-mission flight path?

- How to provision and control the Aerial vehicle about the band restrictions in its flight path?

This solution proposes to enable the 5GC to provision and configure the UAV-UE on the no-transmission zone information and band restriction per geography information (or even TA/RA and Cell level).

It is proposed to use a new API to provision NTZ information in UDR at a group level (e.g., UEs of certain group or categories are restricted) or individual UE level (e.g., only selected UEs are restricted), or even PLMN level (e.g., all the UEs of all PLMNs or any specific PLMN are restricted).

Any external entity (such as USS/TPAE) will use 5GC to provide NTZ information at group, PLMN, or individual UE levels.

PCF will use this new UDR provisioning information to come up with NTZ information and deliver it to UE as part of the NTZ policy delivery process.

UE will follow the NTZ information to behave differently in different geographies by following the configured NTZ policy.

Also, the pre-mission flight path plan received from an external entity to 5GC will enable 5GC to devise advance policy notification if a UE’s upcoming flight path goes via the NTZ or band-restricted area.

# Proposal

A solution is proposed for KI#3 for incorporation in the FS_UAS_Ph3 TR23.700-59.

*** Start of changes (all new text) ***

## 6.X	Solution #X: NTZ restriction Provisioning aspects

### 6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #3, " Support of No Transmit Zones ".

### 6.X.2	Description

### Besides preconfiguration, this solution addresses the dynamical update of NTZ policies.

-

### 6.X.3	Procedures

#### 6.X.3.1	NTZ Provisioning

The call flow below is based on clause 4.15.6.7 Service-specific parameter provisioning in TS 23.502[4].

NOTE 1:	The UE can be preconfigured with NTZ polices and updates can be applied based on the procedure below. The procedure can also be delta update.

Figure 6.X.3.1-1 NTZ Parameter provisioning

- To create a new request, the USS/TPAE invokes an Create / Update / Delete NTZ parameters’ service operation (or reuse any existing NEF APIs) to UAS NF to request. The request may include subscription information to the report on the UE(NTZ) policy delivery outcome.

To update or remove an existing request, the USS/TPAE invokes an NTZ parameter Update or Delete service operation together with the corresponding Transaction Reference ID which was provided to the USS/TPAE in Create response message.

2.	The USS/TPAE sends its request to the NEF/UAS-NF. The NEF/UAS-NF authorizes the USS/TPAE request. The NEF/ UAS-NF performs the following mappings:

2a.	(Optional) The NEF/UAS-NF may invoke Nudm_SDM_Get service operation to perform the following mappings:

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into the Internal Group Identifier, according to information received from UDM. (e.g. 3GPP UAV ID, CAA Level UAV ID)

If the USS/TPAE subscribed to the outcome of UE Policy delivery, the USS/TPAE indicates where (resource URI of the target recipient of the notification) the USS/TPAE receives the corresponding notifications.

2b.	 (Optional) (in the case of Create or Update): The NEF/UAS-NF may need to authorize the service-specific parameter provisioning request with the UDM by sending a Nudm_ServiceSpecificAuthorisation_Create service operation as the USS/TPAE entity is outside the 3gpp domain.

(in the case of NTZ parameter delete): The NEF/UAS-NF requests the UDM to remove the authorization of the service-specific parameters provisioned by sending a Nudm_ServiceSpecificAuthorisation_Remove service operation.

3.	(in the case of NTZ parameters’ Create or Update): The NEF/UAS-NF stores the USS/TPAE request information in the UDR as the "UEPolicySet" (Data Subset setting to " NTZ Info") together with the assigned Transaction Reference ID.

(in the case of NTZ parameters’ Delete): The NEF/UAS-NF deletes the USS/TPAE request information from the UDR.

4.	The NEF/UAS-NF responds to the USS/TPAE. In the case of Create response message, the response message includes the assigned Transaction Reference ID. If the UE is registered to the network and the PCF performs the subscription to notification to the data modified in the UDR by invoking Nudr_DM_Subscribe (USS/TPAE NTZ parameter provisioning information, SUPI, Data Set setting to "UEPolicySet", Data Subset setting to " NTZ Info") at step 0, the following steps are performed:

5.	The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

NOTE 2:	PCF does not have to subscribe for each UE the application specific information, e.g. if PCF has already received the application specific information for a group of UE or PLMN. The same NTZ specific information is delivered to every UE in a group.

6.	The PCF initiates UE Policy delivery (as specified in section 6.X.3.2).

7.	(Optional) If the USS/TPAE subscribed to notifications about the outcome of UE Policies delivery due to NTZ parameter provisioning targeting a single UE and the PCF is notified of UE Policy Container from the AMF, the PCF notifies the UE Policy delivery result contained in the UE Policy container as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposure_Notify.

If the PCF is notified about UE Policy delivery failure from the AMF due to UE not reachable, the PCF may determine to retry step 6 of this procedure when the UE becomes reachable. In such a case, the PCF may report the interim status i.e. UE is temporarily unreachable as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposoure_Notify.

If the PCF determines the failure of the UE Policies delivery procedure, the PCF notifies the failure with an appropriate cause such as UE is unreachable as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposure_Notify.

8.	(Optional) When the NEF/UAS-NF receives Npcf_EventExposure_Notify, the NEF/UAS-NF performs information mapping (e.g. AF(USS/TPAE) Transaction Internal ID provided in Notification Correlation ID to AF(USS/TPAE) Transaction ID, SUPI to GPSI, etc.) and triggers the appropriate Nnef_ServiceParameter_Notify message.

#### 6.X.3.2	UE Configuration Update procedure for transparent UE Policy delivery

This procedure is initiated when the PCF wants to update UE policy information (i.e. UE policy) in the UE configuration. The below call flow procedure follows the steps from clause 4.2.4.3 in TS 23.502[4]. The UE policy delivery mechanism is reused for NTZ policy delivery.

-

#### 6.X.3.3 No-transmission zone parameters & UE behaviour

The NTZ policy information will be based on the coordinates and the indication flag whether NTZ is applicable, as well as the list of allowed frequency bands. The below tables are described as an example. More details on the data structure need to be defined by stage3.

Editor's note:	This clause represents an example of NTZ parameters. The detailed and complete parameters are FFS based on provisinong data.

Table 6.X.3.3-1 Sample NTZ Parameters provided by USS/TPAE

| NTZ parameters | Description |
| --- | --- |
| Geographical area | Identifies the geographical area indicated by the geographical area field (including coordinates with (longitude, latitude & height) (similar data types can be applied for height-dependent NTZ when served by RAN) |
| Radio Parameters | Radio parameters, e.g., which bands are allowed and whether the transmission is restricted for UE in a certain geography |

This solution assumes that the UE, upon applying UE Policy (NTZ policy) with NTZ information, implements the following behaviour:

- UE does not do cell search or any radio transmission in a no-transmission zone.

- UE does not use the restricted bands in band-restricted geography.

Editor's note:	When NTZ band restrictions information is configured, how UE handles RFSP is FFS.

### 6.X.4	Impacts on services, entities and interfaces

UE

- Complying to new NTZ information based on policy information.

NEF

- Supporting new API/new parameters to support NTZ provisioning.

UDR

- Storage of new NTZ information.

PCF

- Support of new NTZ policy.

*** END of changes ***