SA WG2 Meeting #S2-163	S2-2405859

27 - 31 May, 2024, Jeju, South Korea

3GPP TSG-CT WG1 Meeting #148	C1-242934

# Changsha, China, 15 -19 April 2024

# Title:	LS on Indicating the support of slice based N3IWF/TNGF selection from the UE to the network

# Response to:

# Release:	Rel-18

# Work Item:	5WWC_Ph2

Source:	CT1

To:	SA2

Cc:	CT3, CT4

Contact Person:

Name:	Mohamed Amin Nassar

E-mail Address:	mohamed.a.nassar@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

TS 23.502 states the following in clause 4.12.2.2:

Then the AMF triggers the UE PCF to update the UE policies for slice specific N3IWF selection by either indicating the request in Npcf_UEPolicyControl_Create or, if a UE policy already exists, by issuing a Npcf_UEPolicyControl_Update. (…)

NOTE 8:	The UE is assumed to inform PCF whether the UE supports Extended Home N3IWF identifier configuration and Slice-specific N3IWF prefix configuration as part of the UE policy update procedure. Details will be specified in Stage 3 specifications.

Also TS 23.502 states the following in clause 4.12a.2.2:

Then the AMF triggers the UE PCF to update the UE policies for slice specific trusted access selection by either indicating the request in Npcf_UEPolicyControl_Create or, if a UE policy already exists, by issuing a Npcf_UEPolicyControl_Update. (…).

NOTE 6:	The UE is assumed to inform PCF whether the UE supports Extended WLANSP or ANDSP as part of the UE policy update procedure. Details will be specified by CT WG1.

CT1 is discussing the impact of the above NOTEs on stage-3 specifications and has the following observations:

As per the current stage 2 requirements, the UE already indicates to the AMF whether it supports the slice-based N3IWF selection and/or the slice-based TNGF selection within the REGISTRATION REQUEST message during the registration procedure, which is in line with the legacy approach.

However, as per the above stage 2 NOTEs, the UE also provides separate indications to the PCF about supporting those features. CT1 does not see the reason to step away from the legacy approach in which the AMF indicates to the PCF whether the UE supports slice-based N3IWF selection and/or the slice-based TNGF selection as this is aligned the protocol design defined from Rel-15. CT1 therefore asks SA2 to consider re-using the existing approach.

2. Actions:

To SA2 group:

ACTION: 	CT1 would like SA2 to take the above information into account, and to provide any feedback if needed.

3. Date of Next CT1 Meetings:

CT1#149                           27th May- 31st May 2024                       India, IN

CT1#150                           19th August- 23rd August 2024..............Maastricht, NL