SA WG2 Meeting #163  	  S2-2405901

27 May-31 May 2024, Jeju island, Korea

Source:	ZTE

Title:	TR 23.700-84: KI#2 Conclusion

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

# 1. Discussion

Interim conclusion for Key issue 2.

# 2. Proposal

---------- Start of change(All text new) ----------

8.3 Key Issue #2: 5GC Support for Vertical Federated Learning

The following principle is proposed to be the interim conclusion of KI#2:

- VFL server and VFL client naming should be used both in VFL training and VFL inference, with the definition below:

-VFL Sever: An NF(i.e. NWDAF or AF) with labels for a VFL training task that may have related input data.

-VFL Client: A VFL client with access to the required input data without the required labels for a VFL training task. There can be clients in VFL.

- VFL entities register to NRF with NF profile including VFL capability information (e.g. VFL Server, VFL Client, VFL Sever and Client)

- VFL server select VFL client(s) from NRF for VFL training process, which will also be used for subsequent VFL inference process.

For VFL training process:

- The VFL sever sends an initial model to every VFL client.

- In every iteration in the training process, VFL server receives intermediate training result from VFL client and sends loss information or gradient information to VFL client.

- In every interation in the training process, the VFL client perform local model training, receive loss information or gradient information from VFL server and update its local model by computing gradient.

For VFL inference process:

-The VFL client performs local inference with its trained local model.

-The VFL server aggregates the local inference result from VFL client and send the final result to the consumer NF.

---------- End of change ----------