3GPP TSG-WG SA2 Meeting #163	S2-2405907

# Jeju, Korea, May 27 – May 31, 2024

Source: 	China Mobile, Huawei, vivo, China Unicom, NEC, Tencent, CATT, Lenovo,

Title: 	Conclusion on KI#1: Network energy related information exposure

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: Propose an update for conclusion on KI#1: Network energy related information exposure.

1	Discussion

This paper provides the update of conclusion of KI#1: Network energy related information exposure.

2	Proposal

It is proposed to include the below changes into TR 23.700-66 v0.6.0.

FIRST CHANGE

8.1	Interim Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

-	The following granularities of the energy consumption information are supported to be exposed, based on operator policy/configuration:

-	per Service, which are derived based on per-UE-per-QoS flow level.

-	per Network Slice level.

-	per-UE-per-QoS flow.

-	per PDU session.

-	per UE.

Editor's note:	Which energy-related parameters to be exposed is FFS.

-	The energy related information which can be exposed to AF or 5GC NF, includes:

-	Energy consumption information;

-	Renewable energy ratio information;

-	Energy credit status (e.g. energy credit for a granularity is reached);

-	AF or 5GC NF may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

-	The information for the calculation of the Energy Consumption information is obtained from the following sources:

-	OAM: provides the energy consumption information at the NF level, i.e., energy consumption of the gNB(s) and UPF(s) serving the UE, or Network Slice serving the UE;

-	OAM: provides the overall data volume for the gNB;

-	UPF: provides the overall data volume of the UPF;

-	UPF: provides the data volume for the QoS flow;

-	New functionality in the 5GC determines the E2E energy consumption per the granularities above based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

Editor's note:	How E2E energy consumption information is determined is FFS.

Editor's note:	Whether other energy related information can be exposed and how it is collected in the system is FFS.

-	New functionality exposes the energy consumption information of the QoS flow including UPF and gNB energy consumption.

NOTE 1:	Exposure and collection of renewable energy and carbon emission information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

Editor's note:	Whether in this release the NG-RAN will provide per UE level energy consumption information is FFS.

Editor's note:	It is FFS whether the new functionality will be supported by a new and/or the existing NF(s).

Editor's note:	It is FFS whether the contribution of other NFs, e.g. SMF, AMF, will be considered.

End of CHANGES