SA WG2 Meeting #163	S2-2405997

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#1, Interim conclusion

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#1.

# 1	Discussion

Based on the solutions documented in TR 23.700-66, this paper proposes a set of principles to be considered in the normative work for KI#1.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change **********

## 8.1	Interim Conclusion for KI#1: Network energy related information exposure

The following principles are agreed for normative work for KI#1:

-	The following granularities of the energy-related information are supported to be provided (collected and/or computed and/or estimated) by 5GC , based on operator policy/configuration:

-

-	per Network Slice level.

-	per UE

-	per PDU session per UE

-	per QoS Flow per UE

-	The following types of energy-related information are supported in the first bullet:

-	Energy consumption information

-	Prediction of energy consumption

-	Renewable energy consumption information

-	Based on the operator's policy and configuration, the supported granularities in the first bullet are exposed to the authorized consumer together with the performance statistic information (e.g. the data rate, packet delay and packet loss) according to the following table:

Table 8.1-1: Energy related information exposure

|  | AF | OAM (including Charging) | Other NFs in 5GC | UE |
| --- | --- | --- | --- | --- |
| Per network slice | No | Yes | Yes | No |
| Per UE | No | Yes | Yes | Yes |
| Per PDU session per UE | No | Yes | Yes | No |
| Per QoS Flow per UE | Yes | Yes | Yes | No |

-	The authorized consumer may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

-	The information for the calculation of the Energy Consumption information is obtained from the following sources:

-	OAM

-	UPF

Editor's note: How E2E energy consumption information is determined is FFS.

Editor's note: Whether other energy related information can be exposed and how it is collected in the system is FFS.

NOTE 1:	Exposure and collection of renewable energy and carbon emission information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

Editor's note: Whether in this release the NG-RAN will provide per UE level energy consumption information is FFS.

-	A new NF is introduced to provide energy-related information besides the other functionalities specified in KI#2 and KI#3 for this new NF.

-	In the current release of the specification, only the energy-related information of data plane communication (not control plane signalling) is supported.

********** End of Changes **********