SA WG2 Meeting #163	S2-2405998

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#2, Interim conclusion

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#2.

# 1	Discussion

Based on the solutions documented in TR 23.700-66, this paper proposes a set of principles to be considered in the normative work for KI#2.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change **********

## 8.2	Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

For KI#2, the following enhancements on subscription and policy control are agreed as interim conclusions to support energy efficiency and energy saving as service criteria in the 5GS:

-	The following are supported:

-	Subject to operator's policy, subscription policies include a maximum energy credit limit for services without QoS criteria.

-	Energy saving authorization information to allow the network to perform energy saving for the UE at the following granularity level.

-  At UE level

Editor's note:	It is FFS whether and what energy related limits (e.g. energy consumption limits) per UE is to define, controlled and used, which depends on whether and what energy related information per UE can be obtained as concluded in KI#1.

-	Energy saving authorization information are stored in UDM/UDR.

NOTE 1: Other NFs (e.g., AMF, SMF, and PCF) use the stored information in the UDM/UDR for energy related policy control/enforcement.

Editor's note:	Whether the energy credit is stored in UDM/UDR or handled by the charging subsystem is FFS (decided in cooperation with SA WG5).

-	The PCF may take into account energy related information including subscription data when making policy decisions on the AM, SM and UE policies.

NOTE 1: How and which aspects of AM, SM and UE policies may be determined by PCF is the internal logic of PCF and based on the operator's policy.

Editor's note:	Whether the ratio of renewable energy and carbon emission information can be considered by the PCF for policy control is FFS, which depends on whether the information can be obtained as concluded in KI#1.

Editor's note:	Whether energy related information (e.g. energy consumption) per Application ID, UE, PDU Session or QoS flow can be considered by the PCF for policy control is FFS, which depends on whether the information can be obtained as concluded in KI#1.

Editor's note:	Whether energy related analytics provided by NWDAF can be considered by the PCF for policy control is FFS, which depends on the conclusion in KI#3.

Editor's note:	Whether the interaction with CHF for policy enforcement and charging is FFS.

-	PCF and AF may negotiate a time windows and/or QoS requirements for data transfer.

-	A new NF is introduced to provide energy-related information needed by PCF besides the other functionalities specified in KI#1 and KI#3 for this new NF.

********** End of Changes **********