SA WG2 Meeting #163	S2-2406212

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2404579)

Source:	LG Electronics

Title:	KI#2, Sol#4: Updates to resolve ENs

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: This paper proposes to update Solution#4.

Discussion

Solution #4 "MWAB authorization handling" includes two ENs that need to be resolved.

1)	Editor's note: The need of the indications on RRC and NGAP is FFS.

It is proposed to add the following NOTE while removing this EN.

NOTE 1:	This solution can also work without the MWAB Indication via RRC and NGAP messages and the MWAB authorization status to BH-gNB. If the BH-AMF selected by the BH-gNB does not support MWAB, AMF re-allocation is triggered e.g. based on requested slices, MWAB Indication included in the Registration Request message.

2)	Editor's note: The needs and usage of the additional indications from step 1 is FFS.

Each serving network may have different preference or operator policy regarding MWAB-UE's registration status and BH PDU Session(s)' status when MWAB-UE authorization status is changed from allowed to not-allowed. That is, in this case, some PLMNs may prefer to deregister MWAB-UE while some PLMNs may prefer to keep MWAB-UE registered. And in this case, some PLMNs may prefer to release BH PDU Session(s) while some PLMNs may prefer to keep BH PDU Session(s).

In addition, based on location/time restriction for MWAB, MWAB's location and current time, keeping MWAB-UE registered or BH PDU Session(s) may be more efficient or appropriate than deregistering MWAB-UE or releasing BH PDU Session(s), e.g. if it is expected that the MWAB-UE authorization status is changed soon from not-allowed to allowed.

Therefore, having considering various preferences/policies in the MWAB-UE's serving PLMN, MWAB's location/time restriction, flexible handling for MWAB-UE, etc., it is proposed to add this text for clarification while removing this EN.

When MWAB-UE authorization status is changed from allowed to not-allowed, the BH-AMF may provide one of the following additional information in the UE Configuration Update Command message based on e.g. operator local policy, location/time restriction for the MWAB, MWAB's location, current time:

Proposal

It is proposed to agree the following changes into TR 23.700-06.

* * * * Start of 1st Change * * * *

6.4	Solution #4: MWAB authorization handling

6.4.1	Introduction

This solution addresses issues related to KI#2. It provides methods to support MWAB operation authorization in different roaming scenarios. It also provides means to handle authorization status change handling for both MWAB-gNB logic and MWAB-UE logic.

6.4.2	Functional Description

When MWAB node includes MWAB-gNB function and MWAB-UE function as described in clause 4, the authorization of the MWAB operation is based on subscription information linked to the MWAB-UE and the authorization status can depend on the location and/or time.

The MWAB-gNB and MWAB-UE may be connected to the same network/5GC, or they may be connected to different network/5GC. The UDM which holds the subscription information for MWAB-UE can be in the BH-5GC (i.e. where the MWAB-UE is registered) or in a different 5GC (which has roaming agreement with the network serving the MWAB-UE).

Figure 6.4.2-1: MWAB-gNB and MWAB-UE connect to the same 5GC

Figure 6.4.2-2: MWAB-gNB and MWAB-UE connect to different 5GCs

The BH-AMF, where the MWAB-UE is registered, is responsible for the authorization of the MWAB operation during the NAS registration procedure in all scenarios (i.e. non-roaming as well as roaming) based on the subscription information linked to the MWAB-UE.

The BH-AMF provides the MWAB node authorization information to the MWAB-UE via NAS registration related message and to the BH-gNB via NGAP message.

The MWAB-UE provides the authorization information to MWAB-gNB. The communication between MWAB-UE and MWAB-gNB is based on implementation.

The MWAB-gNB initiates the gNB operations (e.g. it requests MWAB-UE to setup IP connections for needed backhaul communication and sets up the RAN-CN connection towards AMF), if authorization information indicates that the MWAB is allowed to operate as MWAB node.

6.4.3	Procedures

6.4.3.1	MWAB node authorization and operation initiation

This procedure describes the MWAB node authorization steps and the operations with focus on the 5GC aspects.

Figure 6.4.3.1-1: MWAB node authorization and operation initiation

MWAB-UE registration and authorization phase:

1.	The MWAB-UE triggers registration towards the selected PLMN in NR cell. The MWAB-UE provides the MWAB Indication via RRC and NGAP message to BH-AMF.

2.	The BH-AMF retrieves MWAB-UE subscription data from UDM and authorizes the MWAB operation.

3.	The BH-AMF accepts the MWAB-UE registration request and provides the MWAB authorization status to MWAB-UE and BH-gNB.

NOTE 1:	This solution can also work without the MWAB Indication via RRC and NGAP messages and the MWAB authorization status to BH-gNB. If the BH-AMF selected by the BH-gNB does not support MWAB, AMF re-allocation is triggered e.g. based on requested slices, MWAB Indication included in the Registration Request message.

MWAB-gNB operation initiation:

4.	Based on authorization allowed information provided, the MWAB establishes the IP connectivity for backhaul usage. Or the MWAB-UE may establish PDU Session to provide the IP connectivity for backhaul usage based on the MWAB authorization status (i.e. authorized) provided by the AMF.

NOTE 1:	The detailed communication between MWAB-gNB and MWAB-UE is implementation based and not in SA2 scope.

5.	The MWAB-gNB connects to the AMF via the backhaul IP connectivity provided by the MWAB-UE.

6.	The MWAB-gNB initiates the service towards UE.

NOTE 2:	How the MWAB-gNB receives the parameters needed for operation (e.g. PLMN ID, TAC, Cell information with CAG IDs, AMF information for connection) is addressed by other solutions.

UE registration via MWAB-gNB cell:

7.	The UEs allowed to access the MWAB cell selects the cell and trigger Registration Request.

6.4.3.2	MWAB authorization status change for Registered MWAB

This procedure is used when MWAB authorization status changes for a registered MWAB-UE .

Figure 6.4.3.2-1: MWAB authorization status change handling

Registered MWAB-UE authorization status change:

1.	BH-AMF triggers NAS UE configuration update procedure to inform the MWAB-UE with the MWAB authorization status change.

When MWAB-UE authorization status is changed from allowed to not-allowed, the BH-AMF may provide one of the following additional information in the UE Configuration Update Command message based on e.g. operator local policy, location/time restriction for the MWAB, MWAB's location, current time:

a)	Indication that the MWAB-UE needs to be deregistered.

b)	Indication that the BH PDU Session(s) need to be released.

NOTE 1: 	The triggering for this may be different (e.g. subscription data change, location restriction, time restriction, local policy in BH-AMF).

MWAB-UE authorization status change from allowed to not-allowed:

2.	Based on authorization allowed information provided by the MWAB-UE, the MWAB-gNB triggers the move of connected UEs to other cells.

3.	After all the UEs are moved, the MWAB-gNB may remove the TNLA and NGAP connection towards the AMF.

4.	The MWAB releases the IP connectivity or the MWAB-UE may release the BH PDU Session(s) based on the additional information received in step 1.

The MWAB-UE may deregister based on the additional information received in step 1. Or the BH-AMF may deregister the MWAB-UE from the network based on local policy, after the BH PDU Sessions(s) have been released or a timer that started at step 1 expires.

The MWAB-gNB shuts down the air interface.

NOTE 2:	Deregistration of MWAB-UE can be performed without performing BH PDU Session(s) release separately.

NOTE 3:	The detailed communication between MWAB-gNB and MWAB-UE is implementation based and not in SA WG2 scope.

MWAB-UE authorization status change from not-allowed to allowed:

5-8.	Same to steps 4-7 in Figure 6.4.3.1-1.

6.4.4	Impacts on services, entities, and interfaces

AMF:

-	Support MWAB authorization handling based on subscription data and local policy.

-	Support NGAP removal procedure.

MWAB-UE:

-	Support Authorization status handling.

-	Support the IP connectivity establishment for the backhaul usage request from MWAB-gNB.

MWAB-gNB:

-	Support NGAP removal procedure.

-	Support the handling of UE move to other cells when authorization status changes.

* * * * End of Changes * * * *