SA WG2 Meeting #S2-163	S2-2406220

27 - 31 May, 2024, Jeju, South Korea	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4816 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Application Layer ID to Nnef_ServiceParameter service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 sends the LS (S2-2405863/C3-242616) to ask SA2 “the Application Layer ID is only added in TS 23.502 clause 5.2.6.11.5 Nnef_ServiceParameter_Get service operation, with GMLC as southbound service consumer which is different from the existing northbound service consumer AF. The other service operations in Nnef_ServiceParameter service is still missing the Application Layer ID; This CR addes the Application Layer ID to other Nnef_ServiceParameter service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Application Layer ID to Nnef_ServiceParameter service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Application Layer ID is missing in the Nnef_ServiceParameter Create/update service operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.11.2, 5.2.6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.6.11.2	Nnef_ServiceParameter_Create operation

Service operation name: Nnef_ServiceParameter_Create

Description: The consumer stores service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier)

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available, Application Layer ID if available), "PLMN ID(s) of inbound roamers", subscribedEvents, notificationDestination.

If this service operation is used by AF to provision Application Layer ID and GPSI mapping for Ranging/SL Positioning defined in TS 23.586 [88], both GPSI and Application Layer ID shall be present.

If identifiers of target UE(s) or a group of UEs or "PLMN ID(s) of inbound roamers" are not provided, then the Service Parameters shall correspond to any UEs of the PLMN of the NEF using the service identified by the Service Description.

Outputs, Required: Transaction Reference ID, operation execution result indication.

Outputs, Optional: None.

Next CHANGE

##### 5.2.6.11.3	Nnef_ServiceParameter_Update operation

Service operation name: Nnef_ServiceParameter_Update

Description: The consumer updates service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier), Transaction Reference ID.

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available, Application Layer ID if available), or "PLMN ID(s) of inbound roamers".

If this service operation is used by AF to provision Application Layer ID and GPSI mapping for Ranging/SL Positioning defined in TS 23.586 [88], both GPSI and Application Layer ID shall be present.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

END OF CHANGES