SA WG2 Meeting #163	S2-2406259

Jeju, Korea, May 27-31 2024

# Title:	KI#2: conclusions

# Source:	Nokia

# Document for:	Approval

Agenda Item:	FS_VMR_Ph2/19.6

Abstract:	this paper provides conclusions on KI#2 on MWAB authorization

1. Introduction

Based on the outcomes of the moderated email discussion we propose to provide an update to the conclusions for KI#2

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# Proposed text

## 8.2	KI#2 Conclusion

.

It is proposed to proceed normatively based on these principles.

1)	From SA WG2 perspective there is no need to specify AS level indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB. To detect a UE intends to operate as MWAB, dedicated S-NSSAI(s) for MWAB operation may be used for a MWAB-UE. If dedicated S-NSSAIs for MWAB operation are used, then the location and time restriction can be based on related slicing features. The MWAB UE, may deregister any S-NSSAI for MWAB operation and request only S-NSSAI(s) that are not dedicated to MWAB Operation if it does not need to use BH PDU sessions.

2)	There is no need to indicate to NG-RAN serving a MWAB-UE that the MWAB-UE is authorized to act as MWAB-UE.

3)	The MWAB-gNB releases the NG connections when it is no longer authorized to operate by OAM. The MWAB-gNB should hand over the UE(s) it serves to other cells before it releases the NG connection. For the case that the BH PDU sessions are released by the MWAB-UE, the MWAB-UE does it only if the MWAB-gNB instructs the MWAB-UE that it may do so.

4) 	The OAM should control the time and location where the MWAB-gNB can operate. The MWAB-UE Configuration in the UE and in the network determining availability for the UE of the dedicated network slices for MWAB operation should ensure the BH sessions are not released due to network slices for MWAB operation becoming unavailable, while the MWAB-gNB needs to still handover UEs it serves before it completes its transition to no operation status under OAM control.

# End of proposed text