3GPP TSG-WG SA2 Meeting #163	S2-2406283

27 - 31 May, 2024, Jeju, South Korea

Source:	T-Mobile USA

Title:	KI#2 AIML Conclusions on VFL

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN/R19

# Proposal

It proposed to adopt the conclusions as below in TR 23.700-84.

# 8	Conclusions

# * * * * First change (all new text)* * * *

## 8.2	Key Issue #2: 5GC Support for Vertical Federated Learning

The architecture principles and specific mechanisms below are considered for the normative work on 5GC support for Vertical Federated Learning:

- 	When the VFL Server is an NWDAF, the NWDAF shall perform sample alignment and generate the intersection of samples.

-	When the VFL Server is an AF, the AF together with the NEF shall perform sample alignment and generate the intersection of samples.

- 	The NEF shall control VFL client selection and control the exchange of all intermediate results and gradients based on, e.g., the VFL task correlation ID and VFL model correlation ID.

-	The NEF shall control the passing through of inference operations based on, e.g., the VFL model correlation ID.

# * * * * End of changes * * * *