SA WG2 Meeting #163	S2-2406329

Jeju, Korea, May 27 – May 31, 2024

Source:	vivo

Title:	KI#4 overall evaluation

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution:  This contribution proposes an overall evaluation on key issue #4.

1 Discussion

This contribution proposes an overall evaluation on key issue #4.

2 Proposal

It is proposed to update TR 23.700-84.

First change

## 7.X	Key Issue #4: NWDAF enhancements to support network abnormal behaviours (i.e. signalling storm) mitigation and prevention

There are 5 solutions for KI#4 (Solutions #35, #36, #37, #38, #39). Table 7.X-1 provides details of signalling storm mitigation and prevention solutions.

Table 7.X-1: Evaluation of the KI#4

| Candidate solution | Use case | Input Data | Output Analytics | Mitigation and Prevention | Open issues |
| --- | --- | --- | --- | --- | --- |
| Solution#35 | UE NF | - Signalling exchange information - Type, number, timer duration of signalling - NF Context information - State transition, timer information of UE - NF load, capability information - UE behaviour related information - existing output of UE related behaviour analytics | Detection: - Target NF ID, - Abnormality ID, - Source SUPI/NF list Prediction: - Target NF ID, - Abnormality ID, - Source SUPI/NF list - Confidence | Massive UE access: - AMF/SMF set back-off timer - AMF/SMF set threshold - AMF triggers RAN on slice or priority based overload control NF abnormal signalling: - Policy control for NF discovery, (re-)selection, UE reestablishment) | Details of input on per UE or per NF, collect from OAM or another place Update may be needed for mitigation and prevention |
| Solution#36 | UE | - Registration signalling information from AMF - Type, number, timer of signalling - NF Context from AMF - Number of UE - AMF resource usage - Application activation time information from AF - UEs within time range | Detection: - List of registration status - list of UEs for normal and abnormal - for abnormal UE, signalling feature - AMF resource usage Prediction: IEs in detection with additional IE Confidence | AMF arranges suitable timer for the particular users |  |
| Solution#37 | NF | - MDAS data - signalling storm/congestion analysis as depicted in TS 28.104 | / | increase or adjust back-off timer by AMF change of e.g. usage monitoring data reports periodicity/charging reports periodicity between SMF and PCF and SMF and CHF | NWDAF us NF specific data to output UE specific control More details on actions |
| Solution#38 | UE | Detection: - UE data - UE ID, back-off timer, expected UE behaviour, priority, location information - Signalling statistics - Type, frequency information from UE/NF - NF Context Information - NF load, status, capacity Recommendation (ReLF) Besides input of detection: NF instance ID | Detection and prediction: - Target NF ID - NF resource usage - Signalling analytics (total number, growing speed, UE information, failure information) - Probability, threshold Recommendation: - Target NF ID - Recommendation for processing request/service/NF profile/UE | / | Recommendation needs further study. |
| Solution#39 | UE NF | - Signalling exchange information - Type, number, timer of signalling - NF Context information - Number of UE/contexts - NF load, capacity information - Unexpected operational event | - Destination NF ID, - Exception ID, - Source NF ID - Reference Point & Service Operation | The service producer entity: Back-pressure, Connection Pooling, etc. The service consumer entity: Request throttling, NF reselection, etc. OAM: Horizontal/Vertical scaling, Proper configuration of the network entities, etc. SCP: Request throttling, Back-pressure, Connection Pooling, etc. NRF: Local policy to isolate the impacted NFs (e.g. avoid selecting the overwhelmed service producer for a time period). | Whether to collect data from SCP How to reduce impact of UEs |

Based on the above aspects, the detailed analysis is elaborated as follows:

Solution #35, #36, #38, #39 proposed to address use case #3 signalling storm caused by UE, while solution #35, #37, #39 proposed to address use case #6 signalling storm caused by NF. Solution #35 and #39 could cover both use cases.

For input data,

-	Most of the solutions require signalling exchange information, including type, number, time of signalling.

-	Most of the solutions require NF context information, including numbers of UE, NF load, status and capacity information.

-	Solution #35, #36, and #38 require UE related information: state transition, timer information of UE, output of existing UE related behaviour analytics, application activation time from UE, expected UE behaviour, priority, location information to help identify abnormal UEs.

-	Solution #37, and #39 require output of existing signalling storm/congestion analytics and unexpected operational event to help identify abnormal NFs.

-	Solution #38 requires ReLF to do recommendation, in this case, additional IE NF instance ID needs to be collected.

For output analytics,

-	All the solutions require both detection and prediction output analytics.

-	All the solutions require target NF ID.

-	Solution #35, #36, #39 require source entity ID who triggers signalling storm.

-	Solution #35 and #39 require Abnormality/Exception ID to describe cause of the signalling storm.

-	solution #36 and #38 require information related status or signalling information, e.g. NF usage information, signalling features.

-	Solution #38 requires ReLF to do recommendation, in this case, additional IEs recommendation for processing request/service/NF profile/UE needs to be output.

For mitigation and prevention,

-	Most of the solutions propose AMF to set back off timer to control UE signalling.

-	Solution #35 proposes additional actions to control UE signalling (SMF set back off timer, AMF/SMF set threshold, AMF triggers RAN on slice or priority based overload control) and actions to control NF signalling (Policy control for NF discovery, (re-)selection, UE reestablishment)

-	Solution #37 proposes additional action to control NF signalling (usage monitoring data reports periodicity/charging reports periodicity between SMF and PCF and SMF and CHF)

-	Solution #39 proposes additional action to control NF signalling (The service producer entity: Back-pressure, Connection Pooling, etc. The service consumer entity: Request throttling, NF reselection, etc. OAM: Horizontal/Vertical scaling, Proper configuration of the network entities, etc. SCP: Request throttling, Back-pressure, Connection Pooling, etc. NRF: Local policy to isolate the impacted NFs (e.g. avoid selecting the overwhelmed service producer for a time period).

End of change