SA WG2 Meeting #163	S2-2406331

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

Source: 	vivo

Title: 	KI#5, Evaluation and Conclusion of KI#5

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides evaluation and conclusion for key issue #5.

1	Discussion

This contribution provides the evaluation and interim conclusions for KI#5.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

# * * * * First change* * * *(all new text)

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x	Key Issue #5: Handling of PS data off exemption for services over IMS DC

For Key Issue #5, the following solutions are proposed:

-	Solution#28 mainly proposes the following:

-	The solution covers the scenario of IMS DC is an exemption service, but only partial DC applications are allowed by operator.

-	If IMS Session with DCs have been established when the user actives the PS Data Off

-	The IMS AS notifies the DCSF the 3GPP Data Off status (active) when receives an indication of PS Data Off status from S-CSCF.

-	The DCSF may choose a specific DC application list for PS Data Off and trigger the release of DCs associated with the applications not in the list.

-	The DCSF provides UE the specific DC application list for PS Data Off which contains only the allowed applications when receiving DC application list request from UE.

-	If IMS Session with DCs has not been established when the user actives the PS Data Off

-	When the UE requests to establish a bootstrap DC, the IMS AS notifies the DCSF the 3GPP Data Off status (active).

-	When receiving the UE request of DC application list through bootstrap DC, the DCSF sends UE the specific DC application list for PS Data Off which contains only the allowed applications.

-	Solution#29 mainly proposes the following:

-	The solution covers the scenario of IMS DC is not an exemption service.

-	If IMS Session with DCs have been established when the user actives the PS Data Off.

-	UE shall immediately stop sending any media packets for IMS DC services.

-	UE can terminate both the application DC and bootstrap DC sessions related to the IMS DC service.

-	optionally terminate the bootstrap DC session after a certain period of time.

# * * * * Next change* * * *(all new text)

8	Conclusions

8.X	Interim conclusion of KI#5

The following interim conclusions for KI#6 "Support of Standalone IMS Data Channel Sessions" are agreed:

-	"Services over IMS Data Channel" is configured as a whole service in 3GPP PS Data Off Exempt.

-	If IMS DC is not an exemption service based on operator policy:

-	UE shall immediately stop sending any media packets for IMS DC services and terminate both the application DC and bootstrap DC if IMS Session with DCs have been established.

-	If IMS DC is an exemption service, and all DC applications are exempt based on operator policy:

-	No impact on ongoing IMS DC sessions if have.

-	If IMS DC is an exemption service, and partial DC applications are exempt based on operator policy: (Operators may control whether each individual DC application is allowed):

-	The IMS AS notifies the DCSF that the user actives the PS Data Off, and the DCSF may

-		release the DCs associated with the applications which cannot be used during PS Data Of if IMS Session with DCs have been established.

-	provide UE the specific DC application list when receiving DC application list request from UE, the roaming status of the subscriber may be considered when choose the specific DC application list.

# * * * * End change* * * *