SA WG2 Meeting #163	S2-2406384

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#2 conclusion

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This paper proposes conclusion to KI#2 in TS 23.700-29.

Discussion

Proposes conclusions for KI#2.

Proposal

It is proposed to agree the following changes into TR 23.700-29.

* * * * Start of 1st Change (All New Texts) * * * *

8.X	Key Issue #2: Support of Store and Forward Satellite operation

The following principles for KI#2 are agreed for normative work.

-	Only eNB is on board the satellite to support Store and Forward operation. The eNB on board the satellite Stores and Forwards the signalling/data between the UE and the MME on the ground.

-	Single and multi-satellites scenarios are supported.

-	The Store and Forward operation support MO/MT SMS services and Control Plane and User Plane CIoT EPS optimisation.

-	The UE and the MME indicate their capability to support Store and Forward each other during the Attach or TAU procedure.

-	Feeder link availability information is provision to the MME via O&M. The MME may derive Downlink Buffering Duration time/requested retransmission time based on the feeder link availability information and provide it to the S-GW/SMS-GMSC.

-	The MME determines which satellite is available to deliver downlink data and/or signalling, that will cover the area where the UE is located.

-	The Store and Forward Satellite Specific Parameters will be defined during the normative work.

NOTE:	Security aspects need to be addressed by SA WG3.

* * * * End of Changes * * * *