SA WG2 Meeting #163	S2-2406418

May. 27th - May 31st, 2024; Jeju, Korea

Source:	China Mobile

Title:	Update on Solution #10: Registration Procedure for Ambient IoT Devices

Document for:	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT /Rel-19

# 1. Introduction

This solution makes updates on solution #10.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change* * * *

## 6.10	Solution #10: Registration procedure for Ambient IoT Devices

### 6.10.1	Description

This solution is for Key Issue #2 "Identification, Subscription, Registration and Connection management".

As depicted in Architecture Requirements, the traffic types of DT and DO-DTT will be studied in this stage. The Ambient IoT devices could be driven by the network for Topology1 or UE for topology 2 before registering to the network. This proposal proposes one potential mechanism for identification, subscription, registration management and the registration procedures as well.

The principles/assumptions are given below:

-	A new network function named Ambient IoT NF may be adopted to manage Ambient IoT devices and procedures. If not, this relevant function can be supported by AMF.

-	In 5GC, each Ambient IoT device has a unique internal device ID which consists of at least Operator ID, Group ID and instance ID. The Operator ID composed of Mobile Country Code (MCC) and Mobile Network Code (MNC) is preferred. The Group ID can represent the Company information.

-	The solution is based on an operator-controlled Ambient IoT device.

### 6.10.2	Procedures

#### 6.10.2.1	Procedures for AF triggered Registration

The following figure presents a procedure of AF triggered registration for Topology 1.

5. AF Triggered Registration Request (Group ID, EPC info …)

8. AF Triggered Registration Response

10. AF Triggered Registration Response

1. AF Triggered Registration Request (  EPC info lists, Location,  …)

2. AMF or New Ambient IoT NF selection

9. AF Triggered Registration Response

3. AF Triggered Registration Request (TA Lists, EPC info …)

AMF or New

Ambient IoT NF

Ambient AF

fd

AAA

NEF

Ambient IoT Device

- Pre-configuration

4. NG-RAN Selection

6. Device Access

7.  Authentication and Registration

UDM

NG-RAN

Figure 6.10.2-1 AF triggered Registration Procedure for Topology 1

- The Ambient IoT devices are pre-configured with default internal AIoT device ID and credentials.

The 5GC is pre-configured with the default Ambient IoT devices profile which contains the default device ID, default credentials and status too. The default internal AIoT device ID contains Operator ID, Company info, and instance ID 0000……0000.The device status is set as inactive.

1.	AF triggers registration and sends AF Triggered Registration Request to NEF. The service information such as the Operator ID, the Group ID, TID code list (See GS1 TDS Release 2.1 [10]), and location information are included. TID can be used for matching the targeted device.

2.	The NEF performs authentication and authorization for AF, and selects an AMF or Ambient IoT NF that supports Ambient IoT services based on the location information. It will map the location information into the TA list.

3.	The NEF sends AF Triggered Registration Request to the AMF/Ambient IoT NF, including the the Operator ID, Group ID, TID list, and the TA list.

4.	The AMF/Ambient IoT NF selects NG-RAN based on the TA list.

5.	The AMF/Ambient IoT NF forwards the registration request to NG-RAN, including the Operator ID, Group ID, and the TID list.

6.	NG-RAN activates the AIoT devices based on the Operator ID, the Group ID and TID list. The devices which match the Operator ID, the Group ID and the TID list access and register to the network via NG-RAN with default device ID, TID, and default credentials. A receiving limit time may be configured on NG-RAN. Once timeout, the message received after this time will be discarded by NG-RAN.

7.	The AIoT devices perform interaction with 5GC for Authentication/Security based on default device ID, TID, and default credentials. After successful authentication, the below actions are performed.

1) 5GC generates a instance ID filled in the default AIoT device ID.

2) The UDM or AAA updates credentials.

3) 5GC sets the device status as active.

4) The new device ID and credentials are written into the device.

8.	The NG-RAN returns the Response to the AMF/Ambient IoT NF.

- The AMF/Ambient IoT NF returns the response to the NEF.

- The NEF returns AF Triggered Registration Response to the AF.

#### 6.10.2.2	Procedures for UE triggered Registration

For Topology 2, the registration may be triggered by UE which performs as a reader. The UE interact with AMF or Ambient IoT NF via NG-RAN, which could be regarded as the supplement for AF triggered registration procedure.

Figure 6.10.2-2 UE triggered Registration Procedure for Topology 2

- The UE interacts with AF via the application layer. The UE GPSI and Group information are informed to AF.

- AF sends UE Triggered Registration Request to NEF, including the Operator ID, the Group ID, TID list , and the UE GPSI. TID can be used for matching the targeted device.

3.	The NEF performs authentication and authorization for AF, and selects an AMF or Ambient IoT NF that supports Ambient IoT services based on the UE GPSI.

4.	The NEF sends UE Triggered Registration Request to the AMF/Ambient IoT NF, including the the Operator ID, Group ID, TID list, and UE GPSI.

5.	The AMF/Ambient IoT NF selects NG-RAN and UE based on UE GPSI.

6.	The AMF/Ambient IoT NF sends the registration request to the UE via NG-RAN, including the Operator ID, Group ID, and the TID list.

7.	The UE activates the AIoT devices based on the Operator ID, the Group ID and TID list. The devices which match the Operator ID, the Group ID and the TID list access and register to the network via UE and NG-RAN with default device ID, TID, and default credentials.

8.	The AIoT devices perform interaction with 5GC for Authentication/Security based on default device ID, TID, and default credentials. After successful authentication, the below actions are performed.

1) 5GC generates a instance ID filled in the default AIoT device ID.

2) The UDM or AAA updates credentials.

3) 5GC sets the device status as active.

4) The new device ID and credentials are written into the device.

- The UE returns the Response to the AMF/Ambient IoT NF via NG-RAN.

- The AMF/Ambient IoT NF returns the Response to the NEF.

- The NEF returns the Response to the AF.

### 6.10.3	Impacts on services, entities and interfaces

NEF:

- The NEF supports conversion between the internal AIoT device ID and EPC code.

AMF/Ambient IoT NF:

- The AMF/Ambient IoT NF selects NG-RAN based on the TA list or gNB list and activates the AIoT devices.

UDM:

- The UDM stores the subscription information of Ambient IoT devices in group.

NG-RAN as a reader:

- The NG-RAN performs paging and receives response of Ambient IoT devices in Topology 1.

UE as a reader:

- The UE performs paging and receives response of Ambient IoT devices in Topology 2.

- The UE supports to trigger the registration procedure.

Ambient IoT device:

- The device stores the internal AIoT device ID or the group ID.

# * * * * End of changes * * * *