3GPP TSG-WG SA2 Meeting #163 	S2-2406436

May 27 – 31, 2024, Jeju Island, Korea	(revision of S2-220xxxx)

Source:	Intel (Rapporteur)

Title:	KI#2: Conclusion update based on NWM discussion

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes to update the conclusion for KI#2 based on NWM discussion.

# 1. Introduction

This contribution proposes conclusion update for KI#2 based on NWM discussion.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

## 8.2	Conclusion for KI#2

The following principles are concluded for normative work:

-	SMF selects local PSA UPF considering N6 delay, when available.

-	SMF collects node level N6 delay measurement from L-PSA UPF per pair of L-PSA UPF and EAS/Designated IP (range) of the data center corresponding to the DNAI.

-	EAS load is not aware by SMF/EASDF for (re)selection of L-PSA UPF or EAS.

-	N6 delay between L-PSA UPF and EAS is measured by leveraging existing mechanisms (e.g., defined by IETF, PING, TWAMP, OWAMP, etc.).

-	Interaction between AF and 5GC may be needed to enable the measurement, AF request can include candidate DNAI(s), target EAS IP address(es)/Designated IP (range) per DNAI, the measurement method which is further indicated to L-PSA UPF.

# * * * * End of changes * * * *